Report 2

October 2012

Monitoring of ecosystem responses to the delivery of environmental water in the Edward-Wakool system, 2011-2012

Watts, R.J.a, McCasker, N.a, Thiem, J. b, Howitt, J.A.a, Grace, M.c, Healy, S.d, Kopf, R.K.a, Dyer, J.G.a Conallin, A.e, Wooden I.b, Baumgartner L.b, Bowen P.e (2014). Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool river system, 2013-14. Institute for Land, Water and Society, Charles Sturt University, Final Report. Prepared for Commonwealth Environmental Water.

a. Institute for Land, Water and Society  		b. NSW Department of Primary Industries

Charles Sturt University 			   	   Narrandera Fisheries Centre,

PO Box789, Albury, NSW 2640			   PO Box 182, Narrandera NSW 2700

c. Water Studies Centre 				d. NSW Office of Environment and Heritage

Monash University,				   PO Box 363, Buronga NSW 2739

Clayton, Victoria, 3800

e. Murray Local Land Services

PO Box 979, Albury, NSW 2640

Copyright
© Copyright Commonwealth of Australia, 2014

‘Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool river system, 2013-14’ is licensed by the Commonwealth of Australia for use under a Creative Commons By Attribution 3.0 Australia licence with the exception of the Coat of Arms of the Commonwealth of Australia, the logo of the agency responsible for publishing the report, content supplied by third parties, and any images depicting people. For licence conditions see: http://creativecommons.org/licenses/by/3.0/au/   This report should be attributed ‘Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool river system, 2013-14’. The Commonwealth of Australia has made all reasonable efforts to identify content supplied by third parties using the following format ‘© Copyright.

Disclaimer

The views and opinions expressed in this publication are those of the authors and do not necessarily reflect those of the Australian Government or the Minister for the Environment. While reasonable efforts have been made to ensure that the contents of this publication are factually correct, the Commonwealth does not accept responsibility for the accuracy or completeness of the contents, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the contents of this publication.

Authorship

This project was undertaken as a collaboration among Charles Sturt University, NSW DPI (Fisheries), Murray Local Land Services, Monash University and NSW Office of Environment and Heritage. James Abell and Chris Smith were the key team members undertaking the field sampling for this project. The table below lists the lead author(s) for each section of the report. All authors contributed to project design, implementation, and review of the report.

| Section | Section Title | Lead author(s) |
| --- | --- | --- |
|  | Executive summary | Robyn Watts |
| 1 | Introduction | Robyn Watts |
| 2 | Commonwealth environmental water use options and objectives 2013-14 | Robyn Watts. |
| 3 | Commonwealth environmental watering 2013-14 | Robyn Watts |
| 4 | Location of monitoring | Robyn Watts |
| 5 | Indicators | Robyn Watts |
|  | Responses of indicators to Commonwealth environmental watering |  |
| 6.1 | Fish community | Jason Thiem, Ian Wooden, Lee Baumgartner |
| 6.2 | Fish movement | Jason Thiem, Ian Wooden, Lee Baumgartner |
| 6.3 | Fish spawning and reproduction | Nicole McCasker |
| 6.4 | Fish recruitment | Keller Kopf |
| 6.5 | Riverbank and instream vegetation | Sascha Healy, Robyn Watts, Nicole McCasker |
| 6.6 | Water quality and chemistry | Julia Howitt, Nicole McCasker, Mike Grace |
| 6.7 | Organic matter characterisation | Julia Howitt |
| 6.8 | Whole stream metabolism | Mike Grace |
| 6.9 | Shrimp | James Dyer, Anthony Conallin |
| 6.10 | Frogs | Sascha Healy, Nicole McCasker |
| 7 | Synthesis | Robyn Watts |
| 8 | Recommendations | Robyn Watts |

Report 2 May 2012

Monitoring of ecosystem responses to the delivery of environmental water in the Murrumbidgee system

Table of Contents

6.	Responses of indicators to Commonwealth environmental watering	4

6.6	Water quality and chemistry	4

6.7 	Organic matter characterisation	20

6.8	Whole stream metabolism	31

6.9	Shrimp	51

6.10	Frogs	63

7.	Summary and synthesis	70

8.	Recommendations	77

9.	Acknowledgements	80

10.	References	81

11.	Appendices	89

Responses of indicators to Commonwealth environmental watering

6.6	Water quality and chemistry

Photos: left, filtering water for carbon analysis; right, Colligen Creek near Niemur River offtake, 11 February 2014

Key findings

- Water quality was monitored through a combination of continuous logging (dissolved oxygen and temperature) and fortnightly spot measurements (EC, pH, turbidity) and samples (nutrients and organic carbon).

- Commonwealth environmental watering met the objective of maintaining or supporting water quality outcomes as it did not trigger any adverse water quality outcomes or a blackwater event. Commonwealth environmental water should continue to be delivered to mitigate adverse water quality events (see flow recommendations).

- Environmental watering actions did not significantly alter dissolved carbon or total carbon in treatment rivers relative to control rivers. A moderate increase in dissolved organic carbon was recorded during unregulated flows in August and September, resulting in a small but statistically significant difference between rivers during the Yallakool maintenance flow.

- The strongest influence on dissolved oxygen concentrations was a period of very hot weather in January and February 2014. No hypoxic blackwater event was observed during the study period.

- While total nitrogen and total phosphorus frequently exceeded ANZECC (2000) trigger levels, bioavailable nutrients were routinely extremely low in these rivers. Some statistically significant differences in nutrient concentrations were found when assessing effects of watering actions, but these differences were extremely small and not ecologically important.

Background

Changes in flow in a river system can influence water quality both positively and negatively with the outcome dependent on initial water quality, size of the flow, time of year, duration and other hydrological and catchment conditions. A range of water quality parameters are monitored as indicators of these changes and may be directly or indirectly affected by alterations in flow.  For example, dissolved oxygen and temperature are affected by flow through changes in water volume, depth, turbulence and through indirect processes, such as alterations in rates of bacterial metabolism and photosynthesis. Nutrients and organic matter concentrations may be reduced by dilution with increased flows or increase under flow conditions that result in water contacting parts of the channel or floodplain which were previously dry and which have stores of nutrients and carbon in both plant materials and the soil (Baldwin, 1999; Baldwin and Mitchell, 2000).

Reconnection of the stream channel with backwater areas may also result in additional nutrients and organic carbon. The majority of these additions will occur in the first day or two after rewetting or reconnection. Inputs of these substances may have a positive influence on the river community through the stimulation of productivity and increased food availability for downstream communities (Robertson et al., 1999). For example, the connection between the river and floodplain has been shown to generate essential carbon stores to sustain the system through drier periods (Baldwin et al., 2013). However, excessive nutrient and organic carbon inputs can result in poor water quality through the development of algal blooms or blackwater events resulting in very low dissolved oxygen concentrations (Howitt et al., 2007; Hladyz et al., 2011). Inputs of large amounts of organic matter and nutrients during hot weather are particularly problematic due to the influence of temperature on the rates of microbial processes and organic matter leaching (Howitt et al., 2007; Whitworth et al., 2014). This project aims to assess changes to water quality in response to alterations in flow and to consider changes in both the quantity and type of organic matter present in the system.

Hypotheses

- Environmental watering is expected to stimulate ecosystem productivity by moving nutrients and carbon between the main channel, benches and low commence-to-flow floodrunners.

- Watering actions are expected to result in only very small increases in nutrient concentrations, unless there is significant inundation of the floodplain.

- There will be a short-term increase in dissolved organic carbon and particulate organic carbon levels following environmental watering, but the watering actions are not expected to trigger blackwater events.

Methods

Water temperature and dissolved oxygen were logged every ten minutes at two sites within each of the four focus river reaches, with loggers located approximately 3-5 km apart. Data were downloaded and loggers calibrated approximately once per month depending on access. Light and depth loggers were also deployed during the commencement phase of the 2013-14 monitoring period and data were downloaded on a monthly basis. The data collected by the loggers was used to calculate daily average temperature (Figure 38) and dissolved oxygen concentrations (Figure 39) for each of the rivers from August 2013 to April 2014. Gaps in dissolved oxygen data were the result of problems with the loggers (discussed in more detail in Section 6.8 on Stream Metabolism).

Water quality parameters (temperature (oC), electrical conductivity (mS/cm), dissolved oxygen (%), pH, and turbidity (NTU)) were also measured as spot recordings fortnightly at two sites within each river reach.

Water samples were collected from four sites within each river, and from Stevens Weir on the Edward River, and processed according to the methods detailed in Watts et al. (2013b) to measure:

- Total Organic Carbon (TOC)

- Dissolved Organic Carbon (DOC) and Particulate Organic Carbon (POC)

- Nutrients (Ammonia (NH4+), filtered reactive phosphorus (FRP), dissolved nitrate + nitrite (NOx), Total Nitrogen (TN) and Total Phosphorus (TP))

An asymmetrical BACI (before-after, control-impact) (Underwood, 1991) statistical design was used to test the effect of the Yallakool Creek Perch Pulse Flow and the Colligen-Niemur River continuation flow on water chemistry parameters in the Edward-Wakool system. Differences in mean values between control/impact rivers and before/during/after environmental freshes were evaluated statistically for each watering action using 2-way nested ANOVA (nesting sampling date in period (before or during) and nesting river in control-impact) on un-transformed data. Impact rivers received environmental freshes, while Control rivers were those that did not receive environmental water. The impact river was either Yallakool Creek or Colligen Creek (depending on the watering action), and the control rivers were Little Merran Creek and the Wakool River for all occasions. A significant interaction between Period and CI (Contol/Impact) would suggest a significant effect of the watering action. A significant ‘Period’ suggests temporal or seasonal changes in all rivers are alike, and a significant ‘CI’ results suggests that rivers are just naturally different, and not changing differently as a result of receiving environmental water.

For the Yallakool Creek Cod maintenance flow, instead of a BACI approach, we tested whether POC, DOC and nutrient concentrations were significantly higher in rivers that received environmental flows, using a one way ANOVA, with River as the fixed effect, and sampling trip was included as a random effect. A Tukey’s post hoc test was run to examine the significant groupings of rivers based on mean DOC.

The monitoring results were assessed against ANZECC (2000) water quality guidelines trigger levels. Trigger levels are concentrations of key water quality parameters designed to provide guidance for ecological protection. Where it has been determined that the measured concentration of one or more parameters in the water body exceed this level (or for some parameters, fall outside the given range) the trigger levels are designed to ‘trigger’ further investigation to establish whether the concentrations are causing harm in that system. Exceedance of a trigger level is not an absolute indicator of ecological harm. The ANZECC water quality guidelines do not provide trigger levels for total organic carbon and dissolved organic carbon, and this reflects the expectation that there will be large variation in the ‘normal’ concentrations of organic carbon between ecosystems and also in the chemical and biological reactivity of the mixture of organic compounds making up the DOC and TOC at a particular site. Given the variable make-up of organic carbon, and the possible range of ecological responses to this mixture, a trigger level for this parameter would not be appropriate. However, trigger levels are provided for a number of nutrients and these are discussed below.

Results and Discussion

Water temperature was very consistent between sites and heavily influenced by ambient weather conditions (Figure 38). Particularly high water temperatures were recorded during mid January and early February, corresponding to prolonged heatwave conditions with air temperatures in excess of 40 oC. The dissolved oxygen concentrations recorded at these study sites remained at acceptable levels throughout the study period, indicating that no hypoxic blackwater event was associated with any of the environmental watering actions (Figure 39). The lowest recorded concentrations were associated with the heatwaves in summer, where high water temperatures act to both decrease oxygen solubility and increase the rates of microbial respiration, lowering the concentration of oxygen dissolved in the water column.  The Niemur River continuation flow was delivered via Colligen Creek in February to reduce the impact of this process in that river.

Figure 38. Average Daily Water Temperature (oC) in four rivers the Edward-Wakool system in 2013-14.

Figure 39. Average Daily Dissolved Oxygen concentrations (mg O2/L) in four rivers the Edward-Wakool system in 2013-14. Data is omitted where loggers were out of the water or malfunctioning.

Dissolved and particulate organic carbon

Dissolved organic carbon (DOC) concentrations were moderately elevated but very similar between all sites during the unregulated flow in August and September. A substantial decline was observed in the Wakool, Yallakool and Colligen in late September, consistent with improved water quality in the Edward River (Figure 40). The water in Little Merran Creek is sourced from the Murray River and did not reflect this decrease in DOC to the same extent. A small increase in DOC was observed in all rivers in October and then concentrations steadily declined to the low range (2-4 mg/L) normally observed for this system. Declines in DOC were slower in Little Merran Creek, consistent with the trend observed in previous years (Watts et. al. 2013a, Watts et. al 2013b). At no point in the study did the concentrations reach the high levels (between 15-30 mg/L) observed during the unregulated flow event in March and April 2012 (Watts et al. 2013a). Particulate organic carbon (POC) concentrations remained low throughout the study period (Figure 40).

Statistical analysis indicated that neither POC nor DOC concentrations were altered as a result of environmental watering actions (Table 19, Figure 41). There were statistically significant differences in DOC between rivers at the time of the Cod maintenance flow in Yallakool Creek (sampling dates 23/10/2/13 to 4/12/13)(Table 19). Little Merran Creek had the highest DOC levels (4.15 mg C/L), followed by Wakool (3.75 mg C/L), while Yallakool and Colligen had the least (3.5 mg C/L) (Figure 41c). While statistically different, the difference between 3.5 and 4.1 mg C /L is not ecologically significant. The difference between Yallakool Creek, Colligen Creek, the Wakool River and Little Merran Creek can be explained by a combination of the slower decline in the hydrograph for Little Merran Creek following the spring unregulated flows and the considerably larger wetted surface area at relatively low flows due to the stream morphology of this river (Watts et al 2013b). The larger wetted surface area will result in greater contact between the water and both soil organic carbon and organic matter derived from leaf litter, resulting in leaching of DOC into the water column. In addition, Little Merran Creek receives water from the Murray River rather than the Edward River and carbon inputs may be influenced by transit through the Koondrook State Forest.

POC was not significantly affected by environmental watering for Yallakool perch pulse, maintenance, or Colligen-Niemur continuation flows (Table 19). The DOC significant ‘period’ (p<0.001) means that DOC concentrations changed in all rivers from before to during the time that Yallakool was experiencing the perch pulse flow. This decrease (Figure 41) affected all rivers alike and is a result of sampling across the period where the DOC was still declining following the earlier unregulated flows.

Figure 40. Mean (±SE) concentrations of a) dissolved organic carbon and b) particulate organic carbon in Yallakool Creek, Colligen Creek, Wakool River, Little Merran Creek and the Edward River between August 2013 and April 2014. Blue green bars represent the start and finish dates of environmental watering.

Table 19. Statistical results for 2 way mixed-effects Analysis of Variance (ANOVA) on DOC and POC results. A significant interaction between the two fixed factors: Period (before, during, after/recession) and CI (control rivers, impact rivers) indicates that the concentrations of DOC or POC within Impact Rivers and Control rivers responded different across time. A one way ANOVA was run for Cod maintenance flows to test for differences across Rivers. Results highlighted in bold were statistically significant.

|  | WQ Parameter | Main effect | d.f | F-test | p-value |
| --- | --- | --- | --- | --- | --- |
| Perch pulse flow: Yallakool creek |  |  |  |  |  |  |
|  | DOC | Period (B-D) | 1,44 | 14.014 | <0.001 |  |
|  |  | CI (C-I) | 1,44 | 1.437 | 0.237 |  |
|  |  | Period*CI | 1,44 | 2.976 | 0.091 |  |
|  | POC | Period (B-D) | 1,44 | 0.141 | 0.708 |  |
|  |  | CI (C-I) | 1,44 | 0.001 | 0.964 |  |
|  |  | Period*CI | 1,44 | 1.43 | 0.238 |  |
| Cod maintenance: Yallakool creek |  |  |  |  |  |  |
|  | DOC | River | 4,72 | 22.391 | <0.001 |  |
|  | POC | River | 4,72 | 0.266 | 0.897 |  |
| Colligen-Niemur continuation flow |  |  |  |  |  |  |
|  | DOC | Period (B-D) | 1,43 | 0.780 | 0.382 |  |
|  |  | CI (C-I) | 1,43 | 0.065 | 0.799 |  |
|  |  | Period*CI | 1,43 | 0.039 | 0.843 |  |
|  | POC | Period (B-D) | 1,43 | 0.401 | 0.529 |  |
|  |  | CI (C-I) | 1,43 | 0.112 | 0.738 |  |
|  |  | Period*CI | 1,43 | 1.679 | 0.201 |  |

Figure 41. Statistical comparison of DOC and POC concentrations between treatment and control rivers for a) the Colligen-Niemur continuation flow (BACI), b) the Yallakool perch pulse flow (BACI), and c) the Yallakool maintenance flow. The letters above the bars indicate significant differences in DOC between rivers during the Yallakool maintenance flow.

Nutrients

Concentrations of Total Nitrogen, NOx (nitrate plus nitrate) and ammonia (Figure 42) indicate that as observed in previous years (Watts et al 2013a, Watts et al 2013b) the nitrogen profile of Little Merran Creek is different to the other rivers during the spring, reflecting differences in both source water and flow regime (Figure 42).  All forms of nitrogen were elevated in the Little Merran relative to the other rivers over August and early September.  There was a slight increase in bioavailable nitrogen in the other rivers during the unregulated flow in August with a rapid decline in September. From October onwards, nitrogen concentrations in all rivers were similar to each other.

As shown in Figure 42, although the TN concentrations at times exceeded the ANZECC (2000) trigger value for lowland rivers of 0.5 mg N/L, the mean and median values were below the trigger level for all rivers. The bioavailable forms of nitrogen (NOx and ammonia) were well below the trigger values on all occasions with the exception of Little Merran Creek, where NOx exceeded the trigger value in August and early September and ammonia at the beginning of the sampling period in August. Median NOx concentrations were at or below the limit of detection for all but Little Merran Creek.

This pattern is repeated with the concentrations of phosphorous (P) in the rivers: while Total P concentrations (Figure 43a) frequently exceeded the trigger value of 0.05 mg P/L with median and mean concentrations for all rivers at or above this trigger level, the filterable reactive P (the bioavailable fraction) was well below the trigger value of 0.02 mg P/L (Figure 43b). A slight increase in bioavailable P occured in August/September and was greatest in Little Merran Creek but FRP was not impacted by any of the environmental watering actions. The ANZECC Water Quality Guidelines recommend that in order to minimize the risk of algal blooms (and other adverse outcomes) in lowland rivers in south-eastern Australia, these bioavailable concentrations should be below 0.02, 0.02 and 0.04 mg/L for ammonia, FRP and NOx respectively. The median concentrations were nearly an order of magnitude lower than these guidelines in most instances.

Figure 42. Mean (±SE) concentrations of a) total nitrogen, b) nitrate and nitrite, and c) ammonia in the Yallakool River, Colligen Creek, Wakool River, Little Merran Creek and Edward River between August 2013 and April 2014. Blue and green bars represent the start and finish dates of environmental watering. The dashed line indicates the AANZECC (2000) trigger levels for each nutrient.

Figure 43. Mean (±SE) concentrations of a) total phosphorus, and b) filterable reactive phosphorus in the Yallakool River, Colligen Creek, Wakool River, Little Merran Creek and Edward River between August 2013 and April 2014. Blue and green bars represent the start and finish dates of environmental watering. The dashed line indicates the AANZECC (2000) trigger levels for each nutrient.

Table 20 summarizes the findings of the BACI analysis for the effects of environmental watering on. each nutrient for the Colligen-Niemur continuation flow, the Yallakool perch pulse flow and the Yallakool cod maintenance flow. The data used in these analyses is summarized in Figures 44 to 46.

There were no effects of the perch pulse flow on TP, NH4+, FRP, or NOX due to perch pulse flow (Table 20, Figure 44). TN levels changed from after the perch pulse flow, but that this happened in control and impact rivers alike suggesting the changes were due to seasonal factors (Table 20, Figure 44a).

There were statistically differences in NOx, ammonia and FRP (but not TN or TP) across rivers during the Yallakool cod maintenance flow (Table 20), but these differences arose due to slightly higher concentrations in Colligen Creek rather than Yallakool Creek (Figure 45 c, d, e). A change of ± 1-2 µg N or P/L is only slightly greater than the precision in the analytical chemistry method (0.5 µg N or P/L by Flow Injection Analysis) and certainly does not reflect a major new source of nutrients. As noted earlier, the concentration levels are extremely low and definitely a constraint on primary production in these streams (see Section 6.8 on Stream Metabolism).

There was a significant interaction effect between the control and impact streams for FRP (only) before and after the Colligen-Niemur continuation flow (Table 20, Figure 46). However, it was an increase in the control streams (Wakool River and Little Merran Creek) rather than the impact stream that caused this statistically significant difference (Figure 46d). Again, the actual concentration change was extremely small (<1 µg P/L), and not considered to be ecologically significant.

There was a significant difference between control and impact rivers for NH3, in that control rivers had slightly more than the Colligen Creek, however the difference was not ecologically significant (Table 20, Figure 46).

Overall, there were no ecologically significant changes in carbon or nutrient concentrations in reponse to environmental watering actions, consistent with previous years (Watts et. al 2013a, Watts et al 2013b). Increases in carbon and nutrients in these rivers have been observed in response to unregulated flow events and stimulation of productivity through inputs of carbon and bioavailable nutrients in this system is expected to require either the import of these substances from upstream (requiring larger flows from upstream and the water having been in contact with the floodplain) or larger inundation events occurring locally which reconnect previously dry sections of the in-channel habitat with the water column.

Table 20. Statistical results for 2 way mixed-effects Analysis of Variance (ANOVA) on Nutrient Concentrations. A significant interaction between the two fixed factors: Period (before, during, after/recession) and CI (control rivers, impact rivers) indicates that the relevant nutrient concentration within Impact Rivers and Control rivers responded different across time. An effect due to the watering action could be determined if the significant interaction was due to the relevant mean concentration changing in the impact river as compared to the control river, not vice-versa (See accompanying Figures 44-46). A one way ANOVA was run for Cod maintenance flows (see methods) to test for differences across Rivers.

|  | WQ Parameter | Main effect | d.f. | F-test | p-value | Significant effect due to watering action? |
| --- | --- | --- | --- | --- | --- | --- |
| Perch pulse flow: Yallakool creek |  |  |  |  |  |  |
|  | TN | Period (B-D) | 1,44 | 25.200 | <0.001 |  |
|  |  | CI (C-I) | 1,44 | 0.132 | 0.717 |  |
|  |  | Period*CI | 1,44 | 0.069 | 0.793 | No |
|  | NOX | Period (B-D) | n/a |  |  |  |
|  |  | CI (C-I) | n/a |  |  |  |
|  |  | Period*CI | n/a |  |  | No |
|  | NH4+ | Period (B-D) | 1,41 | 0.033 | 0.856 |  |
|  |  | CI (C-I) | 1,41 | 0.236 | 0.629 |  |
|  |  | Period*CI | 1,41 | 0.048 | 0.826 | No |
|  | TP | Period (B-D) | 1,44 | 0.000 | 0.985 |  |
|  |  | CI (C-I) | 1,44 | 0.250 | 0.619 |  |
|  |  | Period*CI | 1,44 | 0.073 | 0.832 | No |
|  | FRP | Period (B-D) | 1,41 | 1.491 | 0.229 |  |
|  |  | CI (C-I) | 1,41 | 0.255 | 0.615 |  |
|  |  | Period*CI | 1,41 | 0.045 | 0.832 | No |
| Cod maintenance: Yallakool creek |  |  |  |  |  |  |
|  | TN | River | 3,57 | 2.081 | 0.112 | No |
|  | NOX | River | 3,53 | 5.147 | 0.003 | No |
|  | NH4+ | River | 3,53 | 4.854 | 0.004 | No |
|  | TP | River | 3,57 | 1.043 | 0.380 | No |
|  | FRP | River | 3,53 | 5.536 | 0.002 | No |
| Colligen-Niemur continuation flow |  |  |  |  |  |  |
|  | TN | Period (B-D) | 1,43 | 0.539 | 0.466 |  |
|  |  | CI (C-I) | 1,43 | 0.034 | 0.854 |  |
|  |  | Period*CI | 1,43 | 1.701 | 0.199 | No |
|  | NOX | Period (B-D) | 1,43 | 0.000 | 0.987 |  |
|  |  | CI (C-I) | 1,43 | 1.398 | 0.243 |  |
|  |  | Period*CI | 1,43 | 0.077 | 0.781 | No |
|  | NH4+ | Period (B-D) | 1,43 | 0.273 | 0.603 |  |
|  |  | CI (C-I) | 1,43 | 5.371 | 0.025 |  |
|  |  | Period*CI | 1,43 | 0.137 | 0.712 | No |
|  | TP | Period (B-D) | 1,43 | 1.216 | 0.276 |  |
|  |  | CI (C-I) | 1,43 | 0.345 | 0.560 |  |
|  |  | Period*CI | 1,43 | 0.000 | 0.981 | No |
|  | FRP | Period (B-D) | 1,43 | 1.435 | 0.237 |  |
|  |  | CI (C-I) | 1,43 | 0.000 | 0.989 | No |
|  |  | Period*CI | 1,43 | 5.600 | 0.022 | Yes |

Figure 44. Yallakool Perch flow: BACI comparison of nutrient concentrations between treatment and control rivers before and after the watering action. Tthe first bar of each pair is before (B) the watering action, the second is after (A) the watering action.

Figure 45. Yallakool maintainence flow: Comparison of nutrient concentrations between treatment and control rivers during the Yallakool sustained flow. The letters above the bars indicate significant differences between rivers.

Figure 46. Colligen-Niemur continuation flow: BACI comparison of nutrient concentrations between treatment and control rivers for the Colligen-Niemur continuation flow – the first bar of each pair is before (B) the watering action, the second is after (A) the watering action.

6.7 	Organic matter characterisation

Photo: Example of blackwater in a redgum forest. This type of event did not occur n 2013-14 at the sites monitored in the Edward-Wakool system.

Key findings

- Water samples to facilitate the characterisation of organic matter were collected fortnightly from August 2013 until April 2014. The organic matter profiles of all river sites were generally very similar to each other for a given sampling date, with the exception of the Little Merran Creek during spring.

- Commonwealth environmental watering met the objective of maintaining or supporting water quality outcomes as it did not trigger any adverse water quality outcomes. No hypoxic blackwater event occurred during this study period and environmental watering actions did not result in changes to the organic matter profiles. Commonwealth environmental water should continue to be delivered to mitigate adverse water quality events (see flow recommendations).

- Organic matter composition was generally more complex early in the season and simplified over time, consistent with previous years, with the exception of some sites at the end of January where minor modification of the organic matter may have resulted from the prolonged heatwave.The fluorescence results indicate that the low levels of organic matter in these river systems over summer are generally weighted towards carbon of low bioavailability.

Background

Australian riverine ecosystems can be heavily reliant on both algal and terrestrial dissolved organic matter for microbial productivity and can be limited by dissolved organic carbon concentrations (Hadwen et al., 2010). While water quality guidelines (ANZECC 2000) include trigger values (concentrations of concern) for a number of water quality parameters such as chlorophyll a, nutrients, dissolved oxygen and pH, no guidelines are given for organic matter and aquatic environments are expected to have quite varying dissolved organic matter concentrations.

Organic matter is made up of a complex mixture of compounds from a diverse range of sources. Microbial communities do not respond to all types of organic matter in the same way (Baldwin 1999; O’Connell et al. 2000; Howitt et al. 2008) although it has been shown that bacterial communities can respond to changes in organic carbon source quite rapidly. The very large, complex type of organic matter referred to as humic substances has been shown to be less available to bacterial communities than simpler non-humic carbon (Moran and Hodson 1990) although this can be altered over time with exposure to ultraviolet light (Howitt et al. 2008).

One way of examining the mixture of organic substances present is to measure the spectroscopic behaviour of the substances- i.e. to study which wavelengths of light they absorb, and to examine which wavelengths of light they emit (fluoresce) in response to this absorption of incoming light (Dahlen et al. 1996; Mobed et al. 1996, Baker and Spencer, 2004; Howitt et al. 2008). Both absorbance and fluorescence spectra are used to examine the organic matter in this study. As a general guide, absorbance at longer wavelengths indicates larger, more complex organic matter (Bertilsson and Bergh, 1999).  Absorbance at a particular wavelength may be increased by increasing concentration of organic matter or a change in the type of organic matter.

The consideration of changes in both the quantity and type of organic matter present in the system allows for a more detailed examination of the hypotheses around the movement of organic carbon and the creation of blackwater events.

Hypothesis

If environmental watering facilitates inundation of extensive new sources of organic matter it would result in changes to the spectroscopic responses of the organic matter through alteration of the mixture of compounds that make up dissolved organic matter.

Methods

Samples for organic matter characterisation were collected fortnightly from August 2013 until April 2014. Four samples were collected from each river reach on each sampling date. Samples from the early October field trip were compromised during transport and were not analysed. On one occasion samples from the Wakool sites were not available due to access issues (29/1/14) and water collected from the Wakool River futher upstream, immediately above the Mulwala Canal escape. No water was being released from the canal so these results are expected to be representative.  Colligen Creek samples could not be collected in April.

Water samples were filtered through a 0.2 m pore-sized membrane at the time of sampling and then stored on ice until returned to the laboratory and then analysed within a day of returning from the field. Absorbance scans were collected using a Varian Cary 4000 instrument across a wavelength range of 550 nm to 200 nm (green through to ultraviolet) with a 1 nm step size.  Absorbance is a measure of light absorbed by the sample and is a logarithmic scale. An absorbance of 1 indicates that only 10% of the light of that wavelength is transmitted through the sample. Fluorescence scans were collected using a Varian Eclipse spectrofluorometer scanning both emission and excitation wavelengths to give an excitation-emission matrix (EEM). Excitation wavelengths were scanned from 200 to 400 nm with a 10 nm step size and for each excitation wavelength, emission of light at 90O to the source was recorded from 200 nm to 550 nm with a 1 nm step size. Fluorescence results were corrected for sample absorption and plotted as contour plots (Howitt et al., 2008). To correct for drift in the instrument zero position, each contour plot was scaled by subtracting the average emission intensity across the range 200-210 nm for an excitation of 250 nm from all fluorescence intensities (effectively setting this region of the contour plot to zero on all plots).

An example of a contour plot is shown in Figure 47. The contour plots have the excitation wavelength (light shone into the sample) on the y-axis. On the x-axis is the emission wavelength (light given off by the sample). The intensity of the fluorescence (how much light is given off, corrected for absorbance by the sample) is represented by the colours of the contour plot, with more intense fluorescence represented by the blue end of the scale. The two blue diagonal lines are artefacts of the technique and will be present in all samples- key data is found between these two lines.

Figure 47. Sample excitation emission contour plot indicating key features of the data. (Watts et al. 2013a)

Results and discussion

Absorbance scans of representative water samples from each sampling trip are presented in Figure 48. The greatest variation between replicates for each river is expected at 200 nm and the range was commonly less than 5% of the measured absorbance for each river on most occasions, and the maximum range observed for each sampling trip is in Table 21. An exception to this pattern was observed in late January where the variation between replicates was increased in all rivers to between 9 and 25% with Colligen Creek and Little Merran Creek being the most affected. In general, these data indicate that the greatest influence on organic matter composition and concentration over the study period were the unregulated flows in August and September 2013, where all rivers have elevated organic matter compared to May 2013 (Watts et al. 2013b). Over the period from August to October 2013, the organic matter content of Little Merran Creek is generally slightly higher than that found at all the other sample sites, consistent with the DOC results, although the absorbance results imply a more gradual decline in organic matter than the DOC (the sharp dip in late September in the DOC results is less pronounced in the absorbance results). Throughout the study period the absorbance scans for the Wakool River, Colligen Creek and Yallakool Creek closely match the source water from the Edward River, where overbank flows through the Barmah-Millewa Forest can be a source of organic matter upstream. This pattern is consistent with that observed in previous years. The in-channel flows in both Yallakool Creek and Colligen Creek do not result in the absorbance scans at these sites becoming substantially different to the other sites, again supporting earlier conclusions that the areas of in-stream habitat that were re-wet during these flows did not have substantial amounts of accumulated organic material (such as leaf litter) (Watts et al. 2013b).

One method for assessing the relative size of the organic matter in a water sample is to compare the ratio of absorbances at shorter and longer wavelengths, as larger, more complex molecules absorb at longer wavelengths. Table 21 shows the ratio of absorbance at 250 nm to that at 340 nm, based on average value of three replicates for each river. The rivers generally have very similar ratios to each other, and the ratios are in the range previously reported for aged organic matter as found in billabongs (Howitt et al. 2008). There is a very gradual increase in ratio through the sampling season, indicating a relative loss of the larger humic and fulvic materials compared to the smaller molecules, and this trend commences earlier in the rivers with the lowest summer flows (Little Merran Creek and Wakool River), although it should be remembered that these changes are very small and that the overall DOC concentration is quite low by the time these changes are evident.

Representative fluorescence scans are shown in Figure 49. Being a more sensitive spectroscopic technique, the fluorescence results have more dramatically shown the reduction in organic matter between the two sampling dates in September, but also support the conclusion that the Colligen, Yallakool and Wakool sites have organic matter profiles that closely reflect the water quality entering from the Edward. It is noted that the humic peak in the Edward river samples is slightly weaker than the other rivers on the 11/9/13 and this is likely an indication that the decline in organic matter inputs had just commenced upstream. The fluorescence analysis indicates that by late November 2013 the amount and type of organic matter at all sites is very consistent and low, as has been observed in previous years. Consistent with the absorbance results presented above, there is no evidence that the freshes in the Colligen and Yallakool have resulted in changes in the organic matter load or the composition of the mixture of compounds making up the fluorescent dissolved organic matter. There are small increases in the aromatic protein region of the spectrum in all rivers during January and these persist through to April 2014, consistent with the change in absorbance ratios indicating an increased proportion of smaller molecules. This is a seasonal effect consistent with previous years (Watts et al. 2013a; Watts et al. 2013b). Increases in fluorescence in this region of the spectrum may indicate the presence of a more bioavailable fraction of organic matter and may be associated with breakdown of humic and fulvic components by sunlight (Howitt et al. 2008), however it should be noted that the concentrations of these substances remain in a low range. Peaks in this region have also been associated with bacterial metabolism (Elliott et al. 2006), and have been found in marine environments (Coble 1996) where their presence is likely of algal origin.

The reduced oxygen concentrations observed during the heatwave in January 2014 were not accompanied by increases in dissolved organic carbon, however, there was greater variability in the absorbance and fluorescence results for replicates from the same river during this period, and some fluorescence results had peaks in a region not normally present for this river system (see for example, Colligen and Wakool scans from 29/1/14). These additional peaks suggest localised algal production, or products of photochemical degradation of humic and fulvic materials

The patterns observed in the absorbance and fluorescence results are consistent with those observed in previous sampling seasons (Watts et al. 2013a; 2013b). Higher flows during early spring, associated with unregulated river rises, increase the amount and complexity of organic matter in the rivers. The rivers may be different from each other due to varying amounts of newly wetted habitat or where overbank flow may occur for the source rivers upstream, but long periods of in-channel flow result in a reduction and simplification of the organic matter signals and there is consistency across the study sites.

The small in-channel watering actions did not reconnect sufficient area of upper benches and floodrunners to result in substantial exchange of organic matter and nutrients. Therefore, the hypothesis that environmental watering would stimulate ecosystem productivity by moving nutrients and carbon between the main channel, upper benches and small, low commence-to-flow floodrunners was not upheld for the in-channel watering actions in 2013-14.

It was expected that dissolved organic carbon and particulate organic carbon levels would remain relatively unchanged following in-channel environmental watering or that small increases would occur. The environmental watering was not expected to trigger blackwater events in these systems. These hypotheses were supported: in-channel environmental watering did not result in large inputs of dissolved or particulate organic matter and no blackwater event was observed. Organic matter inputs associated with the unregulated flows in August and September 2013 did not result in a blackwater event, primarily due to the low water temperature at this time of year.

Figure 48a. Absorbance Scans for water samples from August to November 2013.

Figure 48b Absorbance Scans for water samples from December 2013 to mid February 2014.

Figure 48c.  Absorbance scans for water samples from late February to April 2014.

Figure 49a Representative fluorescence scans for water samples collected between August and November 2013.

Figure 49b Representative fluorescence scans for water samples collected between December 2013 and mid February 2014. Additional peaks present in the Colligen Creek and Wakool River at the end of January are marked with black circles.

Figure 49c. Representative fluorescence scans for water samples collected between late February and April 2014.

Table 21. Average ratio of absorbance at 250 nm to 340 nm for 3 replicate samples for each river, and the maximum range in absorbances observed between replicates from a single river at 200 nm.

| Date | Colligen | Yallakool | Wakool | Little Merran | Edward | Max range A200 (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 29/08/13 | 2.95 | 3.00 | 3.00 | 3.06 | 3.00 | 7 |
| 11/09/13 | 3.01 | 3.02 | 3.00 | 3.07 | 3.02 | 4 |
| 25/09/13 | 3.03 | 3.01 | 3.02 | 3.20 | 3.02 | 5 |
| 25/10/13 | 3.01 | 3.00 | 3.02 | 3.09 | 2.99 | 3 |
| 6/11/13 | 3.07 | 3.05 | 3.06 | 3.05 | 3.03 | 5 |
| 20/11/13 | 3.03 | 3.01 | 3.09 | 3.05 | 3.03 | 5 |
| 4/12/13 | 3.01 | 2.99 | 3.12 | 3.15 | 3.04 | 6 |
| 18/12/13 | 3.00 | 3.09 | 3.11 | 3.14 | 3.04 | 13 |
| 1/01/14 | 3.18 | 3.12 | 3.19 | 3.22 | 2.85 | 8 |
| 15/01/14 | 3.12 | 3.06 | 3.09 | 3.12 | 3.03 | 7 |
| 29/01/14 | 3.06 | 3.21 | 3.18 | 3.17 | 3.10 | 25 |
| 12/02/14 | 3.13 | 3.10 | 3.19 | 3.18 | 3.05 | 5 |
| 26/02/14 | 3.15 | 3.10 | 3.25 | 3.21 | 3.12 | 12 |
| 12/03/14 | 3.17 | 3.01 | 3.18 | 3.27 | 3.18 | 6 |
| 26/03/14 | 3.12 | 3.26 | 3.18 | 3.31 | 3.22 | 10 |
| 9/04/13 |  | 3.21 | 3.22 | 3.25 | 3.32 | 5 |

6.8	Whole stream metabolism

Photo: Dissolved oxygen logger being calibrated in the field

Key findings

- Battery-powered loggers were deployed at either end of each of the four focus reaches and measure and loggged dissolved oxygen concentration and water temperature every 10 minutes. The metabolic parameters, gross primary production and ecosystem respiration were calculated for each day.

- Rates of primary production and ecosystem respiration during 2013-2014 were at the lower end of the normal range found in streams worldwide.

- Only very small increases in metabolic rates resulted from environmental watering actions, most likely because the flows were contained within the stream channel with little inundation of backwater areas or instream geomorphic features, such as benches. there is a need to increasing our understanding of interaction between instream flows and river productivity (see flow recommendations).

- Primary production and ecosystem respiration are constrained by the very low concentrations of bioavailable nutrients and organic carbon, respectively. This is both beneficial in that the risks of severe algal blooms or anoxic events are very low, but problematic if this lack of organic carbon input into the base of the aquatic foodweb is limiting maintenance and recovery of fish populations.

Background

Whole stream metabolism measures the production and consumption of dissolved oxygen gas (‘DO’) by the key ecological processes of photosynthesis and respiration (Odum 1956). Healthy aquatic ecosystems need both processes to generate new biomass (which becomes food for organisms higher up the food chain) and to break down plant and animal detritus to recycle nutrients to enable growth to occur. Hence measuring metabolism provides a mechanism for assessing the energy base underpinning aquatic foodwebs. The relationships between these processes are shown in Figure 50.

Figure 50. Relationships between photosynthesis, respiration, organic matter, dissolved gases and nutrients

Rates of metabolism are typically expressed as the increase (photosynthesis) or decrease (respiration) of dissolved oxygen concentration over a given time frame. For stream metabolism studies, the most commonly used unit is (change in) milligrams of dissolved oxygen per litre per day (mg O2/L/Day). Sometimes, photosynthetic and respiratory quotients are used to convert these oxygen-based measurements to carbon units, although that is not required here. Typical rates of primary production and ecosystem respiration range over two orders of magnitude, from around 0.2 to 20 mg O2/L/Day with most measurements falling between 0.5 and 10 mg O2/L/Day.

If the rates of these processes are too low, this will limit the amount of food (in the form of bacteria, algae and water plants) available to organisms that consume these entities. This limitation will flow on up through the food web and constrain the populations of larger organisms including fish and amphibians. Rates are expected to vary on a seasonal basis as warmer temperatures and more direct, and longer hours of, sunlight contribute to enhancing primary production. Warmer temperatures and a supply of organic carbon usually result in higher rates of ecosystem respiration (Roberts et al. 2007).

In general, there is concern when rates of these processes are too high. Greatly elevated primary production rates usually indicate probable algal bloom conditions (or excessive growth of plant species, including duckweed and azolla). Several deleterious effects can then occur – blocking of sunlight penetration into the water which can kill off other submerged plants, possible production of potent algal toxins and large swings in diel (over the course of 24 hours) DO. Although photosynthesizing algae can produce extremely high concentrations of DO during daylight hours, overnight, elevated respiration rates can drive the DO concentration very low, even to the point of anoxia (no dissolved oxygen in the water). Also, when an algal bloom eventually collapses, the very large biomass of labile organic material is then respired, often resulting in anoxia for extended periods. Very low (or no) DO in the water can be fatal to many organisms, resulting in fish kills and unpleasant odors. Bloom collapse is also often, but not always, coincident with release of algal toxins; hence the water becomes unusable for stock and domestic purposes as well.

After allowing for seasonal variability, sustainable rates of primary production for a given system will primarily depend on the characteristics of the aquatic ecosystem being considered. Streams with naturally higher concentrations of nutrients (e.g. arising from the geology), especially those with very open canopies (hence lots of sunlight access to the water) will have much higher natural rates of primary production than forested streams, where rates might be extremely low due to heavy shading and low concentrations. The important point is that the ecology of each stream will develop based (partly) on the rates of these fundamental processes. Habitat availability, climate and many other factors also influence food web structure and function. Uehlinger (2000) demonstrated that freshes with sufficient stream power to cause scouring can ‘reset’ primary production to very low rates which are then maintained until biomass of primary producers is re-established.

Hypotheses

The following hypotheses were developed specifically related to the possible impact of environmental watering on the rates of stream metabolism and were derived from a combination of the work of Vink et al. (2005) on impacts of irrigation releases on the middle reaches of the Murrumbidgee River, the predictions of the ‘Flood Pulse Concept’ (Junk et al. 1989) and findings from previous measurements on these streams (Watts et al. 2013b):

- Increased flow via an environmental watering action will result in an initial decline in Gross Primary Production (GPP) due to the enhanced turbidity from that higher flow causing light limitation of photosynthesis.

- Following that initial decline, GPP will increase to a higher rate than pre-watering action levels due to an influx of bioavailable nutrients.

- There will be an initial decline in Ecosystem Respiration (ER) followed by a rapid increase as more labile organic carbon is introduced into the stream channel during environmental watering.

- Changes in stream metabolism will be relatively small (< 5 mg O2/L/Day) unless the increased flow results in inundation of backwaters and perhaps the floodplain.

Methods

Rates of primary production and respiration can readily be estimated by continuously recording DO, temperature and ambient light (known as photosynthetically active radiation or PAR) over periods of many months under a range of meteorological conditions and flows (Grace and Imberger, 2006). Rates are calculated for each day so that the influence of flow, cloud cover and season can be assessed. A typical daily DO trace is shown in Figure 51. The figure illustrates how DO changes in a sinusoidal pattern over 24 hours, with DO increasing after sunrise, reaching a peak in late afternoon (due to photosynthesis throughout daylight hours) and then declining overnight, as ecosystem respiration (which consumes DO) continues 24 hours a day. Estimation of these rates must also include one additional factor – reaeration, which is the physical movement of DO across the air water interface. If the water contains less DO than it can hold (100% saturation), such as overnight when respiration consumes DO, then more oxygen will diffuse from the atmosphere into the water.

Figure 51. Typical Dissolved Oxygen Concentration Profile over a 24 hour period. This unsmoothed data was from Yallakool Creek on the 6th October, 2013. The signal may appear noisy but is actually very stable given the small diurnal change in concentration.

Battery-powered ZebraTech data sondes (DO Loggers) were deployed mid-stream, with one sonde at either end of each of the four focus reaches separated by 3-5 km of stream channel. Each sonde was set to measure and log dissolved oxygen concentration and water temperature every 10 minutes. Approximately every month, data were downloaded from the sondes, which were then recalibrated to ensure high quality data. PAR data were also recorded every 10 minutes on loggers placed in open fields near each of the stream sites. The metabolic parameters, gross primary production and ecosystem respiration, plus the reaeration rate were calculated for each day using the daytime regression method described by Atkinson et al. (2008), implemented in the Bayesian ‘BASE’ model (Grace et al. 2014). Only data where the diel curve reasonably matched the shape shown in Figure 51, and the ‘BASE’ model produced excellent fits to these diel curves, were included in the subsequent analysis of the metabolic rates.

Unlike previous deployments in 2011-2013, some major problems were experienced by many of the sondes post-Christmas 2013. Whilst still the subject of ongoing discussions between the project team and suppliers, it appears that a more ‘vigorous’ cleaning approach may be required to prevent buildup of biofilms on and near the membrane. This buildup is the likely cause of the highly erratic day-time DO signals recorded by many loggers. This meant that there were significant periods – especially from February through to the end of April – when there was almost no usable data. This unfortunate outcome constrained assessment of any ‘after’ effects from the watering actions.

Statistical analyses were performed to examine the hypotheses. As daily metabolic rates are affected by weather and water temperature, the potential impacts of environmental watering actions were assessed by determining whether there was a change in the difference in daily rates between the impacted streams (Yallakool or Colligen Creeks) and the control streams (Wakool River or Little Merran Creek). For example, the ‘pre-watering action’ difference in GPP between Yallakool Creek and Wakool River was calculated each day. This ‘before’ data was then compared to the differences in GPP in the same two streams ‘during’ the watering action. Unlike 2012-2013, the combination of the extended nature of the elevated flows in Yallakool Creek or the late summer watering action in Colligen Creek (as part of the Niemur River water quality improvement action) and the aforementioned difficulties with the DO sondes meant that there was no ‘after’ comparison possible.

For all planned comparisons, the Student t-test was used to test for statistically significant differences (at p < 0.05) between the ‘before’ and ‘during’ data sets assuming that the Shapiro-Wilk normality requirement was met in the two data sets (at p < 0.05). If normality conditions were not met, then a Mann-Whitney Non-Parametric Rank Sum Test was performed.

Results

Metabolic Rates in the four study streams

The rates of both primary production and respiration remain relatively constant over the entire study period (Figures 52 to 55). There were some longer term (seasonal) trends where both primary production and respiration rates increased during summer due to warmer water temperatures, and in the case of primary production, more hours of sunshine. At no stage between September and April did any of the loggers record very low dissolved oxygen concentrations (< 20% DO saturation), even during and immediately after the watering actions. For three of the four focus rivers (Colligen Creek, Wakool River and Yallakool Creek), the %DO saturation barely dropped below 70% saturation, while the other river (Little Merran Creek) only very occasionally fell below 60%. At no point in this period did any river fall below 50% saturation. These findings are consistent with the organic matter characterization results (section 6.7). None of these watering actions resulted in water spreading out onto the floodplain, remaining for days-weeks and then return, which is the most common scenario for development of ‘black water’ (high dissolved organic carbon and very low or no dissolved oxygen).

Table 22 provides summary data for the gross primary production (GPP) and ecosystem respiration (ER) rates for each river. The table lists the lowest and highest rates found, plus the median value. The P/R ratio refers to the balance between primary production rates and ecosystem respiration rates. A value for this ratio of < 1 indicates that more organic carbon is being consumed in the study reach than is being produced by primary production (photosynthesis).

Table 22. Summary of primary production (GPP) and ecosystem respiration (ER) rates for the four study rivers (combined upstream and downstream sonde data), September 2013 - April 2014.

Figure 52. Gross Primary Production (GPP) rates and stream discharge for the Wakool River and Yallakool Creek from September 2013 through April 2014. The paucity of data from mid-February onwards is due to problems with the DO sondes.

Figure 53. Gross Primary Production (GPP) rates and stream discharge for Colligen and Little Merran Creeks from September 2013 through April 2014. The paucity of data from Colligen Creek from mid-February onwards is due to problems with the DO sondes.

Figure 54. Ecosystem Respiration rates and stream discharge for the Wakool River and Yallakool Creek from September 2013 through April 2014. The paucity of data from mid-February onwards is due to problems with the DO sondes.

Figure 55. Ecosystem respiration rates and stream discharge for Colligen and Little Merran Creeks from September 2013 through April 2014. The paucity of data from Colligen Creek from mid-February onwards is due to problems with the DO sondes.

The gross primary productivity and ecosystem respiration rates in the four focus rivers (Table 22) are extremely similar between the four rivers, with median primary production rates and median respiration rates differing by less than a factor of 2 between the four rivers. This is largely unsurprising given the similarity in land forms, land use, the sizes of the rivers and that they are all in the same region. The median values for the P/R ratio (0.48 – 0.66; 0.44 – 0.77 in 2012-2013) indicate that for most of the time, these rivers are net heterotrophic (P/R < 1) implying that there must be another source of organic carbon fuelling respiration. Such carbon additions generally come from further up in the catchment or have fallen in/been washed in from riparian vegetation. It is pertinent to note though that primary production within the stream channel, as measured here by GPP, is also an extremely important contributor to organic carbon supply (which is the food sustaining higher organisms in the food web).

Figure 56 clearly demonstrates that there was also a very large variation in GPP at any particular light value, coupled with the expected increase in GPP with increasing daily light flux. Similar results were obtained for GPP-PAR relationships for Colligen Creek (slope = 0.055, R2 = 0.28, p(slope) < 0.0001) and Little Merran Creek (slope = 0.025, R2 = 0.14, p(slope) < 0.0001) over the same period. Consistent with the findings from 2012-2013, this variability indicates that although sunlight is obviously essential for photosynthesis, it is generally not the major factor limiting primary production in these streams. If that were the case, a much stronger (and less scattered) relationship would be expected between Daily PAR and GPP. This finding suggests another factor or factors must be contributing to the measured GPP rates. One of these important factors is the biomass of organisms capable of photosynthesis. These are typically divided into three major groups: the macrophytes, biofilms and floating algae (phytoplankton). Biofilms are likely to be the dominant primary producer in most reaches of these streams, especially in the shallow, marginal zones of the stream where sunlight easily penetrates through the water. In the water column, photosynthesis can only occur when there is sufficient light. When the water is turbid (‘cloudy’ from suspended colloidal particles), this limits how far into the water light can penetrate. The point at which photosynthesis is no longer biologically viable is known as the ‘euphotic depth’, Zeu. This parameter can also be estimated from simple relationships derived from empirical data. As a relationship is not available for these rivers, we used information for the Darling River, a larger, turbid, lowland river in northern NSW (Oliver et al. 1999). This empirical equation is Zeu = 4.6 / (0.04 x Turbidity + 0.73). Table 23 lists the euphotic depth for each river based on the regular, fortnightly measurements of turbidity taken during this project. For each trip turbidity was measured at 5 locations. Data has been pooled for each site. The euphotic depths indicate that for all reaches deeper than Zeu, insufficient light will reach the sediment surface to allow macrophytes and biofilms to grow. Hence biofilms will be restricted in range to shallower regions on the edges of the stream. This will decrease the overall primary productivity of the rivers. For example, in Little Merran Creek, the stream with the highest median turbidity (83 NYU), all areas with water depth greater than 1.14 m lack light at the sediment surface, hence preventing benthic plant growth, even assuming all other factors (substrate, nutrient concentrations, lack of scour) are conducive to such growth. The lack of biofilms and plants in these still relatively shallow reaches will result in depressed primary production compared to the same stream environment with less turbidity. In turn, this will result in less organic matter in the surficial sediments, thereby diminishing ecosystem respiration rates.

Figure 56. Linear relationships between daily light (as PAR) and gross primary production for the Wakool River and Yallakool Creek. Collated data from September 2013 through April 2014. P-values for the slope term: Wakool < 0.0001; Yallakool < 0.0001

Table 23. Summary of stream turbidity (NTU) plus the median euphotic depth, Zeu, for the four study rivers, August 2013 - April 2014.

|  | Colligen | Little Merran | Wakool | Yallakool |
| --- | --- | --- | --- | --- |
| n | 80 | 85 | 85 | 77 |
| Median | 57 | 83 | 51 | 58 |
| Min | 7 | 41 | 15 | 7 |
| Max | 109 | 151 | 109 | 131 |
| Zeu (m) | 1.53 | 1.14 | 1.66 | 1.51 |

It was expected that increasing water temperatures associated with seasonal change would result in faster physiological rates in organisms and hence greater rates of GPP and ER. The relationship between mean daily water temperature and the rate of daily gross primary productivity (Figure 57) confirms this hypothesis. The figure shows data for Colligen Creek, but the other three streams were similar. Also not shown, but displaying similar positive relationships with mean daily water temperature, are the rates of ecosystem respiration in the four rivers. The slopes and coefficients of variation (R2) for all eight plots are given in Table 24. All four coefficients of variation (R2 = 0.43 - 0.71) indicated a moderately strong, positive relationship between ecosystem respiration and mean daily water temperature, but again suggesting that other factors are also important in determining these rates.

Figure 57. Linear relationship between daily mean water temperature and gross primary production for Colligen Creek. Collated data from September 2013 to April 2014. P-value for slope < 0.0001.

Table 24. Temperature Dependence of GPP and ER Rates for the four study streams.

|  |  | GPP vs Temperature |  | ER vs Temperature |  |
| --- | --- | --- | --- | --- | --- |
| Stream | n | Slope | R2 | Slope | R2 |
| Colligen | 221 | 0.113 | 0.57 | 0.213 | 0.43 |
| Little Merran | 251 | 0.069 | 0.30 | 0.233 | 0.46 |
| Wakool | 220 | 0.161 | 0.51 | 0.316 | 0.46 |
| Yallakool | 194 | 0.092 | 0.52 | 0.427 | 0.71 |

Note: All slopes were significantly different from 0 (p-value < 0.0001)

The temperature dependence of the GPP and ER rates exemplified in Figure 57 and determined by linear regression (Table 24) indicates a potential problem during hot, dry summers (although not realised in this past summer). Higher water temperatures automatically mean lower dissolved oxygen solubilities. By itself this is rarely problematic as solubility is 6.4 mg O2/L at a water temperature of 40oC, well above the ANZECC trigger value of 4.0 mg O2/L. However, as temperature increases, the net drawdown of O2 also increases as respiration rates increase more rapidly with temperature than GPP (Table 24). This net drawdown exacerbates the lower O2 solubility and increases the likelihood of suboxic, even anoxic conditions (this occurrence was the trigger for the water quality watering action conducted for the Niemur River with flows via Collagen Creek in February 2014). Fortunately, physical reaeration by diffusion of O2 from the atmosphere will partially counteract this potential for developing very low dissolved oxygen concentrations. It is highly pertinent to note that reaeration across the air-water interface is significantly higher from a moving water column than from a still one, especially if the current increases water turbulence. Thus maintenance of even slow flow rates, for example by periodic watering actions, is far preferable to standing water in the streams as a means of addressing potential anoxia during hot, dry spells. Note that this is a different scenario to blackwater events, where rapid oxygen drawdown is brought about by direct contact of water with high levels of organic carbon on the floodplain during the warmest times of the year.

Another key factor controlling the rate of photosynthesis is the concentration of nutrients in the water. As was also found previously in 2011-2013, the bioavailable nutrient concentrations in the water column of each of the streams in this study were very low (see section 6.6). In most cases, the nitrate (NOx) concentration was below the detection limit of 1 μg/L (to enable statistical analyses, those samples with concentrations less than this detection limit were assigned a nominal concentration of half that amount i.e. 0.5 μg/L). These very low nutrient concentrations are the major constraint on the overall primary productivity of the rivers. This nutrient limitation means that large algal blooms are unlikely in these streams. As seen regularly with algal blooms in rivers such as those in the Darling River system in northern NSW, high turbidity and hence low light penetration into the water is insufficient to prevent large blooms if nutrient concentrations are high. The high turbidity simply favours species such as the toxic cyanobacteria (blue-green algae) Anabaena, which can alter its position in the water column to move to the surface and obtain sufficient light to grow prolifically. The major difference between the rivers in the Edward-Wakool system and those in the Darling and other regions, is the very low nutrient concentrations which greatly limit algal proliferation. Hence management focus in the study region should continue to include nutrient minimization strategies.

When considering possible nutrient limitation, the form of the nutrient is critically important. Measurements of total concentrations of nitrogen and phosphorus include all N and P in that sample. However, only the bioavailable fractions, nominally FRP for phosphorus and the sum of ammonia and NOx for nitrogen, are in the form readily taken up by primary producers, whereas other N and P species can be relatively recalcitrant (not suitable for assimilation into plant biomass). Table 25 shows several salient features of the nutrient concentrations in the study streams over the period August 2013 to April 2014: i) the median concentrations of the bioavailable forms are extremely low by both national and international standards for lowland rivers; ii) As noted in the Water Quality section of this report (section 6.6), the Total P and Total N concentrations are frequently above the ANZECC Water Quality Guidelines; and iii) typically only 5-6% of the phosphorus in the water column is bioavailable and just 1% (or less) of the nitrogen is bioavailable. This highlights a general finding in turbid waters, that the use of total nutrient concentrations, rather than bioavailable concentrations, can provide extremely misleading information about the potential of a water body to suffer adverse effects from ostensibly high nutrient levels.

Table 25. Median Bioavailable and Total Nutrient Concentrations and the fraction of Bioavailable P & N in the four study streams from August 2013 through April 2014.

|  | Nutrients (μg/L of N or P) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| River | n | Ammonia | FRP | NOx | Total P | Total N | % Bioavailable P | % Bioavailable N |  |
| Colligen | 17* | 2.0 | 3.0 | < 1 | 50 | 480 | 6.0 | 0.6 |  |
| Little Merran | 17* | 2.5 | 3.0 | 2.0 | 55 | 460 | 6.0 | 1.1 |  |
| Wakool | 17* | 2.0 | 3.0 | 1.0 | 50 | 485 | 5.0 | 0.6 |  |
| Yallakool | 17* | 2.0 | 3.0 | < 1 | 50 | 470 | 5.0 | 0.7 |  |

* Sampled on 17 separate occasions with typically 4 replicates in each sample.

The narrow range of ecosystem respiration rates identified in Table 24 can be largely explained by a combination of two factors: i) the remarkably constant concentrations of TOC (and DOC) in the water column (shown in Table 26) resulting largely from the constraint of elevated flow levels to within the main channels; and ii) the constancy of the gross primary productivity rates, which would yield an approximately constant supply of leachate material for microbial respiration.

Table 26. Summary statistics for Total and Dissolved Organic Concentrations in the four study streams from August 2013 through April 2014.

The Effect of Watering Actions on the Metabolic Rates in the four focus rivers

The influence of watering actions on the rates of GPP and ER has been determined by examining these rates before and during each of these events. As noted earlier, the extended duration of the combined Cod and Perch watering actions in Yallakool Creek, and the late summer water quality action in Colligen Creek, coupled with sonde misbehaviour meant that there was insufficient ‘after’ data to make any assessment of ‘post action’ impacts. The rationale behind this ‘before’ and ‘during’ approach is to isolate the effect of the watering action from weather, seasonal and water temperature effects, all of which are assumed to act equally on both the ‘Impact’ rivers (Yallakool or Colligen Creeks) and the ‘Control rivers’ (Wakool River or Little Merran Creek). The results of all comparisons are presented in Table 27. For illustrative purposes, two sets of results from this Table are presented as Box Plots in Figures 58 and 59.

Previous studies over the period 2011-13 have provided different perspectives on the effects of watering actions on rates of ecosystem respiration and gross primary productivity. During 2011-12, there was an indication of enhanced rates of GPP and ER after watering actions, but this behaviour was not strongly evident during 2012-2013, in contrast to the initial hypotheses.

Although the statistical analysis summarized in Table 27 indicate that there were statistically significant differences in metabolic rates between the control and impact streams induced by the watering actions on several occasions, the direction of the change was variable for GPP and in all cases the magnitude of these differences was very small. From this table, all statistically significant changes were < 1 mg O2/L/Day. This finding is consistent with the fourth hypothesis that predicted changes would be < 5 mg O2/L/Day unless there was significant inundation of backwaters and the floodplain.

Of the four comparisons of ecosystem respiration in Table 29, both Yallakool Creek results showed an enhanced rate of ER in this stream compared to the controls during the extended watering action. ER appeared to be enhanced in Colligen Creek as well, but these changes were either exceptionally small or highly variable, resulting in a lack of statistical significance (using the conventional p < 0.05 as the significance criterion). This preponderance of ER enhancement is largely in line with the hypothesis that a fresh would increase ER by introducing new organic matter. The very small magnitude of the enhancements is entirely consistent with the DOC (and TOC) measurements and the finding that the organic carbon that was measured was relatively non-labile (hence more difficult, and slower, to respire).

The results were less consistent for GPP and seemed to depend on which stream was used as the control. With the Wakool River as the control, both Yallakool Creek and Colligen Creek exhibited a suppression of GPP during the watering actions, whereas with Little Merran Creek as the control, there was an enhancement on GPP. The key point however, is that these differences, although statistically significant, were all extremely small (< 1 mg O2/L/Day). A difference of e.g. 3-5 mg O2/L/Day would indicate an ecologically important change in rates.

The most important finding of this statistical analysis is that any suppression or enhancement of GPP or ER was very small; typically limited to within the range ± 1 mg O2/L/Day (Figures 58 and 59). It was noted earlier that the metabolic rates in these streams are relatively low (but ‘normal’) compared to other streams around the world and it is apparent that these freshes are sometimes causing small fluctuations in these ‘low’ rates. Freshes that result in water moving onto the floodplain and then returning to the stream channel may have a much larger impact on gross primary production and ecosystem respiration, but this was unable to be examined during the 2013-14 period as there were no bankfull or overbank flows due to the need for water managers to limit the risk of potential third party impacts.

Figure 58. Box plots showing the effect of the combined watering action on metabolism in Yallakool Creek. A negative value on the Y-axis indicates that the rate in the control stream (Wakool River) was higher than in Yallakool Creek. The boxes represent the data range 25th to 75th percentile, with the ‘middle’ line in the box being the median. The “whiskers” indicate 10th and 90th percentiles in the data. Outliers are shown as circles.

Figure 59. Box plots showing the effect of the watering action on metabolism in Colligen Creek. A negative value on the Y-axis indicates that the rate in the control stream (Little Merran Creek) was higher than in Colligen Creek. The boxes represent the data range 25th to 75th percentile, with the ‘middle’ line in the box being the median. The “whiskers” indicate 10th and 90th percentiles in the data. Outliers are shown as circles.

It is clear that GPP is strongly constrained by bioavailable nutrient concentrations and that the watering actions were not of sufficient magnitude to entrain higher nutrient concentrations (e.g. from rewetting the floodplain). Neither of the streams receiving environmental water (Yallakool Creek nor Colligen Creek) contained higher nutrient concentrations at any stage (before, during and after the watering action) than the other streams being monitored. One caveat to this statement is that water quality sampling, including nutrient and organic carbon measurements, was performed fortnightly (which provides an excellent synopsis of water quality), but any significant increase in concentrations after inundation of previously disconnected waters would almost certainly occur over a period of only a few days (the ‘first flush’ phenomenon).

Figure 60 depicts the difference in GPP and ER between Yallakool Creek, which experienced the watering actions, commencing on October 17, and Wakool River, representing the control site. The first hypothesis suggests that there will be an immediate decline in GPP following the additional water (marked by the edge of the blue shading in the figure). GPP did decline (the purple circles falling below ‘0’ on the Y axis) but not for at least 10 days after the onset of the watering action. From about three weeks into the action, GPP was suppressed compared to the control for at least two months. This finding contradicts hypothesis two, which suggested that there would be a subsequent increase in GPP following the initial decline, brought about by higher nutrient concentrations.

The third hypothesis that there would be a rapid decline in ER followed by an increase due to the influx of labile organic carbon is partially supported by the data shown in Figure 60. The brown diamonds representing the ER difference between Yallakool Creek and the Wakool control did rapidly fall below 0 (over a few days) as the watering action commenced; following this initial decline, ER rates were on average above those in the control stream for the duration of the period shown. However, these small changes were not associated with any dramatic increase in lability of the organic carbon in the stream. In support of these hypotheses, but with too few data points to test statistically with any confidence, it is interesting to note that both the GPP and ER differences dropped below zero and remained there for a week following the additional water in Yallakool Creek associated with the perch pulse flow starting on the 9th November.

Figure 60. The differences in metabolic rates between Yallakool Creek experiencing the watering actions for cod and perch (over the period designated by the light blue shading) and Wakool River as the control site.

The rationale behind the first hypothesis is that a new source of water, which includes through a watering action, will result in elevated in-stream turbidity. Turbidity data from the routine monitoring program are plotted for the four streams used in metabolism measurements in Figure 61. Contrary to the hypothesis, the advent of additional flow in Yallakool Creek and then in Colligen Creek did not result in any large increase in turbidity, especially when compared to the behaviour in the control streams.

Figure 61. Mean (±SE) Turbidity values (NTU) in Yallakool Creek, Colligen Creek, Wakool River and Little Merran Creek between August 2013 and April 2014. Blue and green bars represent the start and finish dates of environmental watering actions.

These outcomes, including the lack of a major response in metabolic parameters to watering actions, could be seen as largely positive, as these watering actions did not increase the risk of either algal blooms or anoxic events. However, given these very small, and probably ecologically insignificant, changes in metabolic rates, it is not yet possible to determine whether the current rates of organic carbon (i.e. food) production are sufficient to help re-establish the fish populations to those prior to the devastating black water event in late 2010. As noted in the 2012-2013 report (Watts et al. 2013b) and further in 2013-14, there has been some recovery in the apex (fish) predators, but whether this recovery is constrained by food resources is not yet clear. In addition, the effects of an overbank flow on subsequent organic matter production remains unknown. It is expected that such an event would yield much higher nutrient and organic carbon concentrations to stimulate metabolism, but this needs to be assessed when such an event occurs. Consequently, it is strongly recommended that there is continued monitoring of discharge – metabolism – fish community metrics (e.g. recruitment, movement, long term community structure) across a diverse range of base flow, natural flows and watering actions over a wide spread of seasons.

Table 27. Statistical Summary of Effects of Watering Actions on Stream Metabolism. Green shading = rate enhancement, pink = rate suppression, compared to control.

| Impact River | Control River | Watering Action | Data Periods in Relation to Action |  | Effect on Metabolic Rates* |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Before | During | GPP | ER |
| Yallakool | Wakool | 17/10/2013 to 4/2/2014 | 31/8/2013 to 16/10/2013 | 17/10/2013 to 4/2/2014 | M-W, suppression of Yallakool GPP, p < 0.001. ‘before’ median = 0.10, n = 40, ‘during’ median = -0.30, n = 63. | M-W, enhancement of Yallakool ER, p =0.010. ‘before’ median = -0.14, n = 40, ‘during’ median = 0.28, n = 63. |
|  | Little Merran | 17/10/2013 to 4/2/2014 | 31/8/2013 to 16/10/2013 | 17/10/2013 to 4/2/2014 | M-W, enhancement of Yallakool GPP, p < 0.001. ‘before’ median = -0.23, n = 39, ‘during’ median = 0.04, n = 48. | M-W, enhancement of Yallakool ER, p =0.005. ‘before’ median = -0.05, n = 39, ‘during’ median = 0.58, n = 48. |
| Colligen | Wakool | 7/2/2014 to 12/3/2014 | 31/8/2013 to 6/2/2014 | 7/2/2014 to 12/3/2014 | M-W, suppression of Colligen GPP, p = 0.001. ‘before’ median = 0.27, n = 94, ‘during’ median = -0.27, n = 12. | M-W, no sig. effect on Colligen ER, p =0.932. ‘before’ median = -0.02, n = 94, ‘during’ median = 0.02, n = 12. |
|  | Little Merran | 7/2/2014 to 12/3/2014 | 31/8/2013 to 6/2/2014 | 7/2/2014 to 12/3/2014 | M-W, enhancement of Colligen GPP, p =0.006. ‘before’ median = 0.26, n = 95, ‘during’ median = 0.74, n = 15. | M-W, no sig. effect on Colligen ER, p =0.050. ‘before’ median = 0.32, n = 95, ‘during’ median = 1.23, n = 15. |

* ‘T’ = Student's two-tailed t-test; performed if the Shapiro-Wilk normality test was passed (at p = 0.05). ‘M-W’ = Mann-Whitney, Non-parametric Rank Sum Test; performed when normality requirements for t-test not met.

6.9	Shrimp

Photo: Freshwater shrimp Paratya australiensis

Key findings

- Shrimp larvae were sampled from August 2013 to March 2014 to determine if the environmental watering actions in 2013-2014 influenced the relative abundance of larval and juvenile shrimp in the Edward-Wakool river system.

- The spawning of shrimp occurred independently of the environmental watering actions.

- The rivers that received Commonwealth environmental water had fewer shrimp than the rivers that did not receive environmental water. In particular, the abundance of juvenile P. australiensis was significantly lower in Yallakool Creek compared to the control rivers during the Yallkool perch flow event. Thus, it appears that the abundance of Paratya australiensis larvae may have been negatively affected by environmental watering during their spawning season.

- Based on hydraulic modelling undertaken for a range of flow discharges, it is likely that the magnitude and timing of environmental flows watering during the 2013-2014 watering actions decreased the available slackwater habitat in Yallakool Creek that is necessary for shrimp recruitment.

- There is a need to increase the understanding of interaction between instream flows and instream habitat (see flow recommendations)

Background

Shrimp often occur in very large numbers in the Edward-Wakool system and are likely to be an important component of the food web. These shrimp are opportunistic omnivores feeding on algae, detritus and small invertebrates (Burns and Walker, 2000) and their foraging activities can play a key role in processing of organic material and nutrient cycling (Covich et al. 1999; Crowl et al. 2001, March et al. 2001). As well as being important to ecosystem function, shrimp are an important food source for native fish particularly the larger species such as Golden Perch Macquaria ambigua, Murray cod Maccullochella peelii peelii and trout cod Maccullochella macquariensis (Ebner 2006; Baumgartner 2007). One of the objectives of the watering Option 1 in the Edward-Wakool system is to support the condition of native fish. Given that shrimp are an important food source for native fish, an understanding of the impact of environmental watering actions on shrimp recruitment and production in the Edward-Wakool system would be invaluable.

In the Murray-Darling Basin there are two atyid species of shrimp, Paratya australiensis and Caridina mccullochi, and one palaemonid species, Macrobrachium australiense. Previous studies have shown that the distribution and abundance of C. mccullochi and M. australiense may be affected by altered flow regimes, whereas P. australiensis seems to be more tolerant (Richardson et al. 2004). This relationship is thought to be linked to the availability of suitable slackwater habitats during their spawning period (Richardson et al. 2004). Slackwaters act as an important nursery habitat for larval shrimp by providing refuges from current and energetic advantages during a key stage of their development (Humphries et al. 2006; Price and Humphries 2010). Flow regime has the ability to alter the size, availability and permanence of slackwaters (Bowen et al. 2003; Price et al. 2013), thus environmental watering has the potential to affect the recruitment success of shrimp.

The aim of this component of the study was to determine if the environmental watering actions in 2013-2014 influenced the relative abundance of larval and juvenile shrimp in the Edward-Wakool river system.

Hypothesis

The abundance of shrimp larvae and juveniles in the rivers that received environmental water will differ from the rivers that did not receive environmental water. The effects of the environmental watering on shrimp abundance will be influenced by how the environmental freshes influence the availability of slackwaters. Shrimp abundance will be higher in rivers that have more permanent and larger areas of slackwater.

Methods

Shrimp larvae and juveniles were sampled in addition to larval and juvenile fish using quatrefoil perspex traps containing bioluminescent light sticks (see larval fish methods in section 6.3). Three traps were set at five sites within each of the four rivers (15 traps in total per river). Traps were deployed at random along the littoral edge at each site at dusk, and retrieved the following morning (7:00-9:00am). All shrimp collected from the light traps were preserved in 90% ethanol and returned to the laboratory for processing.

Data analysis

The distribution of the shrimp abundance data was examined and a generalised linear model with negative binomial distribution and a log-link function was used to determine whether abundances of larvae, juveniles of each species differed between rivers and among sampling occasions. Pairwise tests based on estimated marginal means were conducted to determine where significant differences occurred. Generalised linear models were performed using SPSS Version 17 (SPSS Inc., Chicago, IL, U.S.A.).

An asymmetrical BACI (before-after, control-impact) (Underwood 1991) statistical design was used to test the effect of specific 2013-2014 environmental water actions on larval and juvenile shrimp abundance in the Edward-Wakool system. Differences in mean number of shrimp between control/impact rivers and before/during/after environmental freshes were evaluated statistically for each watering action using two-way mixed effects analysis of variance (ANOVA). Because there were multiple sampling times used to represent before, during and after environmental flows, and multiple rivers used as ‘Control’ and sometimes ‘Impact’ rivers, sampling trip (random effect) was nested within Period (fixed effect, three levels: before, during and after), and river (random effect) was nested in Treatment (fixed effect, two levels: control rivers, impact rivers). Impact rivers received environmental freshes, while Control rivers were those that did not receive environmental water. For this analysis particular interest is in the Period x Treatment interaction term, which indicates a significant effect of the environmental watering action.

The null hypothesis for all watering actions was that mean larval and juvenile abundance of shrimp in the rivers which received environmental water would not be significantly different to the control rivers. All BACI analyses were carried out using the freeware R and the R package NLME (R Development Core Team 2013).

Results and discussion

Abundance and seasonal timing in appearance of larval and juvenile shrimp

A total of 49,343 larval and juvenile shrimp were collected over the 2013-2014 monitoring period. Of these, 21,037 were P. australiensis and 28,306 were M. australiense (Table 28). The duration and timing of the spawning period was similar across the four rivers (Figures 62 and 63). P. australiensis larvae were found earliest in the spawning season, and were collected as larvae from late September to March, except in the Yallakool Creek where larvae did not appear until early October (Figure 62). Peaks in abundance of P. australiensis larvae occurred at different times in each River. Numbers peaked in October in both Colligen Creek and Little Merran Creek, whereas peak abundance in Yallakool Creek occurred in March in the Wakool River occurred in January (Figure 62). Numbers of M. australiense larvae peaked late January and early February in all four rivers (Figure 62). Peaks in abundance of P. australiensis juveniles occurred in December in Little Merran Creek, January in Yallakool Creek and in February in both Colligen Creek and Wakool River. Peaks in abundance of M. australiense juveniles occurred in February in all four Rivers (Figure 63).

Table 28. Total abundance of larval and juvenile Paratya australiensis and Macrobrachium australiense collected with light traps from the four focus rivers in the Edward-Wakool system during 2013-2014.

| Species | Colligen | Yallakool | Wakool | Little Merran | Total |
| --- | --- | --- | --- | --- | --- |
| P. australiensis | 3946 | 2245 | 5387 | 9459 | 21037 |
| M. australiense | 4907 | 6273 | 7734 | 9392 | 28306 |
| Total | 8853 | 8518 | 13121 | 18851 |  |
| % | 17.9 | 17.3 | 26.6 | 38.2 |  |

Environmental watering event-based analysis

The Cod maintenance and recession flows in the Yallakool creek coincided with the beginning of the spawning season for P. australiensis and M. australiense (Figure 62). The spawning of P. australiensis occurred independently of the environmental watering (d.f=1, 114, F-test=2.27, p>0.05) with the most significant results due to time period (before, during or after) (Table 29) which suggests that the changes in abundance are simply due to seasonal factors that affect all rivers. The spawning of M. australiense also occurred independently of environmental watering (d.f=1, 114, F-test=1.13, p>0.05) (Table 29, Figure 64). The Perch pulse flow in Yallakool creek had no significant impact on larval shrimp numbers however the abundance of juvenile P. australiensis was significantly lower in Yallakool compared to the control rivers during this flow event (d.f=1, 44, F-test=1.67, p<0.05) (Figure 65). The Colligen-Neimur continuation flow also had no significant impact on shrimp numbers (Figure 66) with no significant changes in abundance of P. australiensis larvae or juveniles. Changes in abundance of M. australiense can be attributed to seasonal changes (Table 29).

Figure 62. Temporal tends in mean abundance (±1SE) of larval Paratya australiensis and Macrobrachium australiense in the Edward-Wakool system during 2013-2014. Horizontal bars indicate duration of environmental watering actions in the treatment rivers, Colligen Creek and Yallakool Creek.

Figure 63. Temporal trends in mean abundance (±1SE) of juvenile Paratya australiensis and Macrobrachium australiense in the Edward-Wakool system during 2013-2014. Horizontal bars indicate duration of environmental watering actions in the treatment rivers, Colligen Creek and Yallakool Creek.

Table 29. Statistical results for 2-way mixed-effects Analaysis of Variance (ANOVA) looking at the effect of Commonwealth environmental watering actions on Larval and juvenile shrimp abundance. A significant interaction between the two fixed factors: Period (Before (B), During (D), After(A)) and CI (control rivers, impact rivers) indicates that the mean abundance of shrimp within Impact Rivers was significantly different to changes that occurred over the same period of time within the Control Rivers.

|  | Species | Main effect | d.f | F-test | p-value | Significant effect of watering action? |
| --- | --- | --- | --- | --- | --- | --- |
| Cod maintenance and recession flows: Yallakool creek |  |  |  |  |  |  |
|  | P. australiensis (larvae) | Period (B-D-A) | 2,114 | 3.675 | 0.028 |  |
|  |  | CI (C-I) | 1,114 | 0.546 | 0.461 |  |
|  |  | Period*CI | 1,114 | 2.269 | 0.109 |  |
|  | P. australiensis (juveniles) | Period (B-D-A) | 2,114 | 1.634 | 0.199 |  |
|  |  | CI (C-I) | 1,114 | 0.235 | 0.628 |  |
|  |  | Period*CI | 1,114 | 1.716 | 0.184 |  |
|  | M. australiense (larvae) | Period (B-D-A) | 2,114 | 33.842 | <0.001 |  |
|  |  | CI (C-I) | 1,114 | 0.829 | 0.364 |  |
|  |  | Period*CI | 1,114 | 1.133 | 0.3255 |  |
|  | M. australiense (juveniles) | n/a |  |  |  |  |
| Perch pulse flow: Yallakool creek |  |  |  |  |  |  |
|  | P. australiensis (larvae) | Period (B-A) | 1,44 | 2.022 | 0.162 |  |
|  |  | CI (C-I) | 1,44 | 0.980 | 0.327 |  |
|  |  | Period*CI | 1,44 | 0.358 | 0.552 |  |
|  | P. australiensis (juveniles) | Period (B-A) | 1,44 | 3.832 | 0.056 |  |
|  |  | CI (C-I) | 1,44 | 1.228 | 0.273 |  |
|  |  | Period*CI | 1,44 | 6.768 | 0.013 | Significant effect (-) |
|  | M. australiense (larvae) | Period (B-A) | 1,44 | 1.000 | 0.322 |  |
|  |  | CI (C-I) | 1,44 | 0.339 | 0.563 |  |
|  |  | Period*CI | 1,44 | 0.461 | 0.500 |  |
|  | M. australiense (juveniles) | n/a |  |  |  |  |
| Colligen-Neimur continuation flow |  |  |  |  |  |  |
|  | P. australiensis (larvae) | Period (B-D-A) | 2,66 | 0.032 | 0.968 |  |
|  |  | CI (C-I) | 1,66 | 0.431 | 0.513 |  |
|  |  | Period*CI | 1,66 | 0.237 | 0.789 |  |
|  | P. australiensis (juveniles) | Period (B-D-A) | 2,66 | 0.136 | 0.872 |  |
|  |  | CI (C-I) | 1,66 | 0.173 | 0.678 |  |
|  |  | Period*CI | 1,66 | 0.794 | 0.456 |  |
|  | M. australiense (larvae) | Period (B-D-A) | 2,66 | 3.365 | 0.040 |  |
|  |  | CI (C-I) | 1,66 | 0.633 | 0.429 |  |
|  |  | Period*CI | 1,66 | 1.508 | 0.228 |  |
|  | M. australiense (juveniles) | Period (B-D-A) | 2,66 | 3.533 | 0.035 |  |
|  |  | CI (C-I) | 1,66 | 0.619 | 0.434 |  |
|  |  | Period*CI | 1,66 | 1.034 | 0.361 |  |

Figure 64. Mean abundance (±1SE) of shrimp present in the Edward-Wakool system before, during and after the Cod maintenance and recession flows in Yallakool creek between October 2013 and January 2014. The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls. There were no significant interactions (see Table 29).

Figure 65. Mean abundance (±1SE) of shrimp present in the Edward-Wakool system before and after the Perch pulse flow in the Yallakool creek in Yallakool creek in November 2013. The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls. The abundance of juvenile P. australiensis was significantly lower after the flow compared to the control rivers (see Table 29).

Figure 66. Mean abundance (±1SE) of shrimp present in the Edward-Wakool system before, during and after the Colligen-Neimur continuation flow between January and March 2014. The Wakool River and Little Merran Creek did not receive environmental water and were used as controls. There were no significant interactions (see Table 29).

Difference in total larval and juvenile production across rivers

The abundance of larvae sampled over the entire sampling period was not significantly different across the four rivers (d.f=3, Wald chi-square=2.96, p>0.05). Mean (±SE) numbers of larvae sampled from Colligen Creek, Yallakool Creek, Wakool River and Little Merran Creek were 126 (±26), 123 (±45), 173 (±41) and 275 (±55), respectively. The abundance of P. australiensis larvae was significantly lower in Yallakool Creek compared to the other three rivers (d.f=3, Wald chi-square=45.5, p<0.001), in addition the abundance P. australiensis larvae in the Wakool River was significantly lower than in Little Merran Creek (Figure 67). The abundance of M. australiense larvae was not significantly different between rivers (d.f=3, Wald chi-square=1.169, p>0.05) (Figure 67).

The abundance of juvenile shrimp was significantly greater in Wakool River than in Colligen Creek (d.f=3, Wald chi-square=8.0, p<0.05). Mean (±SE) numbers of Juveniles sampled from Colligen Creek, Yallakool Creek, Wakool River and Little Merran Creek were 25 (±4), 20 (±4), 55 (±22) and 53 (±10), respectively. P. australiensis juveniles were in significantly higher numbers in the Wakool River (d.f=3, Wald chi-square=17.53, p<0.01), there was no significant difference between the other rivers (Figure 67). The abundance of M. australiense juveniles was significantly higher in Little Merran Creek compared the other three rivers (d.f=3, Wald chi-square=69.5, p<0.001) (Figure 67).

Figure 67. Mean total abundance of Shrimp larvae and juveniles in Colligen Creek, Yallakool Creek, Wakool River and Little Merran Creek during the 2013-2014 sampling season.

Conclusion

The aim of this component of the study was to determine if the environmental watering actions in 2013-2014 influenced the relative abundance of larval and juvenile shrimp in the Edward-Wakool system. Similar to the results in 2012-13, the rivers that received environmental water had fewer larval and juvenile shrimp overall compared to the two control rivers. In particular, there were fewer P. australiensis in Yallakool Creek compared to the other rivers. It is possible that the sustained flows received in Yallakool River during the shrimp spawning season reduced the overall availability of slackwaters required for P. australiensis spawning (Price and Humphries 2010). In addition, both Yallakool Creek and Colligen Creek had fewer juvenile P. australiensis than Wakool River, and both Colligen Creek and Yallakool Creek had fewer M. australiense juveniles than Little Merran Creek. This is interesting as the variation in flow was much greater in Colligen Creek and Yallakool Creek compared to Wakool River and Little Merran Creek.

It is possible that the recruitment success of both species may be negatively affected by in-channel flows during spring and summer that do not create slackwater habitat, as shrimp recruitment occurs during the normally low flow summer period (Humphries et al. 2006). Sharp increases in flow may result in higher larvae mortality as a result of displacement and catastrophic drift from slackwater habitats (Humphries et al. 2006; Price and Humphries 2010). Indeed, Hancock and Bunn (1997) found that high flow events limited the recruitment of P. australiensis.

Preliminary hydraulic modelling undertaken for these reaches in the Edward-Wakool system shows the environmental flows were likely to have decreased the size and availability of slackwaters relative to base flows (Watts et al. 2013b; Kingsford and Watts 2014). Only at discharges when benches begin to be inundated inundated, do large slackwaters start to develop and result in a general increase in slackwater area. Veitz et al. (2013) also found that in the Broken River there was a general decrease in the area of slackwaters as flow increased and slackwaters became available only at the channel margins. Therefore, it is likely that the magnitude and timing of environmental flows delivered during the 2013-2014 water actions resulted in a decrease in available habitat for shrimp recruitment. Understanding the relationship between flow and instream features such as woody instream habitat, geomorphological features (such as benches, backwaters) and anabranch systems will allow managers to identify critical thresholds and maximise the benefits of environmental water delivery.

Photo: Peron’s Tree frog Litoria peronii

Key findings

- Frogs were not the main focus of the environmental watering in Yallakool Creek. Commonwealth environmental water was also provided to Tuppal Creek in 2012-13 to achieve frog outcomes, but that area was not monitored by this project.

- Monitoring of the calling activity of adult frogs and the abundance of egg masses, tadpoles and metamorphs was undertaken in the focus rivers from September 2013 to March 2014 to evaluate the breeding response of frogs to Commonwealth Environmental watering actions.

- Whilst a lot of frog activity was observed during the September and October 2013 surveys at Yallakool Creek, frog numbers were similar across all focus rivers. However the Peron’s tree frog abundances differed significantly between rivers with higher numbers of calling adults recorded at Yallakool Creek (received environmental water), and Little Merran Creek (control) compared to Wakool River (control), which had the lowest abundances and did not receive environmental water.

- There was no evidence that the 2013–14 environmental watering actions in Yallakool Creek and the Colligen Creek resulted in frog recruitment, as no egg masses, tadpoles or metamorphs were observed during surveys.

- In channel environmental watering actions did not inundate major in-channel geomorphological features (e.g. benches and backwaters), so may not have provided adequate or suitable habitat to support breeding and recruitment of frogs or adequate refuge from predators. There is a need to increase the understanding of interaction between instream flows and instream habitat (see flow recommendations)

6.10	Frogs

Background

Riverine frogs have a range of life history strategies which allow them to utilise and occupy riverine habitats. In south-eastern Australia nine frog species are wholly dependent on flowing water for reproduction, (Gillespie and Hines 1999) whilst five others use streams for breeding only under certain flow conditions (Gillespie and Hines 1999; Heard et al. 2006). Many generalist riverine frog species utilise a range of habitats created during overbank events, such as back-waters and ground-water pools (Bateman et al. 2008; Wassens and Maher 2011). Permanently inundated systems may not necessarily support frog breeding due to their high predator densities; rather they may be used as refuge habitats during dry periods (Wassens and Maher 2011). Seasonally or intermittently inundated wetland systems, such as the Edward Wakool system, may be important for breeding by species with longer development times (Wassens et al. 2010; Wassens and Maher 2011) therefore in-channel environmental watering is most likely to lead to an increase in frog abundance and calling activity if intermittent and seasonal habitats are created adjacent to the main channel. For example Wassens and Maher (2011) found that temporary water bodies created during environmental flows were preferentially utilised for breeding over the more persistent habitat within the stream channel. Monitoring of the calling activity of adult frogs and the abundance of egg masses, tadpoles and metamorphs was undertaken from September 2013 to March 2014 to evaluate the breeding response of frogs to Commonwealth Environmental watering actions.

Hypotheses

- Frog activity (number of individuals calling and observed) will increase in rivers that receive environmental water relative to rivers that do not receive environmental water.

- Frog breeding (by presence of egg masses, tadpoles and metamorphs) will be observed more often in rivers that receive environmental water relative to rivers that do not.

Methods

Tadpoles were surveyed monthly from September 2013 to March 2014 during the day within a 50 m transect at three locations within each focus river using a large D-bottom sweep net. Tadpoles caught in the net were identified to species level if possible and their developmental stage recorded according to Anstis (2002). Once identified, all tadpoles were released at the point of capture.

Three replicate audio and visual surveys were undertaken at each focus river at monthly intervals, to detect distinct calls of resident frog species and record the number of individuals calling. The number of individuals was estimated when calling frog numbers exceeded the ability of the recorder to differentiate between calling individuals. Frogs were surveyed at night along a 200 m long and 5 m wide transect running parallel to the water’s edge for 30 minutes on each survey occasion. A spotlight was used to search along the water’s edge and within terrestrial habitats. Each individual encountered was identified to species level.

To test if frog activity (number of adult frogs) was greater in rivers that received environmental water, a series of one-way Anovas were run on each frog species, with total mean abundance of adult frogs as the response variable, and river as the grouping variable. Adult frog data did not meet the assumptions for parametric testing, so a Kruskal-Wallace non-parametric ANOVA was run instead. Statistics were run in SPSS.

Results and discussion

Six frog species were recorded across the four focus rivers over the seven month survey period (September 2013 to March 2014) (Table 30). The eastern banjo frog, was not spotted visually, rather it was only identified via calling activity (Table 30). Little Merran Creek initially supported the highest frog diversity which included the only record of the eastern banjo frog (Limnodynastes dumerilii) (Table 30). Colligen Creek contained the highest number of adult frogs, followed by Little Merran Creek, Yallakool Creek and the Wakool River (Figure 68). No frogs listed as vulnerable or endangered under the EPBC Act 1999 were observed or heard calling. No egg masses, tadpoles or metamorphs were observed for any of the frog species during the survey period (Table 30).

Frog community composition was similar across the rivers over the survey period, with the Barking marsh frog (L. fletcheri) being the most commonly visually encountered species followed by the plain’s froglet (Crinia parinsignifera, spotted marsh frog (L. tasmaniensis) and Peron’s tree frog (Litoria peronii) respectively (Figure 68). Significant differences in adult Peron’s tree frog numbers across the four focus rivers were detected (Kruskal-Wallace H=13.88, df 3, p>0.03; Table 31), however this was not observed for L. tasmaniensis, L. fletcheri, C. parinsignifera, or C. signifera (Table 31). Peron’s tree frog was most abundant at Little Merran Creek (n=35), Yallakool Creek (n=24) and Colligen Creek (n=19), while only several individuals were collected in the Wakool River (n=3) (Figure 68).

Table 30. Total abundance of frogs observed in the Edward-Wakool River system during the 2013-2014 study period, for each life history stage.

| Common name | Frog species | Adults | Calling | Eggs | Tadpoles | Metamorphs |
| --- | --- | --- | --- | --- | --- | --- |
| Peron’s tree frog | Litoria peronii | 81 | Y | 0 | 0 | 0 |
| spotted marsh frog | Limnodynastes tasmaniensis | 118 | Y | 0 | 0 | 0 |
| barking marsh frog | Limnodynastes fletcheri | 155 | Y | 0 | 0 | 0 |
| eastern banjo frog | Limnodynastes dumerili | 0 | Y | 0 | 0 | 0 |
| plain’s froglet | Crinia parinsignifera | 120 | Y | 0 | 0 | 0 |
| eastern common froglet | Crinia signifera | 10 | Y | 0 | 0 | 0 |

Table 31. Results of non-parametric one-way anovas (Kruskal-Wallis) comparing adult frog abundance across the four focus rivers in the Edward-Wakool river system. p <0.05 (highlighted in bold) indicates that a significance difference was found in adult frog abundance across rivers in the 2013-2014 study period.

| Frog species | Kruskal-Wallace statistic H | d.f | P |
| --- | --- | --- | --- |
| Peron’s tree frog Litoria peronii | 13.88 | 3 | 0.003 |
| spotted marsh frog Limnodynastes tasmaniensis | 3.79 | 3 | 0.285 |
| barking marsh frog Limnodynastes fletcheri | 6.11 | 3 | 0.106 |
| plain’s froglet Crinia parinsignifera | 6.94 | 3 | 0.074 |
| eastern common froglet Crinia signifera | 2.31 | 3 | 0.511 |

Figure 68. Total mean abundance (± 1SE) of adult frogs identified during visual frog surveys within each focus river between September 2013 and March 2014. The black star indicates there was a significant difference in the abundance of L. Peronii across rivers.

Our findings are comparable to frog diversity and numbers observed along the Murray River between Lock 15 and the South Australian border during an environmental watering action in 2005-06 (Val et al 2007), to environmental flow events along the Murray River (Wassens et al. 2011) and to more recent environmental watering actions in the intermittent Darling Anabranch in 2010-12 which detected 292 individuals during a three year survey period (Bogenhuber et al. 2013) and 449 individuals in 2013-14 (Bogenhuber et al. 2014).

Response to environmental watering

It was hypothesised that frog activity (measured as an increase in the number of individuals observed) would increase in the Yallakool Creek in response to the environmental watering action. Whilst a lot of frog activity was observed during the September and October 2013 surveys at Yallakool Creek, frog numbers were similar across all focus rivers. However the Peron’s tree frog abundances differed significantly between rivers (Table 31), with higher numbers of calling adults recorded at Yallakool Creek (received environmental water), Colligen Creek and Little Merran Creek (control) compared to Wakool River (control), which had the lowest abundances and did not receive environmental water (Figure 68; Figure 69).

There were unregulated flows that achieved above estimated half bankfull levels (Watts et al. 2013b) within the Edward Wakool system prior to the commencement of the environmental watering action and may have accounted for the higher abundances of most frogs observed during the September and October 2013 surveys. An increase in activity at this time was also expected as most frogs that were identified are generally actively calling from September and during the warmer months (Wassens 2011; Wassens and Maher 2011).

Figure 69. Mean abundance (±1SE) of the five frogs visually identified in the Edward-Wakool system during 2013-2014. Coloured bars represent when Commonwealth environmental water was delivered to Yallakool Creek (blue bars) and Colligen Creek (green bars). The Wakool River and Little Merran Creek did not receive environmental water.

Although flows during the Yallakool Creek environmental watering action averaged less than half bankfull, the flows endured for several months, which promoted submerged aquatic vegetation (Section 6.5). Inundated vegetation is used by frogs as calling and spawning locations (Wassens et al. 2010) and as habitat for adults and tadpoles (Healey et al. 1997). Inundated vegetation also acts as substrate for biofilm which is an important food source for tadpoles (Gillespie 2002; Mokany 2007).  Notwithstanding, riverbank vegetation cover and aquatic vegetation cover was generally low in each study river, which potentially reduced suitable habitat for frogs. Concurrent frog surveys found no evidence of increase or decrease in frog numbers, or evidence of breeding (via egg masses, tadpoles or metamorphs) at focus rivers that received environmental watering actions, rather there was little difference in frog numbers between focus rivers during the survey period. The availability of inundated vegetation was generally low across all focus rivers, possibly because environmental watering actions occurred only in channel and did not inundate major in-channel geomorphological features such as backwaters and large benches. This can significantly influence the available wetted surface area available for fauna to inhabit (Watts et al. 2013b). As a consequence, a lack of newly wetted vegetated habitats during environmental watering actions may be an important factor influencing the lack of response of frogs to environmental watering.

High predator densities in the main channel from low flows may also inhibit frog breeding activity and could explain why no tadpoles were caught in-channel during the survey period. Exotic predatory fish including gambusia (Gambusia holbrooki), known to predate upon tadpoles (Anstis 2002; Ralph et al. 2011) and the common carp (Cyprinus carpio), also known to impact frog recruitment (Spencer and Wassens 2009), were present in all focus rivers. The occurrence of these predators, combined with small environmental freshes, may have resulted in reduced suitable inundated habitat being available to provide refuge from predators or support breeding and recruitment of frogs (Wassens and Maher 2011).

Summary and synthesis

Ecosystem responses to environmental watering in 2013-14

Objective 1. Increase movement, condition, reproduction and recruitment of native fish

- There is a general trend towards improvement of the native fish community in the Edward-Wakool system, although this improvement is species and location-specific. Some of the changes may be due to fish migration into the system because recruitment of some species is poor. There was an increase in the abundance of small bodied generalist species (primarily Australian smelt and carp gudgeon) in 2014 compared to 2013. There was an increase in the biomass of Murray cod, goldfish and bony herring and decrease in the biomass of common carp and golden perch in 2014 compared to 2013.

- The Sustainable Rivers Audit measure of expectedness and nativeness were calculated for fish community data collected from 2010 to 2014. All zones were in poor condition in 2014 in terms of nativeness, an improvement over the very poor condition from 2013. In 2014 all zones were in poor to moderate condition in terms of SRA recruitment index.

- Some individuals of acoustically tagged Murray cod, golden perch and common carp dispersed from the refuge pool into new habitats during early season unregulated flows. Based on two years of monitoring data, Murray cod demonstrated a consistent preference for movement into the upper Wakool River over Yallakool Creek during delivery of environmental water. The reasons for this are unknown, although it highlights the importance of maintaining habitat in both the Wakool River and Yallakool Creek. Factors such as loading of woody habitat, overhanging cover, depth of pools or physical or hydraulic barriers may have an influence on this preference.

- The majority of tagged golden perch remained in the refuge pool throughout the environmental watering actions. Those individuals that did move, went both upstream and downstream from the refuge pool, with most movements occurring at the peak of flows or on the recession. It is not known whether these movements resulted in spawning, and this can be evaluated in a future assessment of recruitment by the Long term Intervention Monitoring project. However, combined with results from fish community sampling and fish spawning the results suggest that the 2013-14 environmental watering did not trigger spawning in this species.

- All acoustically tagged golden perch and some Murray cod returned to the refuge pool at the completion of the recession flows, indicating that these flows were appropriately managed to enable native species to return to refuge habitat.

- Nine of the 13 fish species known to occur in theEdward-Wakool River system successfully spawned in 2013-14. Spawning patterns of the Edward-Wakool fish community were independent of the environmental watering actions.

- The environmental watering in Yallakool Creek during the Murray cod spawning season did not result in a significantly greater number of larvae in Yallakool Creek compared to rivers that did not receive environmental water. These findings support the results observed in 2012-13 and the body of knowledge that shows that Murray cod spawn at peak times in November-December, regardless of flow conditions.

- Back-calculated spawning dates indicate that golden perch has spawned in all years from 2004–2010. However, the dominant cohort of golden perch and silver perch was spawned in 2009 during periods of low in-flows into the Edward-Wakool system.

- The Yallakool Creek perch flow did not trigger a golden and silver perch spawning response in the monitored reaches, as evidenced by the absence of larvae or eggs. It is possible that these species spawned elsewhere in the system but were undetected by the current monitoring. Future assessment of fish recruitment undertaken as part of the Long Term Intervention Monitoring project may determine if these species spawned in 2013-14.

- Juvenile golden perch, silver perch and Murray cod recruits were not sampled in large enough numbers to detect whether environmental watering actions influenced recruitment of these species. Recruitment of carp gudgeon occurred between August 2013 and March 2014, peaking in November-January, in all rivers regardless of receiving environmental water. In 2013-14 annual recruitment of carp gudgeon was not positively or negatively affected by environmental watering actions in Yallakool Creek and Colligen Creek. This result is different to previous years, where an increase in the abundance of larvae and juveniles was detected in response to environmental watering in Colligen Creek (Watts et al. 2013a; 2013b). Dissimilar recruitment responses to environmental watering actions among years may be related to differences in the peak magnitude of flows or differences in the relationship between discharge and area of inundation in different rivers. This will require an evaluation of spawning responses across multiple years.

Objective 2. Provide end of system flows and increase hydrological connectivity in ephemeral streams

Monitoring of end of system flows and ephemeral streams was not undertaken in this project.

Objective 3. Maintain/improve vegetation condition, including fringing vegetation and emergent/submerged aquatic plants

- There was a significant increase in the percent cover of submerged aquatic vegetation during the Yallkool Creek cod maintenance flow. The environmental watering enabled the submerged aquatic vegetation (in particular Characeae sp) to persist over an extended period of time. This is a different response to that observed in 2012-13 where the recession was rapid and submerged vegetation was rapidly exposed and desiccated.

- There was no change in the percent cover of terrestrial riverbank vegetation in each river before, during and after the environmental watering, suggesting there was no response to environmental watering actions. However, the monitoring concluded in March 2014, a month after the end of the maintenance flow recession, and may not have continued long enough to detect terrestrial vegetation responses to the environmental watering. Longer term responses of riverbank vegetation to environmental watering will be examined as part of the Long Term Intervention Monitoring project in the Edward-Wakool system (Watts et al 2014).

Objective 4. Maintain/improve water quality within the system, particularly dissolved oxygen, salinity and pH

- Commonwealth environmental watering met the objective of maintaining or supporting water quality outcomes as it did not trigger any adverse water quality outcomes or a hypoxic blackwater event. Environmental watering actions did not significantly alter dissolved carbon, total carbon or the organic matter profiles in treatment rivers relative to the control rivers. Bioavailable nutrients were extremely low in these rivers. Some statistically significant differences in nutrient concentrations were found when assessing effects of watering actions, but these differences were extremely small and were not ecologically important.

- Very small increases in metabolic rates that were not ecologically significant resulted from environmental watering actions, most likely because the flows were contained within the stream channel, with little inundation of backwater areas or instream benches. Rates of primary production and ecosystem respiration during 2013-2014 were at the lower end of the normal range found in rivers worldwide.

Objective 5. Support breeding, recruitment and habitat requirements of a range of native animals, in particular frogs.

- The spawning of shrimp occurred independently of the environmental watering actions. The rivers that received Commonwealth environmental water had fewer shrimp than the rivers that did not receive environmental water. In particular, the abundance of juvenile Paratya shrimp during the Yallakool Creek perch flow was significantly lower in Yallakool Creek compared to the control rivers. The abundance of Paratya larvae may have been adversely affected by environmental watering during their spawning season, because hydraulic modelling has shown that the magnitude of the environmental watering actions in Yallakool Creek decreased the availability of slackwater habitat that is necessary for shrimp recruitment.

- Frogs were not the main focus of the environmental watering in Yallakool Creek. (Note, Commonwealth environmental water was also provided to Tuppal Creek in 2012-13 to achieve frog outcomes, but that area was not monitored by this project). Whilst a lot of frog activity was observed during the surveys, the environmental watering actions in Yallakool Creek and Colligen Creek did not result in frog recruitment, as no egg masses, tadpoles or metamorphs were observed during surveys.

- In channel environmental watering actions did not inundate major in-channel geomorphological features (e.g. benches and backwaters), so may not have provided adequate or suitable habitat to support breeding and recruitment of frogs or adequate refuge from predators. There is a need to increase the understanding of interaction between instream flows and instream habitat (see flow recommendations).

Synthesis of responses to Commonwealth environmental watering

The responses to Commonwealth environmental watering observed in 2013-14 were largely consistent with those observed in previous years. In general, Commonwealth environmental water delivered to the Edward-Wakool system has contributed to the maintenance of water quality, provided opportunities for longitudinal connectivity and fish movement (such as the return movement of fish to the Wakool Reserve refuge pool during recession flows), promoted instream aquatic vegetation, and created a small increase in wetted benthic area. Importantly, the long-term benefits of the Commonwealth environmental watering actions during blackwater events in 2010, 2011 and 2012 are still being realised. The environmental watering during these blackwater events mitigated extreme low dissolved oxygen concentrations (Watts et al. 2013) and thus created an area of refuge habitat and avoided critical loss of fish in the upper reaches of the Wakool River and Yallakool Creek. The benefits of those watering actions are evident, with fish populations in upper part of the Edward-Wakool system maintaining higher biomass than the populations in the lower reaches. The long-term recovery of fish populations in this system is still occurring. However, some of the changes in the fish community in the middle and lower sections of the system are possibly due to other factors, such as immigration of fish into the system.

Some of the expected outcomes of Commonwealth environmental watering actions were not observed in the focal rivers, with no detectable response (positive or negative) to Commonwealth environmental watering observed for several indicators. Although fish reproduction is occurring in this system (nine of the 12 species were collected as larvae in 2013-14), the spawning response in these species could not be attributed to Commonwealth environmental watering. So although there is evidence of some recovery in the fish community in areas impacted by the blackwater events in 2010-2012, recruitment has been limited and the recovery of the fish population has been slow, especialy for large bodied long lived species. There have also been only very small increases in river productivity resulting from environmental watering actions. Hydraulic modelling has shown that Commonwealth environmental watering actions have created small increases in wetted benthic area (Watts et al. 2013b), but this has not been sufficient to trigger an increase in gross primary productivity. The delivery of environmental water is constrained by a limited capacity to deliver higher volumes of water in this system without having impacts on third parties. The CEWO has sought to maximize the flows to a level that is acceptable to third parties in the catchment area. Constraints that limit the delivery of environmental watering actions should be examined further and managers collaborate with the community to minimise factors that may limit the benefits of Commonwealth environmental watering actions (see recommendations).

In addition to the positive and and neutral responses associated with Commonwealth environmental watering there was one negative response observed in 2013-14. There was a lower abundance of juvenile paratya shrimp in Yallakool Creek captured during the perch flow and an overall lower abundance of shrimp larvae in Yallakool Creek. This is thought to be due to a reduction in the area of slackwater during watering actions compared to area of available slackwater during base flows (see hydraulic modelling in Watts et al 2013b, Kingsford and Watts 2014). Slackwater habitat is of vital importance for many organisms including larval fish, macroinvertebrates and frogs and there is a need to increase our understanding of the interaction between instream flows and instream habitat to help managers identify critical thresholds and maximise the benefits of environmental water delivery (see flow recommendations).

A summary of the responses to Commonwealth environmental watering is presented in Table 32. The responses were classified as:

- positive, resulting in improved outcomes (dark green)

- positive, resulting in maintenance of outcomes (light green)

- negative response resulting in adverse outcomes (red)

- no detectable response (neither positive or negative) (white)

- response not assessed by this project (grey)

If we revisit the ecological objectives for the Edward-Wakool system outlined in the water use minute (CEWO, 2013b) we can conclude:

- Objective 1 (Increase movement, condition, reproduction and recruitment of native fish) has been partially achieved, but an increase in reproduction and recruitment has not occurred.

- Objective 2 (Provide end of system flows and increase hydrological connectivity in ephemeral streams (this objective was not assessed in this project) was not assessed by this project

- Objective 3 (Maintain/improve vegetation condition, including fringing vegetation and emergent/submerged aquatic plants) was partly achieved, with an increase in submerged plants but not terrestrial riverbank plants.

- Objective 4 (Maintain/improve water quality within the system, particularly dissolved oxygen, salinity and pH) was achieved.

- Objective 5 (Support breeding, recruitment and habitat requirements of a range of native animals, in particular frogs) was not achieved.

Table 32. Summary of ecosystem responses to Commonwealth environmental watering in the Edward-Wakool system in 2013-14. Dark green shading indicates improved outcomes, Light green indicates positive outcome resulting in maintenance, red shading indicates adverse outcome, white shading indicates no detectable response (neither positive or negative). Grey boxes are where no evaluation was undertaken. N/A = not evaluated.

| Indicators | Responses to unregulated flows in Aug and Sep | Short-term responses to Commonwealth environmental water (responses to individual watering events or flows) |  |  |  |  | Annual response (Comparison among rivers across year) | Longer-term responses (across multiple years) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Yallakool cod maintenance flow and recession | Yallakool perch flow |  | Colligen-Niemur continuation flow |  |  |  |
| Fish community | N/A | N/A | N/A |  | N/A |  | N/A | Improvement of the native fish community over time |
| Fish movement | Movement of Murray cod and golden perch out of refuge pool | Preference by Murray cod for Wakool R compared with Yallakool Creek | No change in habitat occupation detected in golden perch |  | N/A |  | Preference by Murray cod for Wakool River compared with Yallakool Creek | N/A |
|  |  | Return movements of golden perch to refuge pool during recession |  |  |  |  |  |  |
| Fish spawning and reproduction | N/A | No response detected | No response detected |  | N/A |  | 9 species spawned but was not related to e-watering. More smelt, gudgeon in Edward R | N/A |
| Fish recruitment | N/A | N/A | N/A |  | N/A |  | No effect of e-watering on recruitment in carp gudgeon | N/A |
| Riverbank and instream veg | N/A | Increase in % cover of submerged aquatic veg. | N/A |  | N/A |  | Significant differences in veg between rivers but not related to e-watering | N/A |
|  |  | No response in riverbank terrestrial vegetation |  |  |  |  |  |  |
| Water quality and chemistry | HIgher DOC, slower decline in Little Merran Creek | Maintain water quality, no advserse response observed | Maintain water quality, no advserse response observed |  | Maintain water quality, no advserse response observed |  | N/A | N/A |
| Organic matter characterisation | Inc in amount and complexity of organic matter | Maintain water quality | Maintain water quality |  | Maintain water quality |  | N/A | N/A |
| Stream metabolism | N/A | Maintain productivity. There were very small changes in gross primary production and ecosystem respiration that are not ecologically important (< 1 mg O2/L/Day) |  |  |  |  | N/A | N/A |
| Shrimp | N/A | No response detected |  | Significantly lower numbers of juvenile Paratya shrimp in Yallakool Creek |  | No response detected | Abundance of P. australiensis larvae lower in Yallakool Creek. | N/A |
| Frogs | N/A | No response detected |  | No response detected |  | No response detected | No breeding response detected | N/A |

Recommendations

Recommendation 1. Use Commonwealth environmental water to manage the recession of unregulated flows and environmental watering actions

The rate of recession under regulated flows in the Edward-Wakool system is likely to be much faster than the rate of change under natural flow conditions. Slowing down the rate of recession can provide ecological benefits by creating conditions that the biota in these systems would be more adapted to. Commonwealth water contributed to a recession flow in 2013-14, and this promoted the growth and longer duration of instream aquatic vegetation in Yallakool Creek compared to 2012-13 when rates of recession were much faster. In 2013-14 all acoustically tagged golden perch and some Murray cod returned to the refuge pool at the completion of the recession flows, indicating that these flows were appropriately managed to enable native species to return to refuge habitat. Commonwealth environmental water should continue to be allocated to manage recession flows following unregulated flow events or at the end of environmental watering actions.

Recommendation 2. Continue to use Commonwealth environmental water to mitigate adverse water quality events

Commonwealth environmental water has been used on several occasions to mitigate the advserse outcomes of poor water quality events. Monitoring results have demonstrated that these actions have been successful in maintaining water quality. Rapid action and coordination of information by the Edward-Wakool e-flows Group and the Water Murray and District Dissolved Oxygen Group are a vital part of this action.

Recommendation 3. Focus timing of Commonwealth environmental watering on late winter/spring

Available hydrological modelling suggests that the flow regime of the Edward-Wakool system has been significantly altered by river regulation, with changes to the timing and volume of flows. Natural flows in the system would have been high in winter/spring and low in summer and autumn. Late winter/spring flows were a key feature in these systems and biota are likely to be adapted to this regime. Monitoring results have shown that early season unregulated flows in 2013-14 enabled all fish species to disperse from the refuge pool into new habitats. Environmental watering in winter/spring also minimises risks of adverse water quality outcomes. In the absence of natural or regulated flows in late winter or early spring, Commonwealth environmental water should be targeted at this time of the year to enhance dispersal opportunities and maximise growth and reproductive opportunities. Winter/spring flows can be delivered to complement other watering actions delivered at other times of the year targeting other ecological outcomes (e.g. fish spawning).

Recommendation 4. Introduce flow variability into environmental watering actions

The Edward-Wakool river ecosystems have evolved in the presence of flow variability, and there is good evidence to suggest that flow variability can lead to healthy and resilient populations. It is recommended that some of the natural levels of variability should be incorporated within managed environmental watering actions. In general, long periods of constant flow and rapid flow recession should be avoided. One approach to achieve this is to use a whole of system approach to manage flows in the Murray system, using upstream triggers to guide variability. Recent flow events could provide a baseline to enable this approach to be tested to determine its benefits. See flow recommendation 7 for a recommended approach to help guide these decisions.

Recommendation 5. Deliver a variety of flows over time to improve understanding of responses to environmental watering

Some results from monitoring and evaluation of environmental watering are difficult to interpret, as it is not always possible to disentangle the responses to flow from responses related to a specific river. For example, Murray cod have demonstrated a consistent preference for the upper Wakool River over Yallakool Creek based on the past two years of monitoring, however it is unclear whether this is due to a general preference for the Wakool River over Yallakool Creek, or is related to delivery of environmental water. Future targeted watering of the upper Wakool River to maintain or maximise nest site inundation should be considered, as this would improve our understanding of responses to flows by disentangling the factors of river and flow. In addition, the upper Wakool River has not been the target of environmental watering actions over the past three years.

Recommendation 6. Increasing understanding of interaction between instream flows and instream habitat

There is a need to identify and quantify the instream geomorphological features in the rivers likely to receive Commonwealth environmental water to help better target environmental watering actions, especially decisions around the magnitude of water delivery. Understanding the relationship between flow and instream features such as large woody instream habitat, geomorphological features (such as benches, backwaters) and anabranch systems will allow managers to identify critical thresholds and maximise the benefits of environmental water delivery.

Recommendation 7. Undertake comprehensive flows assessment for the smaller creeks and rivers of the Edward-Wakool system

There is a need for a scientific expert panel to use a reputable flows method to provide guidelines for delivery of environmental water, considering the breadth of geomorphological and ecological responses. While there has been some modelling undertaken for the Edward River, there is currently a lack of hydrological modelling on unregulated flow regimes of the smaller creeks and rivers in the Edward-Wakool system and this information is required to underpin decisions on environmental watering in this system. The flows assessment would need to consider factors such as natural rate of recession and rise in flows, short term and long term variability in changes to water height, timing and duration of instream pulses, and low flow periods. Based on the flows assessment, it would be possible to consider which aspects of the flow regime are most or least affected by regulation, and consider how these changes are likely to affect ecological features or assets of the Edward-Wakool system (eg vegetation, bench inundation, flow requirement for fish and birds etc). This would guide decisions and operating guidelines for future environmental watering actions. The flows recommendations should not be single species or group focussed, but consider all aspects of the river ecosystem. This information would underpin environmental watering plans and actions and maximise the benefit for the whole ecosystem.

Recommendation 8. Examine constraints that limit the delivery of environmental watering actions. Collaborate with other management agencies and the community to minimise factors that may limit the benefits of Commonwealth environmental watering actions

There were no observable increases in gross primary productivity, food resources or fish spawning in response to the Commonwealth environmental watering in 2013-14. If the proposed flows assessment (see recommendation 7) recommends that higher discharges are required to provide these instream benefits, then a comprehensive examination of flow constraints and a concurrent community consulation process will required to examine constraints to the delivery of larger in-channel environmental watering events. In the meantime, it is strongly recommended that there is continued monitoring of discharge, metabolism, food resources, habitat and fish community metrics (e.g. recruitment, movement, long term community structure) across a diverse range of base flow, natural flows and watering actions over a wide spread of seasons to better understand the conditions that are required to illicit ecosystem responses to flows. There are other factors (e.g. small instream barriers) that may limit how the ecosystem responds to Commonwealth environmental watering. The CEWO should actively work with other agencies and the community to reduce the impacts of these other factors to produce better ecosystem responses to environmental watering.

Acknowledgements

We respectfully acknowledge the Traditional Owners, their Elders past and present, their Nations of the Murray–Darling Basin, and their cultural, social, environmental, spiritual and economic connection to their lands and waters.

We extend our thanks to the Wakool River Association and landholders in the Edward-Wakool river system for allowing access to their properties and for their keen interest in this project. Community liaison was undertaken by staff from the Murray Local Land Services, with particular thanks to Josh Campbell. We particularly thank James Abell and Chris Smith who undertook the majority of the field work. Assistance with fieldwork and/or laboratory work was undertaken by Randy Adjonu, Tyson Arapali, Josh Campbell, Jonathon Doyle, Chris Ensor, Paris Grant-Preece, Jamie Hutchinson, Nick Ingram, Luke Jones, Tim Kaminskas, Kate Martin, Suzanne McDonald, Rohan Rehwinkel, Jack Simpson, John Stuart and Daniel Svozil. Maps were prepared by Simon McDonald and Deanna Duffy (Charles Sturt University Spatial Analysis Unit), Rod Martin (NSW DPI) and Ian Wooden (NSW DPI). Tina Hines at the Monash University Water Studies Centre processed carbon and nutrient samples. Fish sampling was carried out under NSW Fisheries license (P11/0003-1.0) and approved by the CSU Animal Care and Ethics Committee (10/101). Frog and tadpole surveys were approved by The Animal Ethics Committee of the Chief Executive of the Office of Environment and Heritage (AEC approval number: 110912/03) and were approved under NPWS scientific license (SL100218). This project was funded by the Commonwealth Environmental Water Office with in-kind contributions from Charles Sturt University, Murray Local Land Services, NSW Department of Primary Industries, Monash University and NSW Office of Environment and Heritage.

References

Agostinho, A. A., Gomes, L. C., & Pelicice, F. M. (2007) Ecology and management of fish resources in reservoirs in Brazil. Eduem.

Agostinho, A.A., Pelicice, F.M., Petry, A.C., Gomes, L.C., and Júlio Jr, H.F. (2007) Fish diversity in the upper Paraná River basin: habitats, fisheries, management and conservation. Aquatic Ecosystem Health & Management 10, 174-186.

Anderson, M., Gorley, R. N., & Clarke, R. K. (2008). Permanova+ for Primer: Guide to Software and Statistical Methods.

Anstis, M. (2002) 'Tadpoles of South-eastern Australia: a guide with keys.' (Reed New Holland: Sydney, NSW)

ANZECC water quality guidelines (2000) Australia and New Zealand guidelines for fresh and marine water quality . In ‘Australian and New Zealand environment and conservation council and Agriculture and resource management council of Australia and New Zealand’. Available at http://www.mincos.gov.au/publications/australian_and_new_zealand_guidelines_for_fresh_and_marine_water_quality

Atkinson, B.L., Grace, M.R., Hart, B.T. and Vanderkruk, K.EN. (2008) Sediment instability affects the rate and location of primary production and respiration in a sand-bed stream. Journal of the North American Benthological Society 27, 581-592.

Baker, A. and Spencer, R.G. (2004) Characterization of dissolved organic matter from source to sea using fluorescence and absorbance spectroscopy. Science of the Total Environment 333(1), 217-232.

Balcombe, S. R., Arthington, A. H., Foster, N. D., Thoms, M. C., Wilson, G. G., & Bunn, S. E. (2006) Fish assemblages of an Australian dryland river: abundance, assemblage structure and recruitment patterns in the Warrego River, Murray–Darling Basin. Marine and Freshwater Research, 57(6), 619-633.

Baldwin, D.S. (1999) Dissolved organic matter and phosphorus leached from fresh and 'terrestrially' aged river red gum leaves: implications for assessing river-floodplain interactions. Freshwater Biology 41, 675-685.

Baldwin, D.S. and Mitchell, A.M. (2000) The effects of drying and re-flooding on the sediment and soil nutrient dynamics of lowland river-floodplain systems: a synthesis. Regulated Rivers: Research & Management 16, 457-467.

Bateman, H.L., Harner, M.J. and Chung-MacCoubrey, A. (2008) Abundance and reproduction of toads (Bufo) along a regulated river in the southwestern United States: Importance of flooding in riparian ecosystems. Journal of Arid Environments 72(9), 1613-1619.

Baumgartner, L. J. (2007) Diet and feeding habits of predatory fishes upstream and downstream of a low-level weir. Journal of Fish Biology 70, 879–894.

Baumgartner, L.J., Conallin, J., Wooden, I., Campbell, B., Gee R., Robinson, W.A. and Mallen-Cooper M. (2013) Using flow guilds of freshwater fish in an adaptive management framework to simplify environmental flow delivery of semi-arid riverine systems. Fish and Fisheries Early access DOI: 10.1111/faf.12023

Beesley, L., A. J. King, F. Amtstaetter, J. D. Koehn, B. Gawne, A. Price, D. L. Nielsen, L. Vilizzi, and S. N. Meredith. (2012) Does flooding affect spatiotemporal variation of fish assemblages in temperate floodplain wetlands? Freshwater Biology 57, 2230-2246.

Bertilsson, S., Bergh, S. (1999) Photochemical reactivity of XAD-4 and XAD-8 adsorbable dissolved organic compounds from humic waters. Chemosphere 39, 2289-2300.

Bice, C. M., Gehrig, S. L., Zampatti, B. P., Nicol, J. M., Wilson, P., Leigh, S. L., & Marsland, K. (2014) Flow-induced alterations to fish assemblages, habitat and fish–habitat associations in a regulated lowland river. Hydrobiologia, 722(1), 205-222.

Blom, C.W.P.M., Voesenek, L.A.C.J., Banga, M., Engelaar, W.M.H.G., Rijnders, J.H.G.M., Vandesteeg, H.M. and Visser, E.J.W. (1994) Physiological ecology of riverside species - adaptive responses of plants to submergence. Annals of Botany 74 (3), 253-263.

Blom, C.W.P.M. and Voesenek, L.A.C.J. (1996) Flooding - the survival strategies of plants. Trends in Ecology & Evolution 11 (7), 290-295.

Bogenhuber, D., Linklater, D., Pay, T., Stoffels, R. and Healy, S. (2013) The Darling Anabranch Adaptive Management Monitoring Program Draft Report 2010–2013 Baseline to a decade. Draft Report prepared for the NSW Office of Environment and Heritage by The Murray-Darling Freshwater Research Centre, MDFRC Publication 11/2013, June, 171pp

Bogenhuber D, Wood D, Pay T and Healy S (2014) The Darling Anabranch Adaptive Management Monitoring Program Final report 2014. Final report prepared in conjunction with NSW Office of Environment and Heritage by The Murray-Darling Freshwater Research Centre, MDFRC Publication 01/2014, March, 65 pp.

Bowen, Z. H., Bovee, K. D. and Waddle, T.J. (2003) Effects of flow regulation on shallow-water habitat dynamics and floodplain connectivity. Transactions of the American Fisheries Society 132: 809-823.

Brock M, Capon S, Porter J (2006) Disturbance of plant communities dependent on desert rivers. In 'Ecology of Desert Rivers’. (Ed. R Kingsford) pp. 100-132. (Cambridge University Press: Cambridge, UK).

Bunn, S.E. and Arthington A.H. (2002) Basic Principles and Ecological Consequences of Altered Flow Regimes for Aquatic Biodiversity. Environmental Management 30, 492-507

Burns, A. and K. F. Walker (2000) Biofilms as food for decapods (Atyidae, Palaemonidae) in the River Murray, South Australia. Hydrobiologia 437: 83–93.

Cadwallader, P. L. (1977) JO Langtry's 1949-50 Murray River investigations. Fisheries and Wildlife Division, Victoria.

Casanova, M., Brock, M. (2000) How do depth, duration and frequency of flooding influence the establishment of wetland plant communities? Plant Ecology 147, 237-250.

Clements, S., Jepsen, D., Karnowski, M., & Schreck, C. B. (2005) Optimization of an acoustic telemetry array for detecting transmitter-implanted fish. North American Journal of Fisheries Management, 25(2), 429-436.

Coble, P.G. (1996) Characterization of marine and terrestrial DOM in seawater using excitation-emission matrix spectroscopy. Marine Chemistry 51, 325-346.

Commonwealth Environmental Water Office (2013a) Environmental watering in the Edward-Wakool River System: Environmental Watering 2012-13  Accessed February 25, 2013 from: http://www.environment.gov.au/ewater/southern/murray/edward-wakool.html

Commonwealth Environmental Water Office (2013b) Water Use Minute number 142 (dated 6 September 2013) Use of Commonwealth environmental water in the Edward-Wakool River System within the mid-Murray catchment in 2013-14.

Covich, A.P., Palmer, M.A., and Crowl, T.A. (1999) The role of benthic invertebrate species in fresh-water ecosystems. BioScience 49:119–28

Crook DA (2004a) Movements associated with home-range establishment by two species of lowland river fish. Canadian Journal of Fisheries and Aquatic Sciences 61, 2183–2193

Crook, D. (2004b) Is the home range concept compatible with the movements of two species of lowland river fish. Journal of Animal Ecology 73, 353-366.

Crook, D. A., O'Mahony, D. J., Sanger, A. C., Munro, A. R., Gillanders, B. M., & Thurstan, S. (2009) Development and evaluation of methods for osmotic induction marking of golden perch Macquaria ambigua with calcein and alizarin red S. North American Journal of Fisheries Management, 29(2), 279-287

Crowl, T.A., McDowell, W.H., Covich, A.P. and Johnson, S.L. (2001) Freshwater shrimp effects on detrital processing and nutrients in a tropical headwater stream. Ecology 82, 775–73

Dahlen, J., Bertilsson, S., and Pettersson, C. (1996) Effects of UV-A irradiation on dissolved organic matter in humic surface waters. Environment International 22(5), 501-506.

David, B.D. & Closs, G.P. (2002) Behaviour of a stream-dwelling fish before, during, and after high-discharge events. Transactions of the American Fisheries Society 131: 762–771.

Ebner, B. (2006) Murray cod an apex predator in the Murray River, Australia. Ecology of Freshwater Fish 15, 510–520.

Elliott, S., Lead, J.R., Baker, A. (2006) Characterisation of the fluorescence from freshwater, planktonic bacteria. Water Research 40, 2075-2083.

Gillespie, G (2002) Impacts of sediment loads, tadpole density, and food type on the growth and development of tadpoles of the spotted tree frog Litoria spenceri: an instream ecperiment. Biological conservation 106, 141-150.

Gillespie, G. and Hines, H. (1999) Status of temperate riverine frogs in South-Eastern Australia. In 'Declines and Disappearances of Australian Frogs'. (Ed. A. Campbell) pp. 109-130. (Environment Australia: Canberra)

Gilligan, D., Vey, A., & Asmus, M. (2009) Identifying drought refuges in the Wakool system and assessing status of fish populations and water quality before, during and after the provision of environmental, stock and domestic flows. NSW Department of Primary Industries-Fisheries Final Report Series, (110).

Grace, M.R. and Imberger, S.J. (2006) Stream Metabolism: Performing & Interpreting Measurements. Water Studies Centre Monash University, Murray Darling Basin Commission  and New South Wales Department of Environment and Climate Change. 204 pp. Accessed at http://monash.edu/science/wsc/

Green, D. (2001) The Edward-Wakool System: River Regulation and Environmental Flows. Department of Land and Water Conservation. Unpublished Report.

Hadwen, W.L., Fellows, C.S., Westhorpe, D.P., Rees, G.N., Mitrovic, S.M., Taylor, B., Baldwin, D.S., Silvester, E., Croome, R. (2010) Longitudinal trens in river functioning: patterns of nutrient and carbon processing in three Australian rivers. River Research and Applications 26, 1129-1152.

Hancock, M.A. and Bunn, S.E. (1997) Population dynamics and life history of Paratya australiensis Kemp, 1917 (Decapoda:Atyidae) in upland rainforest streams, south-eastern Queensland, Australia. Marine and Freshwater Research 48, 361–369

Healey, M., Thompson, D. and Robertson, A. (1997) Amphibian communities associated with billabong habitats on the Murrumbidgee floodplain, Australia. Australian Journal of Ecology 22, 270-278.

Heard, G.W., Robertson, P., and Scroggie, M.P. (2006) Assessing detection probabilities for the endangered growling grass frog (Litoria raniformis) in southern Victoria. Wildlife Research 33(7), 557–564

Hladyz, S., Watkins, S.C., Whitworth, K.L., and Baldwin, D.S. (2011) Flows and hypoxic blackwater events in managed ephemeral river channels. Journal of Hydrology 401(1-2), 117-125.

Howitt, J.A., Baldwin, D.S., Rees, G.N., Hart, B.T. (2008) Photodegradation, interaction with iron oxides and bioavailability of dissolved organic matter from forested floodplain sources. Marine and Freshwater Research 59, 780-791.

Howitt, J.A., Baldwin, D.S., Rees, G.N., and Williams, J.L. (2007) Modelling blackwater: Predicting water quality during flooding of lowland river forests. Ecological Modelling 203(3-4), 229-242.

howitt et al 2008

Humphries, P., King, A.J. and Koehn, J.D. (1999) Fish, flows and flood plains: Links between freshwater fishes and their environment in the Murray-Darling River system, Australia. Environmental Biology of Fishes 56, 129-151.

Humphries, P., & Lake, P. S. (2000) Fish larvae and the management of regulated rivers. Regulated Rivers: Research & Management, 16(5), 421-432.

Humphries, P., Serafini, L.G. and King, A.J. (2002) River regulation and fish larvae: variation through space and time. Freshwater Biology 47, 1307-1331.

Humphries, P., Cook, R.A., Richardson, A.J. and Serafini, L.G. (2006) Creating a disturbance: manipulating slackwaters in a lowland river. River Research Applications 22, 525–542.

Humphries, P., Richardson, A.J., Wilson, G.A. and Ellison, T. (2013) River regulation and recruitment in a protracted-spawning riverine fish. Ecological Applications 23, 208-225.

Johansson, M.E. and Nilsson, C. (2002) Responses of Riparian Plants to Flooding in Free-Flowing and Regulated Boreal Rivers: an Experimental Study. Journal of Applied Ecology 39, 971-986.

Jones MJ, Stuart IG. (20090 Lateral movement of common carp (Cyprinus carpio L.) in a large lowland river and floodplain. Ecology of Freshwater Fish 18, 72–82.

Junk, W.J., Bayley, P.B.,  Sparks, R.E. (1989) The flood pulse concept in river-floodplain systems. Canadian Journal of Fisheries and Aquatic Sciences 106, 110-127.

Kaminskas, S., & Humphries, P. (2009) Diet of Murray cod (Maccullochella peelii peelii)(Mitchell) larvae in an Australian lowland river in low flow and high flow years. Hydrobiologia 636(1), 449-461.

Kehr, J.M, Merritt, D.M. and Stromberg, J.C .(2013) Linkages between primary seed dispersal, hydrochory and flood timing in a semi-arid region river. Journal of Vegetation Science, http://dx.doi.org/10.1111/jvs.12061

King, A. J. (2004) Density and distribution of potential prey for larval fish in the main channel of a floodplain river: pelagic versus epibenthic meiofauna. River Research and Applications 20(8), 883-897.

King, A. J., Crook, D. A., Koster, W. M., Mahoney, J., & Tonkin, Z. (2005) Comparison of larval fish drift in the Lower Goulburn and mid‐Murray Rivers. Ecological Management & Restoration 6(2), 136-139.

King, A.J, Humphries, P. and McCasker, N. (2013) Reproduction and early life history. In: 'Ecology of Australian Freshwater Fish'. (Eds. P Humphries  and K Walker ). pp.159-192. (CSIRO Pub.: Collingwood, Vic.)

King, A. J., Humphries, P., & Lake, P. S. (2003) Fish recruitment on floodplains: the roles of patterns of flooding and life history characteristics. Canadian Journal of Fisheries and Aquatic Sciences 60(7), 773-786.

King, A.J., Ward, K.A., O'Connor, P., Green, D., Tonkin, Z. and Mahoney, J. (2009) Adaptive management of an environmental watering event to enhance native fish spawning and recruitment. Freshwater Biology 55, 17-31.

King, A.J., Tonkin, Z. and Mahoney, J. (2009) Environmental flow enhances native fish spawning and recruitment in the Murray River, Australia. River Research and Applications 25, 1205-1218.

Kingsford, R.T., Watts, R.J. (eds) (2014) Flow dependent ecological responses. A technical report from the Ecological Responses to Altered Flow Regimes Flagship Research Cluster (SubProject 3). CSIRO Water for a Healthy Country Flagship, Australia.

Koehn, J. D. (2009) Multi‐scale habitat selection by Murray cod Maccullochella peelii peelii in two lowland rivers. Journal of Fish Biology 75(1), 113-129.

Koehn J.D. and Harrington D.J. (2006) Conditions and timing of the spawning of Murray cod (Maccullochella peelii peellii) and the endangered trout cod (M. macquariensis) in regulated and unregulated rivers. Rivers Research and Applications 22, 327-343.

Koehn JD, McKenzie JA, O’Mahony DJ, Nicol SJ, O’Connor JP, O’Connor WG. (2009) Movements of Murray cod (Maccullochella peelii peelii) in a large Australian lowland river. Ecology of Freshwater Fish 18, 594–602.

Koster WM, Dawson DR, O’Mahony DJ, Moloney PD, Crook DA (2014) Timing, Frequency and Environmental Conditions Associated with Mainstem–Tributary Movement by a Lowland River Fish, Golden Perch (Macquaria ambigua). PLoS ONE 9(5): e96044. doi:10.1371/journal.pone.0096044

Lake, J. S. (1967) Rearing experiments with five species of Australian freshwater fishes. I. Inducement to spawning. Marine and Freshwater Research 18(2), 137-154.

Leigh, S. J., & Zampatti, B. P. (2013) Movement and mortality of Murray cod, Maccullochella peelii, during overbank flows in the lower River Murray, Australia. Australian Journal of Zoology 61(2), 160-169.

Lowe, B.J. (2002) Impacts of altered flow regime on riverbank vegetation in a lowland floodplain river. Unpublished PhD thesis, Charles Sturt University.

Lucas, M. C., Baras, E., Thom, T. J., Duncan, A., & Slavík, O. (2001) Migration of freshwater fishes (Vol. 47). Oxford: Blackwell Science.

Lyon, J. P., & O'Connor, J. P. (2008) Smoke on the water: Can riverine fish populations recover following a catastrophic fire‐related sediment slug? Austral Ecology 33(6), 794-806.

Lyon, J., Stuart, I., Ramsey, D., & O'Mahony, J. (2010) The effect of water level on lateral movements of fish between river and off-channel habitats and implications for management. Marine and Freshwater Research 61(3), 271-278.

Lytle, D. A. and N. L. Poff. (2004) Adaptation to natural flow regimes. Trends in Ecology & Evolution 19, 94-100.

Macdonald, J. I., & Crook, D. A. (2014) Nursery sources and cohort strength of young‐of‐the‐year common carp (Cyprinus carpio) under differing flow regimes in a regulated floodplain river. Ecology of Freshwater Fish 23(2), 269-282.

Mallen‐Cooper, M., & Stuart, I. G. (2003) Age, growth and non‐flood recruitment of two potamodromous fishes in a large semi‐arid/temperate river system. River research and applications 19(7), 697-719.

March, J. G., Benstead, J. P., Pringle, C. M., and Ruebel, M. W. (2001) Linking shrimp assemblages with rates of detrital processing along an elevational gradient in a tropical stream. Canadian Journal of Fisheries and Aquatic Sciences 58(3), 470-478.

McCasker, N. G. (2009) Of life and death in a lowland river: Investigating mortality during the early life history stages of Murray-Darling Fishes. PhD Thesis. Charles Sturt University. Pp 102.

Mobed, J.J., Hemmingsen, S.L., Autry, J.L. and McGown, L.B. (1996) Fluorescence characterization of IHSS humic substances: total luminescence spectra with absorbance correction. Environmental Science & Technology 30(10), 3061-3065.

Mokany, A. (2007) Impact of tadpoles and mosquito larvae on ephemeral pond structure and processes. Marine and Freshwater Research 58, 436 -444.

Moran, M.A. and Hodson, R.E. (1990) Bacterial production on humic and nonhumic components of dissolved organic carbon. Limnology and Oceanography 35(8), 1744-1756.

O'Connell, M., Baldwin, D.S., Robertson, A.I. and Rees, G. (2000) Release and bioavailability of dissolved organic matter from floodplain litter: influence of origin and oxygen levels. Freshwater Biology 45, 333-342.

O’Connor JP, O’Mahony DJ, O’Mahony JM (2005) Movements of Macquaria ambigua, in the Murray River, south-eastern Australia. Journal of Fish Biology 66, 392–403.

Odum, H.T. (1956) Primary production in flowing waters, Limnology & Oceanography 2, 102–117.

Oliver, R.L., Hart, B.T., Olley, J., Grace, M.R., Rees, C.M. and Caitcheon, C.G. (1999) The Darling River: Algal Growth and the Cycling and Sources of Nutrients, Murray Darling Basin Commission Report M386, CRC for Freshwater Ecology & CSIRO Land and Water, pp 200.

Price, A. E., and Humphries, P. (2010) The role of dispersal and retention in the early life stages of shrimp in a lowland river. Canadian Journal of Fisheries and Aquatic Sciences 67(4), 720-729.

Price, A. E., Humphries, P., Gawne, B. and Thoms, C. (2013) Effects of discharge regulation on slackwater characteristics at multiple scales in a lowland river. Canadian Journal of Fisheries and Aquatic Sciences 70, 253-262.

R Development Core Team (2013) R: A Language and Environment for Statistical Computing. R Foundation for Statistical Computing, Vienna, Austria.

Ralph, T., Spencer, J. and Rayner, S. (2011) Fish. In 'Floodplain Wetland Biota in the Murray-Darling Basin.’ (Eds K Rogers and TJ Ralph) pp. 205-251. (CSIRO Publishing: Collingwood, VIC).

Rayner, T. S., K. M. Jenkins, and R. T. Kingsford. (2009) Small environmental flows, drought and the role of refugia for freshwater fish in the Macquarie Marshes, arid Australia. Ecohydrology 2, 440-453.

Reid, M., Ogden, R. and Thoms, M. (2011) The influence of flood frequency, geomorphic setting and grazing on plant communities and plant biomass on a large dryland floodplain. Journal of Arid Environments 75(9), 815-826.

Reynolds, L. F. (1983) Migration patterns of five fish species in the Murray-Darling River system. Marine and Freshwater Research 34(6), 857-871.

Richardson, A.J., Growns, J.E. and Cook, R.A. (2004) Distribution and life history of caridean shrimps in regulated lowland rivers in southern Australia. Marine and Freshwater Research 55, 295-308.

Roberts, D. T., Duivenvoorden, L. J., & Stuart, I. G. (2008) Factors influencing recruitment patterns of Golden Perch (Macquaria ambigua oriens) within a hydrologically variable and regulated Australian tropical river system. Ecology of Freshwater Fish 17(4), 577-589.

Roberts, J. and Marston, F. (2011) Water regime for wetland and floodplain plants: A source book for the Murray-Darling Basin. National Water Commission, Canberra.

Roberts, B.J., Mulholland, P.J. and Hill, W.R. (2007) Multiple Scales of Temporal Variability in Ecosystem Metabolism Rates: Results from 2 Years of Continuous Monitoring in a Forested Headwater Stream, Ecosystems 10, 588–606.

Robertson, A.I., Bacon, P. and Heagney, G. (2001) The responses of floodplain primary production to flood frequency and timing. Journal of Applied Ecology 38, 126-136.

Robertson, A.I., Bunn, S.E., Boon, P.I., and Walker, K.F. (1999) Sources, sinks and transformations of organic carbon in Australian floodplain rivers. Marine and Freshwater Research 50, 813-829

Robinson, W.A. (2012) Calculating statistics, metrics, sub-indicators and the SRA Fish theme index: A Sustainable Rivers Audit Technical report, Consultancy report to the Murray-Darling Basin Authority.

Rolls, R. J., I. O. Growns, T. A. Khan, G. G. Wilson, T. L. Ellison, A. Prior, and C. C. Waring. (2013) Fish recruitment in rivers with modified discharge depends on the interacting effects of flow and thermal regimes. Freshwater Biology 58, 1804-1819.

Rowland, S.J. (1983) Spawning of the Australian freshwater fish Murray cod, Maccullochella peeli (Mitchell), in earthen ponds. Journal of Fish Biology 23, 525-534.

Rowland, S.J. (1998) Aspects of the reproductive biology of Murray cod, Maccullochella peelii peelii. Proceedings of the Linnean Society of New South Wales 1998, 147-162.

Simpson, R. R., & Mapleston, A. J. (2002) Movements and habitat use by the endangered Australian freshwater Mary River cod, Maccullochella peelii mariensis. Environmental Biology of Fishes 65(4), 401-410.

Sims N.C. and Thoms, M.C. (2002) 2 FLOOD PLAINS: The role of temporary storage in the exchange of sediment and nutrients-What happens when flood plains wet themselves: Vegetation response to inundation on the lower Balonne flood. IAHS Publications-Series of Proceedings and Reports-Intern Assoc Hydrological Sciences 276, 195-202.

Spencer, J.A. and Wassens, S. (2009) 'Responses of waterbirds, fish and frogs to environmental flows in the Lowbidgee wetlands in 2008-09. Final report for the NSW Rivers Environmental Restoration Program.' (Rivers and Wetland Unit, NSW Department of Environment and Climate Change: Sydney.)

Stuart, I. G., & Jones, M. (2006) Large, regulated forest floodplain is an ideal recruitment zone for non-native common carp (Cyprinus carpio L.). Marine and Freshwater Research, 57(3), 333-347.

Stuart, I.G. and Jones, M.J. (2006) Movement of common carp, Cyprinus carpio, in a regulated lowland Australian river: implications for management. Fisheries Management and Ecology 13, 213-219.

Taylor, M. K., & Cooke, S. J. (2012) Meta-analyses of the effects of river flow on fish movement and activity. Environmental Reviews 20(4), 211-219.

Thiem, J. D., Hatin, D., Dumont, P., Van Der Kraak, G., & Cooke, S. J. (2013) Biology of lake sturgeon (Acipenser fulvescens) spawning below a dam on the Richelieu River, Quebec: behaviour, egg deposition, and endocrinology. Canadian Journal of Zoology 91(3), 175-186.

Thoms, M., Reid, M., Christianson, K., & Munro, F. (2006) Variety is the Spice of River Life: Recognising Hydraulic Diversity as a Tool for Managing Flows in Regulated Rivers. In J. S. Rowan, Duck, R.W. and Werritty, A. (Eds), Sediment dynamics and the hydromorphology of fluvial systems, Wallingford, UK, IAHS Press . IAHS Publications 306, 169-178.

Thoms, M. C., & Sheldon, F. (2000) Lowland rivers: an Australian introduction. Regulated Rivers: Research & Management 16(5), 375-383.

Toner, M. and Keddy, P. (1997) River hydrology and riparian wetlands - a predictive model for ecological assembly.  Ecological Applications 7 (1), 236-246.

Tonkin, Z., King, A., Mahoney, J., & Morrongiello, J. (2007) Diel and spatial drifting patterns of silver perch Bidyanus bidyanus eggs in an Australian lowland river. Journal of Fish Biology 70(1), 313-317.

Uehlinger, U. (2000) Resistance and resilience of ecosystem metabolism in a flood-prone river system. Freshwater Biology 45, 319–332.

Underwood, A.J. (1991) Beyond BACI: Experimental designs for detecting human environmental impacts on temporal variations in natural populations. Australian Journal of Marine and Freshwater Research 42, 569-587.

Vietz, G. J., M. J. Sammonds, and M. J. Stewardson (2013) Impacts of flow regulation on slackwaters in river channels. Water Resources Research 49, 1797–1811.

Vink, S., Bormans, M., Ford, P.W. and Grigg, N.J. (2005) Quantifying ecosystem metabolism in the middle reaches of the Murrumbidgee River during irrigation flow releases. Marine and Freshwater Research 56, 227-241.

Walsh, C. T., Reinfelds, I. V., Ives, M. C., Gray, C. A., West, R. J., & van der Meulen, D. E. (2013) Environmental influences on the spatial ecology and spawning behaviour of an estuarine-resident fish, Macquaria colonorum. Estuarine, Coastal and Shelf Science 118, 60-71.

Warfe, D. and Barmuta, L. (2006) Habitat structural complexity mediates food web dynamics in a freshwater macrophyte community. Oecologia 150(1), 141-154.

Wassens, S., Hall, A., Osborne, W. and Watts, R.J. (2010) Habitat characteristics predict occupancy patterns of the endangered amphibian Litoria raniformis in flow-regulated flood plain wetlands. Austral Ecology 35(8), 944-955.

Wassens, S., Healy, S. & Watts, R. (2011) Optimising frog breeding responses to flooding in managed wetlands (ii) frog distributions through the Murray Floodplain report to Murray Wetlands Working Group, Albury

Wassens, S. and Maher, M. (2011) River regulation influences the composition and distribution of inland frog communities. River Research and Applications 27(2), 238-246.

Watkins, S., Hladyz, S., Whitworth, K. and Baldwin, D. (2010) Understanding the relationship between low dissolved oxygen blackwater events and managed flows in the Edward-Wakool River system. Report prepared for the Murray Catchment Management Authority by The Murray-Darling Freshwater Research Centre, MDFRC Publication 12/2010, June, 66 pp.

Watts, R.J., Kopf, R.K., Hladyz, S., Grace, M., Thompson, R., McCasker, N., Wassens, S., Howitt, J.A., A. Burns, and Conallin, J. (2013a) Monitoring of ecosystem responses to the delivery of environmental water in the Edward-Wakool system, 2011-2012. Institute for Land, Water and Society, Charles Sturt University, Report 2. Prepared for Commonwealth Environmental Water.

Watts, R.J., McCasker, N., Baumgartner, L., Bowen, P., Burns, A., Conallin, A., Dyer, J.G., Grace, M., Healy, S., Howitt, J.A., Kopf, R.K., Wassens, S., Watkins, S., Wooden I. (2013b) Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool river system, 2012-13. Institute for Land, Water and Society, Charles Sturt University, Final Report. Prepared for Commonwealth Environmental Water.

Watts, R.J., McCasker, N., Baumgartner, L., Bond, N. , Bowen, P. , Conallin, A., Grace, M., Healy, S., Howitt, J.A., Kopf, R.K., Scott, N. , Thiem, J. , Wooden I. (2014) Monitoring and Evaluation Plan for the Edward-Wakool Selected Area’, Commonwealth of Australia 2014’.

White, L. J., Harris, J. H., & Keller, R. J. (2011) Movement of three non‐salmonid fish species through a low‐gradient vertical‐slot fishway. River Research and Applications 27(4), 499-510.

Zampatti, B. P., & Leigh, S. J. (2013a) Effects of flooding on recruitment and abundance of Golden Perch (Macquaria ambigua ambigua) in the lower River Murray. Ecological Management & Restoration 14(2), 135-143.

Zampatti, B. P., & Leigh, S. J. (2013b) Within-channel flows promote spawning and recruitment of golden perch, Macquaria ambigua ambigua–implications for environmental flow management in the River Murray, Australia. Marine and Freshwater Research 64(7), 618-630.

Zampatti, B. P., Wilson, P. J., Baumgartner, L., Koster, W., Livore, J., Thiem, J., Tonkin, Z., & Ye, Q. (2014). Reproduction and recruitment of golden perch (Macquaria ambigua ambigua) in the southern Murray-Darling Basin in 2013/14: an exploration of river-scale response, connectivity and population dynamics. Draft report to the Murray-Darling Basin Authority.

Zuur, A., E. N. Leno N. Walker, A. A. Saveliev, and G. M. Smith. (2009) Mixed effects models and extensions in ecology with R. Springer.

Appendices

Appendix 1. Watering option 1 – Edward-Wakool River system instream flows (from CEWO 2013)

Appendix 2. Watering option 3 – Mid-Murray region water quality and habitat (from CEWO 2013)

Appendix 3. Overall expected outcomes from the use of Commonwealth environmental water in the mid-Murray region (source: CEWO 2013).

| Flow type | Expected outcomes for 2013-14 | Contributions to longer term outcomes | Contribution to the following Basin Plan objective |
| --- | --- | --- | --- |
| Base flows and freshes | End of system flows | Connectivity | Ecosystem function |
|  | Primary production | Process |  |
|  | Resistance | Population resilience | Resilience |
|  | Dissolved oxygen | Chemical | Water Quality |
| Overbank | Dissolved organic carbon | Chemical | Water Quality |
| All flow types | Within ecosystem diversity | Landscape ecosystem diversity | Biodiversity |
|  | Vegetation condition and reproduction | Landscape vegetation diversity |  |
|  | Fish reproduction Fish condition | Landscape fish diversity Fish larval growth and survival |  |
|  | Waterbird survival and condition. Waterbird reproduction. Waterbird fledging | Landscape bird diversity |  |
|  | Other vertebrate condition Other vertebrate reproduction | Other vertebrates |  |
|  | Hydrological connectivity Biotic dispersal Sediment transport | Connectivity | Ecosystem function |
|  | Refuge Avoidance | Landscape refuge | Resilience |