Report 2

October 2012

Monitoring of ecosystem responses to the delivery of environmental water in the Edward-Wakool system, 2011-2012

Watts, R.J.a, McCasker, N.a, Baumgartner, L.b, Bowen, P.c , Burns, A. a, Conallin, A.c, Dyer, J.G.a, Grace, M.d, Healy, S.e, Howitt, J.A.a, Kopf, R.K.a, Wassens, S.a, Watkins, S.a, Wooden I.b (2013). Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool river system, 2012-13. Institute for Land, Water and Society, Charles Sturt University, Final Report. Prepared for Commonwealth Environmental Water.

a. Institute for Land, Water and Society  		  b. NSW Trade & Investment

Charles Sturt University 			                       Narrandera Fisheries Centre,

PO Box789, Albury, NSW 2640			       PO Box 182, Narrandera NSW 2700

c.  Murray Catchment Management Authority

PO Box 835, Deniliquin, NSW 2710

d. Water Studies Centre, Monash University,	   	  e. NSW Office of Environment and Heritage

Clayton, Victoria, 3800, AUSTRALIA		     PO Box 363, Buronga NSW 2739

Copyright
© Copyright Commonwealth of Australia, 2014

‘Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool River, 2012-13’ is licensed by the Commonwealth of Australia for use under a Creative Commons By Attribution 3.0 Australia licence with the exception of the Coat of Arms of the Commonwealth of Australia, the logo of the agency responsible for publishing the report, content supplied by third parties, and any images depicting people. For licence conditions see: http://creativecommons.org/licenses/by/3.0/au/

This report should be attributed as ‘Monitoring the ecosystem responses to Commonwealth environmental water delivered to the Edward-Wakool river system 2012-13, Commonwealth of Australia 2014’.

The Commonwealth of Australia has made all reasonable efforts to identify content supplied by third parties using the following format ‘© Copyright’.

Disclaimer

The views and opinions expressed in this publication are those of the authors and do not necessarily reflect those of the Australian Government or the Minister for the Environment.

While reasonable efforts have been made to ensure that the contents of this publication are factually correct, the Commonwealth does not accept responsibility for the accuracy or completeness of the contents, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the contents of this publication.

Authorship

This project was undertaken as a collaboration among Charles Sturt University, NSW DPI (Fisheries), Murray Catchment Management Authority, Monash University and NSW Office of Environment and Heritage. All authors contributed to project design, implementation and reporting. There was on-going collaboration among team members during the project and internal and external review of the report. The table below lists the lead authors for each section of the report.

| Section | Section Title | Lead authors |
| --- | --- | --- |
| 1 | Introduction | Watts et al. |
| 2 | Background | Watts et al. |
| 3 | Commonwealth water use options and flow-dependant ecological objectives 2012-2013 | Watts et al. |
| 4 | Environmental watering in 2012-13 | Watts et al. |
| 5 | Monitoring design and location of sampling sites | Watts et al. |
| 6 | Indicators | Watts et al. |
| 7 | Responses of indicators |  |
| 7.1.1 | Inundation modelling | Watts |
| 7.1.2 | Habitat assessment | Healy |
| 7.2.1 | Water quality and chemistry | Howitt and Grace |
| 7.2.2 | Organic matter characterisation | Howitt |
| 7.2.3 | Phytoplankton biomass | Watts |
| 7.2.4 | Biofilm biomass and composition | Watts and Burns |
| 7.2.5 | Whole stream metabolism | Grace |
| 7.3.1 | Zooplankton | Watkins |
| 7.3.2 | Shrimp | Dyer and Conallin |
| 7.3.3 | Murray crayfish and the common yabby | Wooden |
| 7.3.4 | Frogs | Healy and Wassens |
| 7.4.1 | Fish movement | Wooden and Baumgartner |
| 7.5.1 | Fish spawning | McCasker |
| 7.5.2 | Fish recruitment | Kopf and Wooden |
| 7.5.3 | Long-term fish community structure (2010-2013) | Wooden and Baumgartner |
| 8 | Stakeholder engagement | Conallin and Bowen |
| 9 | Synthesis |  |
| 9.1 | Objective 1 - Support habitat requirements of native aquatic species, maintain health of existing extent of riparian, floodplain and wetland native vegetation communities | Watts and Healy |
| 9.2 | Objective 2 - Support ecosystem functions that relate to mobilisation, transport and dispersal of biotic and abiotic material | Howitt and Grace |
| 9.3 | Objective 3 - Support breeding and recruitment of frogs and invertebrates | Watkins, Dyer, Conallin, Wooden and Healy |
| 9.4 | Objective 4 - Support ecosystem functions that relate to longitudinal connectivity and lateral connectivity to maintain populations | Watts, Wooden and Kopf |
| 9.5 | Objective 5 - Support breeding and recruitment of native fish | McCasker, Kopf, Wooden, Baumgartner |
| 10 | Environmental watering recommendations | Watts et al. |

Table of Contents

7. Responses of indicators against environmental watering objectives	4

7.4.	Objective 4: Support ecosystem functions that relate to longitudinal connectivity and lateral connectivity to maintain populations	4

7.4.1.	Fish movement	4

7.5.	Objective 5: Support breeding and recruitment of native fish	25

7.5.1.	Fish spawning (abundance of larval fish and eggs)	25

7.5.2.	Fish recruitment	46

7.5.3.	Long-term fish community structure (2010-2013)	63

8. Synthesis	87

8.1.	Objective 1: Support habitat requirements of native aquatic species, maintain health of existing extent of riparian, floodplain and wetland native vegetation communities	87

8.2.	Objective 2: Support ecosystem functions that relate to mobilisation, transport and dispersal of biotic and abiotic material	90

8.3.	Objective 3: Support breeding and recruitment of frogs, turtles, invertebrates	92

8.4.	Objective 4: Support ecosystem functions that relate to longitudinal connectivity and lateral connectivity to maintain populations	94

8.5.	Objective 5: Support breeding and recruitment of native fish	96

8.6.	Conclusion	101

9. Environmental watering recommendations	102

10. Acknowledgements	105

11. References	106

12. Appendices	122

- Responses of indicators against environmental watering objectives

  - Objective 4: Support ecosystem functions that relate to longitudinal connectivity and lateral connectivity to maintain populations

Key findings

- Murray cod, golden perch, silver perch and carp all displayed increased activity in response to increasing temperature and flow during spring and early summer. The period of increased movement corresponded with spawning periods for these species.

- Individual displacement for Murray cod ranged from 10 km downstream to 50 km upstream. Overall displacement of Murray cod from the refuge pool was strongly correlated with the hydrograph; 95% of the larger movements occurred between August and November 2012.

- Golden perch exhibited a displacement range of 150 km downstream to 10 km upstream. Peak displacement occurred from mid-September to mid-October 2012. Environmental water delivered to Yallakool Creek from 0ctober 2012 to December 2012 did not result in large scale displacement of the golden perch population, however some individuals moved 5 to 15 km. The environmental watering in March April 2013 resulted in minimal movement by golden perch. Golden perch spent 80% of their time in the refuge pool.

- Silver perch were highly mobile, undertaking frequent short (<2 km) return movements. Some movement occurred during environmental watering actions, however this pattern of movement was not restricted to these flow events. Sliver perch were detected mostly outside the refuge pool, displaying a preference for upstream areas where flow velocity was higher in the Yallakool Creek. By June 2013 after cessation of flows, all fish returned to the refuge pool, demonstrating the importance of this habitat in sustaining native fish in the region.

- Tagged carp displayed periods of large displacing movements interspersed with periods of sedentary behaviour, with most movement occurring between August and November.

Fish movement

Background

Freshwater fish are highly mobile and move in response to several stimuli in order to spawn, disperse and feed (Lucas et al. 2001). Flow is often a major cue for freshwater fish migration and many species move in response to water flows (Agostinho et al. 2007). Delivery of environmental flows is therefore expected to initiate some type of fish movement response in Australian freshwater fish if appropriately timed and managed to benefit target species.

Acoustic tracking is a useful method for obtaining information on fish movements. The process involves implanting a transmitter into a fish, which is then detected by a series of stationary readers installed in a target stream. Acoustic monitoring can provide high resolution spatial information on fish location, and data can be graphically presented to identify movement patterns (Barnett et al. 2010). In the case of environmental water delivery, the strategic placement of acoustic receivers could provide information on timing of movements, distances travelled, residency, correlation with water flow and, potentially, evidence of spawning behaviour or nest site selection. Such information is of great importance to determine both the delivery success of a particular environmental water volume or to inform the planning of future events.

Murray cod spawn between mid-October and late-November (Humphries 2005) independent of flow conditions (Humphries and Lake 2000, King et al 2009a), thus increasing available spawning habitat through water delivery is expected to lead to increased spawning success. It is hypothesised that environmental water delivery within the Murray cod spawning season would result in a movement response in adult Murray cod consistent with that of spawning fish. That is, Murray cod would take advantage of increased flows, leaving refuge habitat during the spawning period and returning at the completion. It was hypothesised that environmental water delivery outside of the Murray cod spawning period would facilitate return movements in post spawn Murray cod.

Golden perch and silver perch spawning occurs from early November to March (Roberts et al. 2008; King et al. 2009a). Spawning in these species has been linked to a rise in both water level and temperature (Mallen-Cooper and Stuart 2003) with in-channel spawning evident outside of flood years (King et al. 2009). Both these species are known to undertake large migrations, thought to possibly be spawning related (Reynolds 1983). It was hypothesised that environmental water delivery could stimulate similar movements, consistent with that of spawning fish or movement that serves some other life history strategy. This section reports on the success of environmental water delivery and suggestions for management of future events.

Methods

To monitor fish movement responses in relation to environmental water delivery using acoustic telemetry methods, a series of receivers was deployed along the Wakool River and Yallakool Creek (Figure 64). Fish movement studies focused on the largest refuge pools in the system Wakool Reserve, which is located at the junction of Yallakool Creek and Wakool River. The pool is approximately 5 km long in low flow conditions and provides a valuable low flow refuge. The pool supports a diverse native fish community and is a popular recreational angling location. There is scope to manipulate flows into the pool through controlled operation of irrigation escapes and regulators on the Yallakool Creek and Wakool River.

A linear array of 48 VEMCO VR2W (VEMCO Ltd., Halifax, Nova Scotia, Canada) acoustic receivers was deployed in the Edward Wakool river system in 2010. VR2W acoustic receivers are submerged, single channel (69 kHz), omni-directional receivers which record time, date and identity of acoustic tagged fish swimming within detection range of the receiver units. The array incorporates the Wakool River from Gee Gee Bridge upstream to the Edward River offtake and along the length of the Yallakool Creek. From here the Edward array encompassed the Edward River from Stevens Weir upstream to the Murray River Offtake, the length of Gulpa Creek and the lower reaches of Bullatale and Tuppal creeks (Figure 64). The Edward and Wakool arrays combined provide stream coverage of approximately 430 km with a mean distance of approximately 8 km between receivers.  Receiver site locations were strategically selected for approximately equidistant spacing, with additional receivers at key locations such as remnant pools, potential barriers, stream confluences and outer extremities of the study area. Additional receivers were deployed in the remnant junction pool below Wakool Reserve Road Bridge to investigate small scale movement patterns in the refuge pool and to maximise detection of the tagged population during periods of high visitation. The receivers provided continuous monitoring throughout the study period, with data retrieval conducted quarterly and batteries replaced annually.

Figure 64. Overview of acoustic receiver array used to detect fish movements in response to environmental water delivery in the Edward-Wakool system. The array was established to detect movements in the Wakool / Yallakool River (orange) and also the upper Edward system (yellow). Detailed coverage of the original tagging location at the Wakool – Yallakool junction is enlarged for clarity.

Electrofishing was used to capture four large-bodied fish species, Murray cod (Maccullochella peelii), golden perch (Macquaria ambigua), silver perch (Bidyanus bidyanus) and alien common carp (Cyprinus carpio), for insertion with an acoustic tag. Captured fish were transferred to a bank-side surgery station and anesthetised in a 200 L oxygenated tank of ambient water containing 50 mgL-1 benzocaine (ethyl-p-aminobenzoate) (Sigma Aldrich, Shanghai) in preparation for surgical tag insertion. Surgery followed the procedure described in Butler et al. (2009) and tagged fish were released at the capture site (Figure 65, 66). Where possible, the sex of the fish was determined by examining the gonads through the incision prior to transmitter insertion.

Receiver data was downloaded and stored in a purpose built SQL database. Prior to analyses, single detections were removed (Clements et al. 2005). The detection data were then visualised in Eonfusion software (Myriax Software Pty Ltd) by plotting fish movement on the river network to identify and exclude detections falling out of sequence (false detections). All remaining detection data were plotted on to the river network to generate time series movement video files. Data was then used to calculate a range of movement metrics in Eonfusion for further comparison of fish movement in relation to flow. For each transmitter a range of movement metrics were calculated: travel time, daily displacement, cumulative displacement, directional movement, activity, and location specific water flow. Displacement is a measure of geographical distance moved by an individual. It is represented as either daily displacement, the relocation distance in a single day or as cumulative displacement, the distance of a tagged individual from its release point. From here on the terms daily displacement and displacement (cumulative displacement) are used. Activity refers to the aggregate daily movement of a transmitter fish defined as the total combined up and downstream movement per day. Individual transmitter displacement and mean displacement for each species are examined in relation to flow for the study period. Similarly, mean activity and mean daily displacement are presented in relation to flow.

Recession of flows in the Wakool system leads to a series of remnant pools which provide refuge for the fish community. The study area is centralised around a large refuge pool fed by both the Yallakool Creek and Wakool River. Utilisation of this refuge habitat and the river reaches up and downstream by transmitter fish in relation to flow is examined. Daily visitation fish data were grouped into zones (upstream, downstream and refuge) and the proportion of the tagged population for each species is presented in relation to flow. Upstream habitat utilisation is further investigated with the relative proportion of tagged fish using the upper Wakool River and Yallakool Creek determined.

Since establishment of the array 8,922,086 detections have been recorded from the entire acoustic tagged fish population (n=195). For the purpose of this report, only transmitter fish tagged prior to August 2011 (plus 10 silver perch tagged August 2012) that were detected in the array from 1 August 2012 to 30 June 2013 (334 days) were used in the analysis. During this period 1,845,535 detections from 77 transmitter fish were recorded within the study area (Table 22). Detection data is unreliable from an analysis perspective because data can become dominated by fish that take up residence near a receiver. To overcome this, detection data were transformed to location and movement metrics based on time. All data subsequently presented in this chapter is based on daily detection data.

Separate tagging events in August 2010, August 2011, August 2012 and March 2013 resulted in the capture, tag and release of 162 fish: 60 Murray cod (Maccullochella peelii), 52 golden perch (Macquaria ambigua), 16 silver perch (Bidyanus bidyanus) and 34 alien common carp (Cyprinus carpio). All fish were collected from the remnant pool occurring below the junction of the Wakool River and Yallakool Creek.

Correlations between total river discharge and other factors (daily displacement, habitat use and upstream movement) were explored using the Spearman Rank Test function in Microsoft Excel. Analyses sought to determine whether specific aspects of fish behaviour were linked to hydrograph. A BACI design was not appropriate as there was limited spatial coverage in the array due to budgetary and logistical constraints with the array established in a zone most likely to hold native fish populations and receive water. Further statistical analysis was not conducted given the data presented in this report represents a period of one year.

Table 22. Summary of transmitter fish (n), mean length (± s.e.) (total length for Murray cod and golden perch; fork length for silver perch and carp) and weight (± s.e.) detected during the study period.

| Species | n | Length (mm) | Range (mm) | Weight (g) | Range (g) |
| --- | --- | --- | --- | --- | --- |
| Golden Perch | 19 | 421 ±14 | 330-536 | 1,447 (±158) | 643-2933 |
| Murray Cod | 25 | 546 ±22 | 400-900 | 2,759 (±478) | 689-11,732 |
| Silver Perch | 12 | 308 ±16 | 212-376 | 460 (±69) | 132-894 |
| Carp | 21 | 497 ±18 | 313-636 | 2,764 (±335) | 629-6,656 |

Figure 65. Top left; Murray cod having transmitter inserted. Top right; Surgery to insert tags. Bottom left; Chris Smith downloads an acoustic receiver at Yallakool Creek. Bottom right; A VR2W acoustic receiver rigged for deployment next to a V13-1x A69 acoustic tag.

Figure 66. Tagged species; clockwise from top left; Murray cod, golden perch, carp, silver perch.

Results and discussion

Murray Cod movement in response to environmental watering

Individual displacement for Murray cod over the study period ranged from 10 km downstream to 50 km upstream, with 18 km being the maximum displacement recorded by an individual in a single day. 68% of tagged Murray cod moved greater than 5 km, of which 95% undertook these movements from August to November 2012. 24% of tagged Murray cod moved distances greater than 10 km from the refuge pool. The general behaviour exhibited by tagged Murray cod involved periods of rapid upstream movement between August to November 2012 followed by a stationary period of approximately 1 to 4 weeks, followed by a return movement to the Wakool reserve pool. A long period (December 2012 to July 2013) of small localised movement, mostly within the refuge pool, followed this period (Figure 67, WMV file).

Figure 67. . Murray cod movement in the Edward – Wakool system. (Microsoft Word 2010). Time series movement of Murray cod is presented in relation to flow is available at this website. http://www.csu.edu.au/research/ilws/research/SRAs/Water/Water_projects%20-EdwardWakool.htm Discharge (ML/D) for the Wakool River and Yallakool Creek is shown on the left hand side of the video.

Overall displacement of Murray cod from the refuge pool was strongly correlated with the hydrograph (Spearman: R=0.9, P < 0.001, with an R2= 0.812) (Figure 68), with the average displacement of the tag population increasing upstream with flow (Figure 69). Upstream displacement occurred largely during higher flows but peak activity and displacement behaviour occurred from August to November 2012 and coincided with the increased discharge in both the Wakool River and Yallakool Creek.

Figure 68. Murray cod displacement correlated with flow.

Sixty percent of tagged Murray cod moved upstream of the refuge pool (Figure 70) in response to environmental watering between October and December 2012. The timing and duration of these movements are consistent with the behaviour of spawning Murray cod (Koehn & Harrington 2006) and may indicate fish used the freshes to take advantage of new habitat and actively seeking out suitable breeding partners or nesting habitat. Fish undertaking upstream movements during August to November 2012 predominantly moved into the Wakool River, with less than 5% of upstream moving individuals utilising Yallakool Creek (Figure 71). A reason for this preference for the Wakool River is unknown but does highlight the importance of maintaining habitat in this reach. By early January 2013, most Murray cod had returned to the tagging location and performed only localised movements within the pool at Wakool reserve. Environmental water delivery in Yallakool Creek from October to December 2012 was targeted at maintaining inundation of habitat for Murray cod nests prior to hatch. While tagged cod displayed a preference for the upstream Wakool River reach that did not receive a fresh, environmental water delivery in Yallakool Creek may have provided benefits to cod in Yallakool Creek, for example facilitating the return movements of fish to following spawning and nesting.

Murray cod movement was also monitored during the environmental watering in Yallakool Creek and Colligen Creek between February and April 2013. Murray cod exhibited increased activity during these freshes but all tagged fish remained within the refuge pool (Figure 70). Even though no displacement occurred at this time, the increased activity of fish during these flows may indicate other behavioural responses, such as are occurring within the refuge habitat during the environmental watering, increased feeding or access to newly inundated habitat.

Distance (km)

Flow (ML/day)

Proportion of tagged

Fish (%)

Flow (ML/day)

Refuge pool

Upstream

Downstream

Figure 69. Displacement (top) and daily displacement (bottom) (left axis, black) in km for Murray cod, in relation to activity (left axis, grey) and flow (right axis) (Yallakool Creek flow -Blue, Wakool River –red). Environmental water periods shown with the blue shading.

Proportion of tagged

Fish (%)

Flow (ML/day)

Yallakool

Wakool

Figure 70: Daily habitat selection of Murray cod shown as the tagged proportion upstream, downstream and in the refuge pool (left axis)  – in relation to the flow (right axis) (Yallakool Creek flow -Blue, Wakool River –red). Environmental water delivery is highlighted by blue bars.

Figure 71. Upstream habitat selection by Murray cod (left axis) in relation to flow (right axis), (Yallakool Creek flow -Blue, Wakool River red). Environmental water periods shown with the blue boxes.

Golden Perch movement in response to environmental watering

Golden perch exhibited a displacement range of 150 km downstream to 10 km upstream, with a maximum daily displacement of 38 km recorded by an individual. Peak displacement for golden perch during the study period occurred from mid-September to mid-October 2012 (Figure 72). Displacement occurred on larger flows with overall displacement modestly correlated with flow (Spearman: R=0.5, P < 0.001, with an R2=0.8) (Figure 73). Movements during this time are known to coincide with spawning behaviour in the nearby Barmah-Millewa forest (King et al. 2009).

Figure 72. Golden perch movement in the Edward – Wakool system. Time series movement of golden perch is presented in relation to flow is available at this website. http://www.csu.edu.au/research/ilws/research/SRAs/Water/Water_projects%20-EdwardWakool.htm Discharge (ML/D) for the Wakool River and Yallakool Creek is shown on the left hand side of the video.

Figure 73. Displacement of tagged golden perch correlated with flow.

During higher flows, golden perch movements were mostly downstream. Disparity between the upstream and downstream movement is supported by long term movement data for golden perch in the Wakool study area. Overall population displacement has varied from predominantly upstream to downstream in years following a hypoxic blackwater event (unpublished data). The results demonstrate that golden perch are able to travel large distances over relatively short timeframes and were able to make use of new habitat when inundated.

Between August and October 2012 all tagged individuals had an observed displacement of greater than 5 km per day and many of these movements were greater than 10 km (37% of the tag population) during the same period. Large scale displacement (> 10km) was observed more frequently over multiple days but most of these movements occurring between August and October (Figure 74). Between October 2012 and July 2013 overall displacement decreased substantially but individuals that made large scale movements (40 – 150 km) eventually returned to the capture location.

Environmental water delivered to Yallakool Creek (19 0ctober 2012 to 7 December 2012) did not result in large scale displacement for the golden perch tag population as a whole, however some individuals made displacement movements of 5 to 15 km. Subsequent deliveries of environmental water (Yallakool Creek 2 February 2013 to 22 February 2013, 13 March 2013 to 5 April 2013) resulted in minimal increases in activity by golden perch, and displacement movements were typically less than 3 km.

From August 2012 to early January 2013 up to 45% of tagged golden perch were recorded outside the refuge pool (Figure 75). The preference for downstream movement only occurred between August 2012 and January 2013. Fish leaving the refuge pool between the months of February and June 2013 chose to move only upstream. For fish moving upstream, a clear preference was shown for the Wakool River during August 2012 to January 2013, but fish moved into the Yallakool Creek from February to June 2013 (Figure 76). These upstream movements by golden perch into the Yallakool were strongly correlated (Spearman; R=0.8, P < 0.001, with an R2=0.5171) with flow in the creek, including the environmental watering (Figure 77). This activity and other small scale movements may indicate that golden perch are utilising these increases in flow to feed both within and outside the refuge pool.

Distance (km)

Flow (ML/day)

F

Proportion of tagged

Fish (%)

igure 74. Displacement and daily displacement (left axis, black) in km of golden perch in relation to activity (left axis, grey) and flow (right axis). Environmental water periods shown with the blue shading.

Flow (ML/day)

Refuge pool

Upstream

Downstream

Flow (ML/day)

Proportion of tagged

Fish (%)

Yallakool

Wakool

Figure 75. Daily habitat selection of Golden perch shown as the tagged proportion upstream, downstream and in the refuge pool (left axis)  – in relation to the flow (right axis) (Yallakool Creek flow -Blue, Wakool River –red). Environmental water delivery is highlighted by blue bars.

Figure 76. Proportion of tagged golden perch (left axis) upstream of the refuge pool in the Wakool (green) and Yallakool (blue) in relation to the flow (right axis). Yallakool environmental water delivery shown by the blue bars.

Movement data indicated golden perch return to the refuge pool after periods of displacement, spending 80% of their time in this habitat. This display of strong site fidelity is consistent with other studies (Crook 2004; O’Connor et al. 2005), similarly the opportunistic use of available habitat observed in this study by golden perch was also observed by Koster et al. (2012). During extended drought, environmental water delivery should be used for the maintenance of viable golden perch source populations throughout the Wakool. The strong preference for returning to particular refugia should be considered vital for continued survival of golden perch.  Environmental flow delivery should therefore be provided at the appropriate time to maintain drought refugia and encourage movement outside the refuge pool for emigration, feeding and recruitment.

Figure 77. Proportion of upstream displacing tagged golden perch correlated with Yallakool creek flow.

Silver perch movement in response to environmental watering

Silver perch were highly mobile, undertaking frequent short distance (<2 km) return movements throughout the study (Figure 78 and wmv file). Three individuals made large scale (>30 km) displacing movements, ranging from 72 km downstream to 66 km upstream. 42 km was the maximum daily movement recorded by an individual. During the study period individuals amassed movements of more than 1100 km (mean 290 km). Displacement movements greater than 5 km were exhibited by 73% of tagged silver perch, however in contrast to data from 2010 to 2012 (unpublished) only a small percentage (27%) of tagged fish recorded displacements greater than 10 km from the refuge. The targeted silver perch tagging event on 2 October 2012 to boost low number of silver perch remaining in the study area may explain this difference. Few individuals remained in the study area prior to this tagging event.

Movement during environmental watering actions was evident with several silver perch making displacement movements during delivery; however this pattern of movement was not restricted to these flow events with other displacement movements occurring through the year (Figure 79).

Sliver perch were detected mostly outside the refuge pool (63% of all tagged fish) displaying a preference for upstream habitat found in this zone 56% of the time (Figure 80). Silver perch that moved upstream occupied areas where flow velocity was higher in the Yallakool Creek (Figure 81). By June 2013, on cessation of flows in both the Yallakool Creek and Wakool River all fish returned to the refuge pool, demonstrating the importance of this habitat in sustaining native fish in the region.

Silver perch are thought to have a strong flooding requirement for successful spawning (Humphries et al. 1999) but more recent work has shown they are also able to spawn within channel (King et al. 2009; Mallen-Cooper and Stuart 2006). However, little is known about their movement behaviour and the role it plays in the survival of the species, and a synthesis of the last three years data with linkages to other ecosystem components (e.g. recruitment) will help with this.

Figure 78. Silver perch movement in the Edward – Wakool system. Time series movement of silver perch is presented in relation to flow is available at this website. http://www.csu.edu.au/research/ilws/research/SRAs/Water/Water_projects%20-EdwardWakool.htm The discharge (ML/D) for the Wakool River and Yallakool Creek is shown on the left hand side of the video.

Distance (km)

Flow (ML/day)

Refuge pool

Upstream

Downstream

Flow (ML/day)

Proportion of tagged

Fish (%)

Figure 79. Displacement and daily displacement (left axis, black) in km of silver perch in relation to activity (left axis, grey) and flow (right axis). Environmental water periods shown with the blue shading.

Flow (ML/day)

Proportion of tagged

Fish (%)

Yallakool

Wakool

Figure 80. Daily habitat selection of Silver perch shown as the tagged proportion upstream, downstream and in the refuge pool (left axis)  – in relation to the flow (right axis) (Yallakool Creek flow -Blue, Wakool River –red). Environmental water delivery is highlighted by blue bars.

Figure 81. Proportion of tagged silver perch (left axis) upstream of the refuge pool in the Wakool (green) and Yallakool (blue) in relation to the flow (right axis). Yallakool environmental water delivery shown by the blue bars.

Carp movement in response to environmental watering

Tagged carp displayed periods of large displacing movements interspersed with periods of sedentary behaviour (Figure 82 and wmv file). Peak displacement for carp occurred between August and November 2012 coinciding with high flow events; however a small number of individual fish undertaking large scale movements outside of this period resulted in a high mean displacement from January to May. Individual displacements away from the refuge pool in carp ranged between 150 km downstream to 119 km upstream, with a 32 km maximum daily displacement, but these displacements were only exhibited by less than 20% of the overall population. Distances greater than 5 km from the refuge pool were recorded in 76% of tagged carp during the study, with all of these fish undertaking these movements during August to November. Similarly, 67% were recorded at distances greater than 10 km from the refuge pool, with 70% of these movements occurring during the August to November period.

Movements greater than 5 km in a single day were rare, occurring on 1% of the detection days. Similar results were reported in studies by Reynolds (1983) and Stuart and Jones (2006) utilising external tagging data to monitor carp movements but were determined utilising mark recapture techniques which do not account for movement of animals between capture events. The acoustic techniques used in this study provide continuous monitoring without the need to impact behaviour through repeated capture and sampling location bias.

Monitored carp displayed high site fidelity, residing in close proximity to their point of capture or choosing to return to it after short periods of displacement. With the exception of a small number of individuals undertaking large scale movements, carp typically made movements of less than 5 km, spending 64% of their time in the refuge pool (Figure 83). Carp leaving the refuge pool moved in both upstream and downstream directions (Figure 84).  Movement during environmental water releases was evident with an increased proportion of carp moving upstream out of the refuge pool into the Yallakool Creek during environmental flow delivery (Figure 85).

Carp recruitment in Australia occurs from mid spring to summer, however may occur as late as autumn and early winter at temperatures between 16 to 32 °C and is influenced by flow regulation (Driver et al. 2005). Manipulation of flow delivery could be utilised in the system to control carp numbers, for example flow manipulation to desiccate eggs, deny access to spawning habitat, or prevent carp leaving areas where they occur in high abundances in the system (e.g. wetland habitats identified during community sampling).

Stuart et al. (2006) reported on the opportunity to trap migrating carp at regulatory structures using carp separation cages installed in fishways. In the Edward Wakool system, fishways currently operate on Colligen Creek Regulator, Yallakool Creek offtake, Edward River offtake, Gulpa Creek offtake and Steven’s Weir providing opportunity for utilisation of this technology to control carp, in collaboration with environmental water delivery. The most effective timing, magnitude, duration of flows for trapping at regulatory structures could be implemented utilising the movement data collected in this study to maximise resources. However, the results indicate that such control techniques may only target the highly mobile carp and the local population inhabiting the immediate downstream area (5 to 10 km) of the structure. Nevertheless, understanding the movement strategy of carp throughout the system is a critical step in controlling future dispersal on environmental water allocations. Any opportunities to limit dispersal should be explored and included into annual water planning processes.

Figure 82. Common carp movement in the Edward – Wakool system. Time series movement of common carp is presented in relation to flow in a video available at this website. http://www.csu.edu.au/research/ilws/research/SRAs/Water/Water_projects%20-EdwardWakool.htm. The discharge (ML/D) for the Wakool River and Yallakool Creek is shown on the left hand side of the video.

Distance (km)

Flow (ML/day)

Refuge pool

Upstream

Downstream

Flow (ML/day)

Proportion of tagged

Fish (%)

Figure 83. Displacement and  daily displacement (left axis, black) in km for carp, in relation to activity (left axis, grey) and flow (right axis). Environmental water periods shown with the blue shading.

Flow (ML/day)

Proportion of tagged

Fish (%)

Yallakool

Wakool

Figure 84. Daily habitat selection of carp shown as the tagged proportion upstream, downstream and in the refuge pool (left axis) in relation to the flow (right axis) (Yallakool Creek flow -Blue, Wakool River –red). Environmental water delivery is highlighted by blue bars.

Figure 85. Proportion of tagged carp (left axis) upstream of the refuge pool in the Wakool (green) and Yallakool (blue) in relation to the flow (right axis). Yallakool environmental water delivery shown by the blue bars.

General trends in fish movement

All four monitored fish species displayed increased activity and displacement in response to increasing flow during spring and early summer. This period of increased movement corresponded with different spawning periods for many of these fish. Once fish moved away from the tagging location, there was a period of absence, but the vast majority of fish returned to the Wakool reserve pool. These observations suggest two important management issues. Firstly, the Wakool reserve is an important refuge pool in the region and is used by all fish species during periods of low flow. When flow declines in Yallakool creek and the Wakool River, available fish habitat declines, leaving small shallow pools with varying connectivity, with the Wakool reserve forming the largest in both size and depth and may drive much of the site fidelity observed. Secondly, the flow regimes delivered this watering year were sufficient for all tagged fish species to leave the pool and access new habitat, and also to subsequently return when flows ceased. Movement however, differed from previous data in both displacement distance, and the number of fish undertaking these movements was reduced corresponding with reduced flows.

Delivery of environmental water encouraged all species, except Murray cod, to move into Yallakool Creek. Murray cod moved during spring and early summer predominantly into the Wakool River, but this could have been related to habitat differences in the two systems as opposed to water delivery. This pattern of movement was consistent with spawning related behaviour, with cod moving upstream prior to a sedentary period, suggestive of nesting, before returning down river to the refuge pool.

Golden perch made movements consistent with that of spawning behaviour on the unregulated events during September 2012 but no recruitment was detected in community sampling (possibly due to inadequate temperature cues). During environmental watering events, even though temperatures were adequate, movements were not consistent with that of a spawning related behaviour, and this could have been due to inadequate magnitude and duration. Additional work is required to identify the most appropriate flows needed by golden and silver perch.

Silver perch movement did not appear to be spawning related, but the high activity indicates that longitudinal connectivity is important for this species, and that relatively high velocity areas may also be preferred. Environmental water delivery should be focused for this species on maintaining refuge habitat (as with all other native species mentioned) during drought periods, and further testing of relationships with flow through flow manipulation studies.

Carp movement within 2013 did not appear to be directly related to increases in flow or seasonally related to spawning behaviour, even though large unregulated flow events would have inundated spawning habitat such as wetland habitat. Movement during environmental watering did not increase markedly or appear to be spawning related. This movement data suggests that instream flows similar to that delivered to promote Murray cod spawning or flow generalist spawning (CEWO 2012) does not benefit carp recruitment, and even inundation of wetland habitats (unregulated flows) does not always elicit a spawning response. Hence, a positive outcome is that a targeted flow delivery for native fish species did not appear to benefit carp dispersal.

Objective 5: Support breeding and recruitment of native fish

Fish spawning (abundance of larval fish and eggs)

Key findings

- Larvae of ten species of fish, of which 8 were native, were found in the Edward-Wakool system over the 2012-2013 spawning season. Larvae of only one species, carp gudgeon, were found in significantly higher numbers immediately after an environmental fresh when compared to rivers that did not receive environmental water.

- There was no evidence of golden and silver perch spawning as a result of the environmental watering freshes delivered to Colligen Creek in November 2012. No eggs or larvae of silver or golden perch were found during or immediately after the watering actions. Although there were fish moving during all three environmental watering events it is not known if any of these movements resulted in spawning because most adult golden perch migrated downstream outside the larval monitoring zone. Future monitoring should assess spawning in downstream sites.

- There was no evidence that the nursery and larval dispersal conditions of Murray cod were enhanced as a result of the October to December 2012 Yallakool Creek watering action. Although Murray cod larvae were found in the Yallakool Creek, they were not in significantly greater numbers than in the rivers that did not receive environmental water. The spawning and drifting of Murray Cod occurred independently of the environmental watering.

Background

The distribution and abundance of Murray-Darling Basin native fish populations have undergone considerable declines since European settlement, and current populations are estimated to constitute just 10% of that which may have occurred historically (Lintermans 2007). One of the major causes of this decline is river regulation, which has reduced the frequency and magnitude of flow extremes and in many regions has altered the seasonality of river flows (Cadwallader 1978). The ecological requirements of freshwater fish are considered to be closely linked to flow, with changes in hydraulic conditions having considerable effect on pre-spawning condition and movement, spawning and subsequent recruitment, as well as the provision of food and habitat resources (Humphries and Lake 2003). To mitigate some of the effects of regulation on the ecology and health of Murray-Darling rivers, the release of environmental flows is becoming a more commonly applied restoration tool, and in the context improving fish populations, is seen as a potential way of enhancing the spawning and recruitment of native fish species (Murray-Darling Basin Commission 2004).

Murray-Darling fish appear to exhibit a diversity of flow requirements, although for many species, knowledge of specific flow requirements for many species is poor (Humphries et al. 1999; Pusey et al. 2004). Humphries et al. (1999) proposed three broad groups for native fish based on their life history, and likely flow requirements for spawning and recruitment. ‘Mode 1’ fish, are characterised as long-lived, large-bodied fish species whose spawning, or magnitude of spawning is associated with flow pulses, such as Golden and Silver Perch; ‘Mode 2’ fish, are characterised as long-lived, larged-bodied fish species whose spawning is independent of flow conditions, but whose recruitment may benefit from flow events (e.g Murray cod, trout cod, riverblackfish); and ‘Mode 3’ fish are short-lived, small-bodied fish that also spawn independently of flow pulses, and may well flourish under low flow conditions due to the warm temperatures and higher food resources that such environments can provide (e.g carp gudgeon, Australian smelt, unspecked hardyhead, flathead gudgeon) (Humphries et al. 1999). Flow-response studies of native fish spawning and recruitment that have been conducted since the modes proposed by Humphries et al. (1999), and in particular those that have focussed on assessing both overbank and large within channel environmental flows, give support to the life history modes proposed, in particular confirming the importance of flow pulses for golden and silver perch spawning and recruitment (Mallen-Cooper and Stuart 2003; Roberts et al. 2008; Zampatti and Leigh 2013), and the independence of species such as Murray cod spawning from flow conditions. Less has been documented on the role of flow in the spawning and recruitment for smaller bodied species however (but see King et al. 2003; Tonkin et al. 2008a; Tonkin et al. 2011).

One of the objectives of the 2012-2013 watering actions in the Edward-Wakool system was to provide spawning outcomes for native fish. For many rivers in the Murray-Darling Basin, including those in the Edward-Wakool system, constraints to water delivery limit the extent to which environmental watering actions can mimic magnitude, timing and duration of the natural flood regime. While large in-channel pulses have been found to promote recruitment in golden perch, silver perch and Murray Cod, less is known of the effect of smaller in-channel pulses on the spawning and recruitment of native fish. Monitoring of the abundance and diversity of larval fish was undertaken through spring and summer of 2012-2013 to detect which species of the Edward-Wakool fish community had spawned, and to assess the effect of small in-channel pulses on spawning (this section) and recruitment (section 7.5.2).

Hypotheses

- The spawning of some native fish species, as measured by abundance of larvae, will increase either during or following environmental water delivery, compared to nearby rivers not receiving environmental water.

- Production of larvae will be significantly greater in the rivers receiving environmenental freshes compared to those that do not.

Methods

Eggs, larvae (Figure 86) and juvenile fish were sampled to evaluate the spawning response of fish to specific flow events. Sampling, which involved a combination of quatrefoil light traps and boat trawls, was undertaken fortnightly across five rivers in the Edward-Wakool system from August 2012 to April 2013. Colligen Creek and Yallakool Creek received environmental freshes, the Wakool River and Little Merran Creek did not receive environmental freshes (controls)  and the Edward River was included as it was the source of the environmental water delivered to Colligen Creek and Yallakool Creek.

To sample for larval and juvenile fish, three quatrefoil perspex light traps containing bioluminescent light sticks (Figure 87) were set at five sites within each river (15 traps in total per river). Light traps were deployed randomly along the littoral edge at each site at dusk, and retrieved the following morning (7:00-9:00 am).  Five by 5 minute boat trawls were undertaken in each river, at one site (Figure 88). The boat trawls, which targeted pelagic eggs, larvae and juveniles entrained in the water column, involved deploying a 500 µm conical drift net (0.5 m mouth) on one side of the boat, and slowly zigzagging across the river in an upstream direction. The amount of water filtered by each trawl was recorded by a flow meter placed within the mouth of the net.

Additional targeted drift net sampling was undertaken in December 2012 and February 2013 to assess if golden perch and silver perch spawned in response to the in-channel freshes delivered to Colligen Creek and Yallakool Creek by the Commonwealth Water holder (Watering actions 2 &3, see section 3). The catch of larvae and eggs caught in Colligen Creek was compared to two rivers (Wakool River and Yallakool Creek) that did not receive environmental water, over three consecutive nights during the peak of the freshes (12-14 December 2012 and 5-7 February 2013). At each river, three conical drift nets were set in the evening (1 hour after last light), and retrieved first thing in the morning. The amount of water filtered by each trawl was recorded by a flow meter placed within the mouth of the net.

Larvae and juveniles collected from the light traps, boat trawls and drift nets were preserved in 90% ethanol and returned to the laboratory for processing. All individuals were identified, measured (SL mm), and their developmental stage recorded according to Serafini and Humphries (2004). Here, we classify larvae within five key developmental stages; protolarvae (with and without yolksac), flexion, postflexion, metalarvae and juvenile/adults. Otoliths were extracted from a subset of Murray cod and carp gudgeon larvae to determine growth rates and hatch dates (section 7.5.2).

The number of fish/eggs collected in drift nets and boat trawls were standardised to the amount of water filtered by the net. Due to the patchy nature of larval fish aggregation, larvae collected in the three light traps at each river site were pooled to make up one larger composite sample, giving five replicate samples per river, per trip. Response variables were log-transformed prior to statistical analyses when necessary to normalise data and stabilize variances. To test if the production of larvae (total abundance) was significantly different across the 5 rivers across the entire spawning season, total larval abundance was analysed for all species (where there was enough data) using a one way ANOVA with river as the grouping variable. When significant differences were indicated, post hoc pairwise comparisons were undertaken to determine differences between the Rivers. Where log-transformations failed to normalize species abundance data, a generalized linear model with a poisson distribution was used to test the effect of river instead.

An asymmetrical BACI (before-after, control-impact) (Underwood 1991) statistical design was used to test the effect of specific 2012-2013 environmental water actions on larval abundance in the Edward-Wakool river system. Differences in mean number of larvae between control/impact rivers and before/during/after environmental freshes were evaluated statistically for each watering action using two-way mixed effects analysis of variance (ANOVA). Because there were multiple sampling times used to represent before, during and after environmental flows, and multiple rivers used as ‘Control’ and sometimes ‘Impact’ rivers, sampling trip (random effect) was nested within Period (fixed effect, three levels: before, during and after), and river (random effect) was nested in Treatment (fixed effect, two levels: control rivers, impact rivers). Impact rivers received environmental freshes, while Control rivers were those that did not receive environmental water. For this analysis particular interest is in the Period x Treatment interaction term, which indicates a significant effect of the environmental watering action. Visual assessment of mean (±1SE) biomass plots, grouped by Period and Treatment, were used to confirm if the significant interaction term was positively or negatively associated with the environmental watering action.

The dates and rivers used to detect changes in fish larvae is consistent with the approach used to assess water chemistry (Table 9, section 7.2.1). The null hypothesis for all watering actions was that mean larval abundance of native fish species in the rivers which received environmental water would not be significantly different to the control rivers. All statistical analyses were carried out using the freeware R and the R package NLME (R Development Core Team 2013).

Figure 86. Murray cod larva at six days old. The white bar represents 1 mm. From Serafini & Humphries (2004). Preliminary guide to the identification of larvae of fish from the Murray-Darling Basin

Figure 87. Photos of light traps used to sample larval fish, showing bioluminescent light stick in the centre of the trap

Figure 88. Drift net used to undertake boat trawls for larval fish. This sampling method is undertaken after dark.

Results and discussion

Species abundance and diversity and timing of spawning

A total of 33,044 larval and juvenile fish, representing 11 fish species, were collected in the 2012-2013 monitoring study (Table 23). Of these, 6,203 were larvae, and 26,841 were juveniles/adults.

Eight of the 11 fish species collected as larvae/juveniles were native species. Small-bodied species made up the majority of larvae collected across the five rivers, and were represented by (in order of most to least abundant), Australian smelt (Retropina semoni), carp gudgeon (Hypseleotris spp.), flathead gudgeon (Philypnodon grandiceps), unspecked hardyhead (Craterocephalus stercusmascarum fulvus) and Murray River rainbowfish (Melanotaenia fluviatilis) (Table 23).

Large-bodied species that were found as larvae were Murray cod (Maccullochella peelii), river blackfish (Gadopsis marmoratus), silver perch (Bidyanus bidyanus) and carp (Cyprinus carpio). Of these, Murray cod and carp were the most abundant, with 216 and 193 larvae sampled respectively (Table 23). Quatrefoil light traps, boat-trawls and drift nets caught 98.2% (n=6,088), 1.1% (n=68) and 0.7% (n=47) of the total larval catch, respectively. Because of low catch rates of larvae in drift nets and boat trawls, statistical analyses were limited to light trap catch data.

Table 23. Catch summary of fish a) larvae and b) juveniles/adults collected with light traps, boat trawls and drift nets from the five rivers in the Edward-Wakool system during 2012-2013.

|  | Light traps |  |  |  |  |  |  | Boat trawls |  |  |  |  |  |  | Drift nets |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a) Larvae | Col. | Yal. | Wak. | Mer. | Edw. | Total |  | Col. | Yal. | Wak. | Mer. | Edw. | Total |  | Col. | Yal. | Wak. | Total |
| Australian smelt | 83 | 130 | 123 | 1916 | 1653 | 3905 |  | 1 | 1 | 0 | 10 | 5 | 17 |  | 0 | 0 | 0 | 0 |
| carp gudgeon | 469 | 77 | 19 | 112 | 916 | 1593 |  | 12 | 0 | 2 | 8 | 21 | 43 |  | 37 | 1 | 5 | 43 |
| Murray cod | 5 | 97 | 69 | 17 | 28 | 216 |  | 1 | 1 | 0 | 2 | 1 | 5 |  | 0 | 2 | 2 | 4 |
| carp | 0 | 1 | 85 | 98 | 9 | 193 |  | 0 | 0 | 1 | 1 | 0 | 2 |  | 0 | 0 | 0 | 0 |
| unidentified | 10 | 0 | 1 | 11 | 65 | 87 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| flathead gudgeon | 22 | 1 | 4 | 47 | 11 | 85 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| Murray River rainbowfish | 3 | 1 | 0 | 0 | 0 | 4 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| river blackfish | 0 | 0 | 3 | 0 | 0 | 3 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| silver perch | 0 | 0 | 0 | 2 | 0 | 2 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| gambusia | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| unspecked hardyhead | 0 | 0 | 0 | 0 | 0 | 0 |  | 1 | 0 | 0 | 0 | 0 | 1 |  | 0 | 0 | 0 | 0 |
| oriental weatherloach | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| Total | 592 | 307 | 304 | 2203 | 2682 | 6088 |  | 15 | 2 | 3 | 21 | 27 | 68 |  | 37 | 3 | 7 | 47 |
| % | 10 | 5 | 5 | 36 | 44 |  |  | 22 | 3 | 4 | 31 | 40 |  |  | 79 | 6 | 15 |  |
| b) Juvenile/adults | Col. | Yal. | Wak. | Mer. | Edw. | Total |  | Col. | Yal. | Wak. | Mer. | Edw. | Total |  | Col. | Yal. | Wak. | Total |
| Australian smelt | 230 | 104 | 91 | 814 | 259 | 1498 |  | 847 | 201 | 55 | 238 | 293 | 1634 |  | 0 | 2 | 1 | 3 |
| carp gudgeon | 6251 | 2465 | 88 | 813 | 13414 | 23031 |  | 119 | 19 | 1 | 4 | 21 | 164 |  | 237 | 112 | 1 | 350 |
| Murray cod | 0 | 0 | 0 | 1 | 0 | 1 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| carp | 0 | 0 | 2 | 4 | 0 | 6 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| unidentified | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| flathead gudgeon | 93 | 4 | 1 | 1 | 37 | 136 |  | 2 | 0 | 2 | 0 | 1 | 5 |  | 0 | 0 | 2 | 2 |
| Murray River rainbowfish | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 1 | 0 | 0 | 1 |  | 0 | 0 | 0 | 0 |
| river blackfish | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| silver perch | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| gambusia | 0 | 0 | 0 | 1 | 0 | 1 |  | 0 | 0 | 0 | 0 | 1 | 1 |  | 0 | 0 | 0 | 0 |
| unspecked hardyhead | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 0 | 2 | 0 | 0 | 2 |  | 0 | 0 | 0 | 0 |
| oriental weatherloach | 0 | 0 | 0 | 0 | 0 | 0 |  | 0 | 1 | 1 | 0 | 0 | 2 |  | 0 | 0 | 0 | 0 |
| Total | 6574 | 2573 | 182 | 1634 | 13710 | 24673 |  | 968 | 221 | 62 | 242 | 316 | 1809 |  | 237 | 114 | 8 | 359 |
| % | 27 | 10 | 1 | 7 | 56 |  |  | 54 | 12 | 3 | 13 | 17 |  |  | 66 | 32 | 2 |  |

The contribution of the different developmental stages which made up the larval catch varied across fish species (Figure 89). The majority of Australian smelt collected were proto-larvae (the earliest developmental stage) while carp gudgeon were predominately meta-larvae (the most developed larval stage).  Murray cod were only collected as meta-larvae as they are well developed at hatch, and do not have any earlier stages. For the remaining, less common species, relatively similar proportions of proto-larvae, flexion, post-flexion and meta-larvae were sampled.

Figure 89. Total abundance of the different developmental stages of larvae caught for each fish species. n=the number of larvae collected across entire the sampling season. Note y-axis is on a logarithmic scale.

Seasonal timing in appearance of larval fish

On average, five to six species were found as larvae in each of the rivers (Figure 90). The duration and timing of the spawning period for small-bodied species such as Australian smelt, carp gudgeon and flathead gudgeon was variable, and differed across rivers (Figure 90). Australian smelt were found earliest in the spawning season, and were collected as larvae from September to December 2012 in all rivers. The other abundant species (carp gudgeon, Murray cod, flathead gudgeon and carp) peaked around October to December 2012 (Figure 91). Carp gudgeon larvae were found in highest numbers in Colligen Creek and the Edward River, appearing from November 2012 to April 2013 (Figure 91). Carp gudgeon were less abundant in Yallakool Creek, the Wakool River and Little Merran Creek, and the spawning season in these rivers was shorter compared to Colligen Creek (Figure 90 and 91).

Figure 90. Occurrence (blue bars) of the fish larvae sampled in Colligen Creek, Yallakool Creek, Wakool River and Little Merran Creek from August 2012 to April 2013, along with discharge (solid line) and temperature (dashed line) profiles.

Figure 91. Temporal trends in mean larval abundance (±1SE) of the five most common fish species in the Edward-Wakool system over the 2012-2013 spawning season. Coloured bars represent when Commonwealth environmental water was delivered to Colligen Creek (green bars), and Yallakool Creek (blue bars). The Wakool River and Little Merran Creek did not receive environmental water and the Edward River was the source of the environmental water. Note: Y axes vary.

Environmental watering event-based analysis

November 2012 Colligen Creek watering action targeting golden and silver perch spawning

We found no evidence to indicate that golden or silver perch spawned in response to the two freshes delivered to Colligen Creek in November 2012. No golden perch eggs, larvae or juveniles were found in drift nets, boat trawls or light traps. Two silver perch larvae were collected in Little Merran Creek in January 2013, indicating that some level of spawning had occurred independently of the environmental flows delivered to Colligen Creek.

Of the five most abundant species, carp gudgeon was the only species that were detected to significantly increase in abundance after the first environmental fresh compared with rivers that did not any environmental water at this time (d.f= 2, 54, F-test=5.658, p=0.0059) (Figure 92). Abundances of Australian smelt, carp, flathead gudgeon and Murray cod did not significantly increase during or after the first fresh (Figure 92, Table 24). During the second fresh which followed shortly after, carp gudgeon mean abundances increased during the fresh, however site-level variation in abundances meant these observations were not significant (Figure 93, Table 24). Flathead gudgeon larval abundances, which were unaffected by the first environmental fresh, also showed no response to the second fresh (Figure 93, Table 24). Statistical analyses could not be performed for the other fish species due to the low numbers sampled.

November 2012 Yallakool Creek water action targeting Murray cod

Environmental water delivered to Yallakool Creek in November 2012 coincided with the spawning period for Murray cod. Murray cod larvae were found in all 5 rivers, and appeared in the Edward-Wakool system from mid-October through to mid December 2012 (Figure 91). Abundances were greatest in Yallakool Creek and the Wakool River and were in lower abundance in Little Merran Creek, Edward River and Colligen Creek. Numbers of larvae were not significantly greater in Yallakool Creek compared with rivers that did not receive environmental flows (Wakool River and Little Merran Creek) (d.f=2,99, F-test=1.423, p=0.245, Table 24). The spawning and drifting of Murray Cod occurred independently of the environmental watering (d.f=2, 99, F-test=1.42, p>0.05) (Figure 94). There was no significant change in Australian smelt, carp gudgeon, flathead gudgeon or carp abundances in Yallakool Creek during or after the environmental watering (Figure 94, Table 24).

Figure 92.  Mean abundance (±1SE) of fish larvae present in the Edward-Wakool system before, during and after the first environmental fresh in Colligen Creek on 2 November 2012. This flow was aimed at eliciting a golden and silver perch spawning response. The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls. Planned comparisons with significant interactions between control-impact rivers (impact river; Colligen Creek, control rivers; Wakool River, Little Merran Creek) and Period (before, during, after) are marked with an asterisk, indicating that mean larval abundance changed significantly in Colligen Creek either during or after the environmental watering.

Table 24. Statistical results for 2 way mixed-effects Analaysis of Variance (ANOVA). A significant interaction between the two fixed factors: Period (before, during, after) and CI (control rivers, impact rivers) indicates that the mean abundance of larval fish within Impact Rivers was significantly different to changes that occurred over the same period of time within the Control Rivers.

| Environmental flow | Species | Main effect | d.f | F-test | p-value |
| --- | --- | --- | --- | --- | --- |
| Nov 2012 – Colligen Creek fresh #1 |  |  |  |  |  |
|  | carp gudgeon | Period (B-D-A) | 2,54 | 1.440 | 0.245 |
|  |  | CI (C-I) | 1,54 | 0.272 | 0.603 |
|  |  | Period*CI | 2,54 | 5.658 | 0.006 |
|  | Australian smelt | Period (B-D-A) | 2,54 | 2.117 | 0.130 |
|  |  | CI (C-I) | 1,54 | 0.322 | 0.572 |
|  |  | Period*CI | 2,54 | 0.333 | 0.717 |
|  | Murray cod | Period (B-D-A) | 2,54 | 17.92 | <0.0001 |
|  |  | CI (C-I) | 1,54 | 0.916 | 0.342 |
|  |  | Period*CI | 2,54 | 0.295 | 0.745 |
|  | carp | Period (B-D-A) | 2,54 | 32.031 | <0.0001 |
|  |  | CI (C-I) | 1,54 | 15.924 | <0.0001 |
|  |  | Period*CI | 2,54 | 15.774 | <0.0001 |
|  | flathead gudgeon | Period (B-D-A) | 2,54 | 1.385 | 0.259 |
|  |  | CI (C-I) | 1,54 | 0.034 | 0.853 |
|  |  | Period*CI | 2,54 | 0.579 | 0.563 |
| Nov 2012 – Colligen Creek fresh #2 |  |  |  |  |  |
|  | carp gudgeon | Period (B-D-A) | 2,38 | 1.827 | 0.174 |
|  |  | CI (C-I) | 1,38 | 16.021 | 0.155 |
|  |  | Period*CI | 2,38 | 0.0001 | 0.998 |
|  | flathead gudgeon | Period (B-D-A) | 2,38 | 4.921 | 0.012 |
|  |  | CI (C-I) | 1,38 | 0.112 | 0.786 |
|  |  | Period*CI | 2,38 | 0.480 | 0.622 |
| Nov 2012 – Yallakool Creek fresh |  |  |  |  |  |
|  | carp gudgeon | Period (B-D-A) | 2,99 | 4.216 | 0.017 |
|  |  | CI (C-I) | 1,99 | 0.126 | 0.723 |
|  |  | Period*CI | 2,99 | 0.774 | 0.477 |
|  | Australian smelt | Period (B-D-A) | 2,99 | 8.143 | <0.001 |
|  |  | CI (C-I) | 1,99 | 0.211 | 0.646 |
|  |  | Period*CI | 2,99 | 3.365 | 0.03 |
|  | Murray cod | Period (B-D-A) | 2,99 | 2.403 | 0.095 |
|  |  | CI (C-I) | 1,99 | 1.661 | 0.200 |
|  |  | Period*CI | 2,99 | 1.423 | 0.245 |
|  | carp | Period (B-D-A) | 2,99 | 2.581 | 0.080 |
|  |  | CI (C-I) | 1,99 | 5.100 | 0.026 |
|  |  | Period*CI | 2,99 | 5.183 | 0.007 |
|  | flathead gudgeon | Period (B-D-A) | 2,99 | 0.732 | 0.483 |
|  |  | CI (C-I) | 1,99 | 0.576 | 0.449 |
|  |  | Period*CI | 2,99 | 0.336 | 0.69 |
| Feb 2013 – Yallakool Creek fresh |  |  |  |  |  |
|  | carp gudgeon | Period (B-D-A) | 2,69 | 1.512 | 0.227 |
|  |  | CI (C-I) | 1,69 | 0.550 | 0.460 |
|  |  | Period*CI | 2,69 | 0.415 | 0.661 |
| Mar 2013 – Yallakool & Colligen Creek freshes |  |  |  |  |  |
|  | flathead gudgeon | Period (B-D-A) | 2,74 | 1.192 | 0.309 |
|  |  | CI (C-I) | 1,74 | 0.378 | 0.540 |
|  |  | Period*CI | 2,74 | 1.933 | 0.151 |
|  | carp gudgeon | Period (B-D-A) | 2,74 | 1.138 | 0.325 |
|  |  | CI (C-I) | 1,74 | 0.042 | 0.836 |
|  |  | Period*CI | 2,74 | 2.303 | 0.107 |

Figure 93. Mean abundance (±1SE) of fish larvae present in the Edward-Wakool system before, during and after the second environmental fresh in Colligen Creek on 31 November 2012. This flow was aimed at eliciting a golden and silver perch spawning response. The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls. There were no significant interactions (see Table 24).

Figure 94. Mean abundance (±1SE) of fish larvae present in the Edward-Wakool system before, during and after an environmental flow in Yallakool Creek in November 2012. This flow was targeted towards maintaining water levels in Yallakool Creek to enhance Murray Cod spawning and dispersal.  The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls. There were no significant interactions (see Table 24).

February & March 2013 Yallakool Creek and Colligen Creek watering actions targeting the spawning and recruitment of small bodied fish.

Carp gudgeon and flathead gudgeon were the only species present in Yallakool and Colligen Creeks during the February and March 2013 watering actions in large enough numbers to be plotted visually (Murray River rainbowfish were also present in the system at this time, but only several individuals were collected) (Figures 95 and 96). For the February 2012 Yallakool Creek environmental fresh, statistical comparisons of Before-During-After, and Control-Impact could only be conducted for carp gudgeon. We found no significant change in carp gudgeon abundances either during or after the fresh compared to the ‘control’ rivers that did not receive environmental water (d.f=2, 69, F-test=0.41, p>0.05). Similarly, there was no significant different in carp gudgeon or flathead gudgeon abundances in either Yallakool or Colligen Creeks in March compared to Little Merran Creek and Wakool River, despite receiving sustained environmental freshes (Figure 96, Table 24).

Figure 95: Mean abundance (±1SE) of fish larvae present in the Edward-Wakool system before, during and after an environmental flow ‘fresh’ in Yallakool Creek in February 2012. This flow was targeted to eliciting a spawning and recruitment response from small-bodied fish. The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls.

Figure 96. Mean abundance (±1SE) of fish larvae present in the Edward-Wakool system before, during and after two environmental flow ‘fresh’ which occurred simultaneously in Yallakool Creek and Colligen Creek in March 2013. The Wakool River and Little Merran Creek did not receive environmental water, and were used as controls. There were no significant interactions (see Table 24).

Difference in total larval production across rivers

The abundance of larvae sampled over the entire sampling period was significantly different across the five rivers (d.f=4, F-test=32.0, p<0.001). Mean (±SE) numbers of larvae sampled from the Edward River, Little Merran Creek, Colligen Creek, Yallakool Creek and Wakool River were 523 (±118), 438 (±52), 116 (±18), 61 (±13) and 60 (±11), respectively. Australian smelt, the most abundant species, was found in significantly greater numbers in Little Merran Creek and the Edward River weirpool (d.f=4, F-test=20.52, p<0.001) (Figure 97), neither of which received environmental freshes. Carp gudgeon larvae were found in significantly greater numbers in Colligen Creek and the Edward River weirpool (d.f=4, F-test=17.13, p<0.001), Murray cod larvae were significantly more abundant in the Wakool River and Yallakool Creek (d.f=4, F-test=8.325, p<0.001), and flathead gudgeon were significantly more abundant in Little Merran Creek (d.f=4, Wald chi-square=67.24, p<0.001) (Figure 97). In contrast, larval abundances of carp were significantly less in Colligen Creek and Yallakool Creek (d.f=4, Wald chi-square=193.0, p<0.001) (Figure 97).

Figure 97. Mean total abundance of larvae for the five most abundant fish species collected in Colligen Creek, Yallakool Creek, Wakool River, Little Merran Creek and Edward River during the 2012-2013 sampling season; a) Australian smelt, b) carp gudgeon, c)Murray cod, d) flathead gudgeon and c) carp. Y-axes vary. Letters denote homogenous sub-set groups based on Tukey’s post hoc significance tests.

Summary

Ten species of fish, of which 8 were native, were found to spawn (as indicated by presence of larvae) in the Edward-Wakool system over the 2012-2013 spawning season. Larvae of only one species, carp gudgeon, were found in significantly higher numbers immediately after an environmental fresh when compared to rivers that did not receive environmental water. Although there were six environmental freshes delivered to Yallakool Creek and Colligen Creek combined, this increase in abundance of carp gudgeon was only observed after the Colligen Creek November 2012 fresh.

In-channel freshes targeted towards golden and silver perch spawning did not result in eggs or larvae being sampled. The capture of two silver perch larvae in light traps in Little Merran Creek, despite the relatively stable flows prior to their collection, support Mallen-Cooper and Stuart’s (2003) findings that this species is able to breed (albeit at low levels) without sustained in channel flow pulses. Further lines of evidence (for example, movement data and the capture of juveniles at the end of the spawning season in April), will help to provide more definitive conclusions as to whether or not these targeted pulses may resulted spawning responses of these two species.

Sustained and stable flows were maintained in Yallakool Creek in November 2012; with the aim to enhance the spawning and recruitment of Murray cod. The number of Murray cod larvae collected in Yallakool Creek was not significantly different to the Wakool River, which had much lower discharge during this time, indicating Murray Cod will spawn in the Edward-Wakool system regardless of flow conditions. These findings confirm the strong body of knowledge that currently shows Murray Cod spawn at peak times in November-December, and regardless of flow conditions (Rowland 1983; Humphries et al. 2005; Koehn and Harrington 2006; King et al. 2009). While the provision of environmental flows may not be beneficial for Murray cod spawning, the role of flow in aiding the dispersal of larvae of some species (e.g Murray cod, golden perch and silver perch) into suitable nursery and juvenile habitats is relatively unknown, but could be advantageous for recruitment success if larvae are dispersed into areas of high food resources (Welcomme 1979). The community sampling conducted in April 2013, will determine whether more Murray cod juveniles were found in Yallakool Creek compared to the control rivers (Wakool River and Little Merran Creek)(see section 7.5.3) and help elucidate whether or not environmental watering may have provided beneficial conditions for recruitment. However, based on results presented in previous sections we expect that increased recruitment in Yallakool Creek is likely to be limited, because of the lack of a significant boost in productivity of food resources, such as zooplankton and shrimp, as a result of the environmental watering actions.

In this study, we used the total abundance of larvae collected across the entire sampling period as an indicator for the magnitude of spawning occurring in the focus rivers. We tested the hypothesis that production of larvae for fish species would be greatest in the rivers receiving environmental freshes (Colligen Creek and Yallakool Creek) compared to those that did not (Wakool River, Little Merran Creek). Out of the three most abundant native species (carp gudgeon, Australian smelt, Murray Cod) we found no evidence to support this hypothesis. Overall, our findings show that the watering actions in the Edward-Wakool system did not significantly influence the spawning behaviour or spawning magnitude of the native fish species present in the system. For flow-dependent species such as golden and silver perch, it is likely that the magnitude and duration of the water actions was not great enough to promote spawning. For flow-independent species such as Murray cod, flow conditions appear unimportant for spawning, and therefore we would not expect an increase in spawning response from this species (Rowland 1983; Humphries 2005; Koehn and Harrington 2006; King et al. 2009b).For low-flow specialists, such as many of the smaller bodied native fish species, it may be that unless watering actions can provide a significant increase in low flow habitats (such as inundated slackwaters, backwaters and off channel wetlands) for significant periods of time that allow the spawning, hatching and rearing of larvae to take place, then the spawning response of such species in relation to watering actions will also be limited (Humphries et al. 1999).

Fish recruitment

Key findings

- The number of recruits of carp gudgeon was significantly higher in Yallakool Creek and Colligen Creek during and immediately after the November environmental watering actions compared to rivers that did not receive environmental water. However, the timing of larval abundance and hatch dates of recruits within the source river (Edward River) also increased over the same period of time. Thus, it is not possible to determine whether carp gudgeon larvae or recruits were generated from within Colligen and Yallakool Creek, or is due to recruits moving, or being transported, into these systems from the Edward River.

- Back-calculated larval hatch dates of Murray cod ranged from 16 October to 4 December 2012. There was a small peak in Murray cod larvae hatching around 20 October and a larger peak around 4 November 2012, both near the onset environmental watering actions in Colligen Creek and Yallakool Creek. However, there were no significant difference in Murray cod hatch dates among control or impacted river reaches for either of the watering actions. These data support the peer-reviewed literature that spawning of Murray cod is unrelated to river discharge.

- It was not possible to evaluate whether environmental watering actions influenced recruitment success of Murray cod as only 10 individuals were sampled in fish community surveys. Additional efforts targeting young-of-year recruits of large bodied species including Murray cod, golden perch and silver perch are warranted and could include back-pack electrofishing or community-based angling.

Background

One objective of environmental watering option 1 in the Edward-Wakool system (Table 3, Section 3) is to provide recruitment outcomes for native fish. Once reproduction has occurred, the definitive measure of spawning success is the number of individuals that survive to a particular point in time; referred to hereafter as recruitment. For purposes of this report, recruitment was defined as survival to the end of the spring-summer reproductive season. Environmental watering in the Edward-Wakool system during 2012 -2013 aimed to create conditions to facilitate native fish spawning and recruitment. Fish related flow objectives were to: enhance Murray Cod recruitment in Yallakool Creek (19 October - 7 December 2012), provide opportunities for small bodied fish recruitment, and flow specialists (golden perch and silver perch) movement and spawning in Yallakool Creek (1 February – 23 February 2013, and 13 March – 5 April), and to promote small bodied fish recruitment and flow specialist movement and potential spawning in Colligen Creek (13 March – 5 April).

Recruitment in fish that live in flowing water is tightly coupled with historical patterns of the natural flow regime associated with flooding, drought and regular seasonal cycles. Fish fauna of the Murray-Darling Basin evolved in a highly-variable flow system and species have subsequently developed a range of strategies for successful migration, spawning and recruitment (Humphries et al. 1999; Tedesco et al. 2008). It is therefore unlikely that a single flow regime will provide equal benefits for the entire fish community.  Given the relatively high reproductive output of fishes, small changes to recruitment success can result in cascading effects on populations (King et al. 2013). Therefore, improvements in managing regulated river flow regimes through environmental watering actions has potential to benefit native fish communities (e.g. Baumgartner et al. 2013). Likewise, the mismanagement of river flow regimes may be detrimental to recruitment of native fishes and their populations.

There is some disagreement in the peer-reviewed literature concerning how river flow regimes and regulation influences the recruitment of Murray-Darling Basin fishes (see Harris and Gehrke 1994; Humphries et al. 1999). Given the erratic rainfall patterns of south-eastern Australia many, if not most, species of fish in the Murray-Darling Basin are able to spawn and recruit under low-flow conditions (see Humphries et al. 1999). Understanding how flooding (Harris and Gehrke 1994) and natural flow regimes (Humphries and Lake 2000) influences the magnitude of recruitment, however, remains speculation and an emerging area of investigation (see King et al. 2013).

A useful mechanism to determine the timing of spawning and recruitment success following environmental flow delivery is to determine the age of individual fish and relate that back to the time of eggs hatching. For older fish, age can be determined to a specific year, but for young fish (up to 200 days old) the birth date can be estimated. Annual age estimation can be used to identify successful recruitment years, which in turn can inform environmental water delivery practices. Daily age estimates provide more accurate estimates of hatch dates and can be used to evaluate the precise environmental and river flow conditions that maximise recruitment (Humphries et al. 2013). Importantly, this information can then help to determine if fish were hatched before, during, or after a period of environmental water delivery.

In this section we assess whether the timing of carp gudgeon recruitment and Murray cod hatch dates were associated with 2012-13 environmental watering actions.  The null hypothesis tested was that environmental flows would have no effect on recruitment of carp gudgeon, or the hatch date distribution of Murray cod, and it was therefore consistent with the low-flow recruitment hypothesis (Humphries et al. 1999). Daily and annual age estimates were made on a range of other species sampled in fish community surveys (section 7.5.3) and these were used as a pilot study to establish a proof of concept approach for evaluating fish community recruitment outcomes following environmental watering actions. Statistical analyses could only be carried out on carp gudgeon and Murray cod in relation to 2012-13 environmental watering actions. Murray cod were selected because this species was the target of the October environmental watering action in Yallakool Creek. Carp gudgeon was selected because this was the only species that showed a statistically significant change in larvae following environmental watering actions (Table 24). Other fishes including silver perch, golden perch and several other small-bodied species were not sampled in adequate numbers as recruits to make inferences about 2012-13 environmental watering actions.

Methods

Carp gudgeon and Murray cod

Recruitment was calculated as the number of juveniles or early stage adults sampled throughout the end of the spring-summer reproductive season between January and April 2013. The term ‘recruits’ therefore refers to young-of-year fish sampled between January and April.  Otoliths (sagitta) were extracted from 15 to 20 larvae and juvenile carp gudgeon spanning the full range of lengths sampled from each river between January and April 2013. Fewer than 15 Murray cod larvae were available from most river-month combinations and in these cases all sagittae were extracted and polished. Otoliths were fixed to a microscope slide with the sulcus facing up using CrystalBond thermoplastic glue. The sagittal plane was polished flat with a grinding wheel fitted with a 15 µm or 6 µm wet polishing pad or with 6 µm, 3 µm and 1 µm dry lapping film. Microincrements were counted from the hatch check out to the anterior edge (Figure 98) using a compound microscope and 20X or 40X objective.  In sections where a hatch check was not apparent, microincrement counts started at the mean radius of hatch checks determined from otoliths where a check was discernable. In Murray cod, a hatch check radius of 55 µm was used which was based on validated daily age estimates (Humphries 2005).

Figure 98. Image of a carp gudgeon (12.0 mm SL) otolith illustrating an estimated 43 daily microincrements after the hatch check.  Scale bar represents 100 µm.

Recruitment sampling did not target or sample adults approaching the asymptotic length of species’, therefore it was not appropriate to attempt fitting standard fisheries growth curves, such as Gompertz or von Bertalanffy models, to age-length data. A variety of models, including exponential growth, power functions, and polynomial functions, were explored and a three parameter power function provided the best fit to daily age estimates of carp gudgeon, while a simple linear model provided the best fit for Murray cod larvae.  Age-length curves for carp gudgeon were pooled across rivers for each month due to the protracted spawning season and wide range of temperatures that could have affected growth. Age-length curves for Murray cod were developed for each river independently, given that the majority of larvae were collected within one month. Estimated daily ages, post-hatch, were rounded to the nearest whole number and were subtracted from the date of capture to determine individual hatch dates of fish. Duration of the spawning season was determined from back-calculated hatch dates of all stages of larvae, while the timing of recruitment was determined from the hatch dates of juveniles surviving to January through April 2013.

A before-after, control and impact, ‘repeated BACI’ (Underwood 1991), statistical design with multiple controls was used to test the effect of environmental watering actions on recruitment of carp gudgeon and larval hatch dates of Murray cod. The null hypothesis tested was that the mean number of recruits hatched within control rivers was not significantly different to impacted rivers before, during or after environmental watering actions. Means were calculated by the number of recruits hatched per day for the 14 days prior to and after each event and for the duration of each management action.

Statistical analyses were carried out in SigmaPlot (10.0.1). The ability to use hatch dates to evaluate the effect of environmental flows on fish recruitment within rivers relies on assumptions that: 1) selectivity for different age-classes of recruits is comparable among rivers in the BACI design and 2) that changes in the number of recruits within rivers is unrelated to rates immigration or emigration.   To test the first assumption we compared the median daily ages of carp gudgeon recruits and Murray cod larvae sampled among rivers using a Kruskal-Wallis One Way ANOVA.  To test whether potential increases in recruitment could be attributed to immigration we added the source (Edward River) of impacted rivers to the BACI design.  If changes in the number of recruits within the source river was not significantly different to the impacted river during or after environmental flows then the possibility for immigration could not be excluded. Impact rivers received environmental water, while controls were represented by all rivers that did not receive environmental water for each action.

Data were square root transformed to achieve equal variance and normal distribution structure. The mean number of recruits hatched per day were compared among control, impact and source (Edward River) rivers before, during and after environmental watering events using a two-way ANOVA. Post-hoc comparisons were conducted using the Holm-Sidak method which is recommended as more powerful test than Tukey and Bonferroni methods when conducting multiple pairwise comparisons. A Kruskal-Wallis One Way ANOVA on Ranks was used to compare median recruit hatch dates among rivers for the entire year in order to evaluate differences in the seasonal timing of peak recruitment. A Chi-square test was used to evaluate whether a significantly greater proportion of recruit hatch dates occurred during environmental watering actions compared to control rivers.

Fish community recruitment pilot study

Fish suspected to be young of year (0+), based on age-length approximations, were collected from sites during fish community surveys (see section 7.5.3). A sub-sample of otoliths across species and size classes were extracted for daily and annual age analysis. A sample of 30 fish was kept from the focus rivers, including length classes ranging from the smallest to largest estimated young of year fish. All sites occurred within the focus reaches used to monitor other indicators and allow for comparison with larval collection data (section 7.5.1). Additional samples were collected from Werai Forest because this zone may be a source of recruits for the Edward-Wakool river system. The estimated hatch dates of fish younger than 275 days were calculated by subtracting the daily age estimate from the capture date as described in the previous section. Annual age estimates on transverse sections of otoliths were made on older fish by enumerating opaque and translucent annuli (Figure 99).

Figure 99. Left; whole juvenile common carp otolith prior to sectioning. Right; magnified view of an otolith daily aged fish, where the rings on the otolith can be counted to get an accurate calculation of the fishes hatch date.

Results and discussion

Carp gudgeon

Daily age-length relationships estimated from readable carp gudgeon otoliths (n=323) were fitted successfully by a power-function for each month between January and April 2013 (Figure 100). An additional 39, or approximately 12% of carp gudgeon otoliths, were considered unreadable due to poor preparation and individual variation in clarity. No significant differences in age-length relationships among rivers for months combined suggested that variation in growth rates could not be attributed to environmental watering actions within impact rivers. Although microincrement banding in otoliths was clearly apparent (see Figure 98) and growth rates were similar to other small-bodied freshwater fish species where ages have been validated (see Tonkin et al. 2008b), it is important to note that the daily periodicity of microincrement formation in carp gudgeon otoliths has not been established.

Power functions provided a statistically significant (P<0.0001) fit to age-length relationships of carp gudgeon for all months (Table 25), although there was wide variation in standard error estimates of individual parameters. Standard error estimates associated with parameters yo and a were consistently high (Table 25) which suggested that model fits were imprecise for the smallest and largest sizes of fish. This variation was likely attributable to relatively few early-stage larvae being sampled in January to April 2013 and the decreasing precision of age estimates with increasing size of fish. Irrespective of the imprecision surrounding large and small fish, 95% confidence bands fitted to power functions (Figure 101) illustrated that age-length models for carp gudgeon provided precise age estimates for fish between lengths of 7 mm SL and 22 mm SL. Hatch dates of carp gugegon recruits (n=20072) sampled between January and April 2013 between the lengths of 8.1 and 25.2 mm SL were back-calculated to compare with the seasonal timing of environmental watering actions and peaks in larval abundance.

Figure 100. Estimated daily age-length curves for carp gudgeon (n=323) sampled each month in the Edward-Wakool river system between January and April 2013. Dashed lines represent 95% confidence bands.

Table 25. Power function (daily age = y0+a X SL ^ b) parameters fitted to carp gudgeon daily age-standard length (SL) data used to back-calculated hatch dates of juvenile recruits sampled between January and April 2013.  Parameter estimates ± Standard Error (SE).

| Month | n | r2 | yo | SE | a | SE | b | SE | F | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| January | 92 | 0.88 | -55.40 | 30.06 | 11.35 | 10.07 | 0.88 | 0.24 | 324.93 | <0.0001 |
| February | 80 | 0.89 | -212.22 | 233.03 | 110.11 | 180.41 | 0.34 | 0.29 | 296.65 | <0.0001 |
| March | 75 | 0.86 | -15.02 | 13.13 | 3.48 | 2.60 | 1.16 | 0.21 | 233.66 | <0.0001 |
| April | 77 | 0.87 | -212.92 | 252.34 | 123.32 | 203.63 | 0.32 | 0.29 | 245.04 | <0.0001 |

Back-calculated hatch dates of carp gudgeon recruits were estimated to fall between 24 August 2012 and 20 March 2013, with peaks occurring in December 2012 or early January 2013 in all rivers (Figure 100). There were small but significant differences (K-W ANOVA Ranks; H=2899; DF = 4; P<0.001) in the median hatch dates of carp gudgeon recruits among rivers. However, environmental watering actions did not significantly shift the timing of recruitment in control rivers compared to impact rivers. The earliest median hatch date (9 December 2012) occurred in Colligen Creek and the latest (1 January 2013) in the Edward river. Hatch dates of recruits generally occurred later in the year compared to the timing of peak larval abundance within respective rivers (see section 7.5.1) and compared to the timing of peak larval abundance in Barmah Millewa Forest (see King et al. 2013) upstream of the Edward-Wakool system. Later season recruitment compared to peak larval abundance of carp gudgeon observed here was a similar mismatch to that observed in Australian smelt by Humphries et al. (2013).

The majority of recruits in Colligen Creek (62%) and 42% of recruits in Yallakool Creek were hatched during environmental watering actions, although these percentages were significantly lower (Chi-square both > 5.89; DF=1; P< 0.05) than the percentages of recruits hatched within control rivers. These results could suggest that the average survival of early life history stages of carp gudgeon in Colligen Creek and Yallakool Creek may have been lower than in control rivers following environmental watering actions. This speculation highlights the importance of managing rivers to provide adequate river flow and ecological conditions following environmental watering actions that are targeted for enhancing fish spawning. Some evidence suggests that carp gudgeon recruitment is maximised by low-flow (Humphries et al. 1999) conditions during summer. It is plausible, therefore, that administering multiple environmental flows after spring-time spawning peaks could be detrimental to survival of carp gudgeon early life-history stages.

Figure 101. Back-calculated daily hatch dates of carp gudgeon (n=20072) recruits (juveniles and adults) sampled between January and April 2013. Black lines represent daily discharge and grey lines represent the proportion of recruits hatched each day.  Green and blue bands represent the timing of environmental watering actions.

Although proportionally fewer carp gudgeon larvae may have survived in the impacted rivers following environmental watering, the mean number of recruit hatch dates generated from the October 2012 Yallakool Creek watering action and the November 2012 Colligen action (TW ANOVA both; F>5.25; P<0.003) were significantly higher (Holm-Sidak both; t > 4.49; P<0.05) compared to control rivers (Figure 102). However, trends in the timing of larval abundance (see section 7.5.1) and hatch dates of recruits within the source river (Edward River) also increased over the same periods of time (Figure 102). This similarity means that it is not possible to determine whether larvae or recruits were generated from within Colligen and Yallakool Creek, or were transported downstream from the Edward River source.

These data lead to two possible conclusions: 1) the magnitude of carp gudgeon recruitment observed in Colligen Creek and Yallakool Creek during 2012-13 was augmented by environmental watering actions, or 2) the number of recruits observed after environmental watering actions in Colligen Creek and Yallakool Creek was attributed to immigration from the Edward River (source). Given that primary productivity and larval fish food availability did not increase in response to environmental watering and the body of empirical evidence suggesting that carp gudgeon are a low-flow recruitment specialist (Humphries et al. 1999), the second conclusion appears to be most probable. In the future it would be useful to investigate how, or if, environmental flows influence downstream dispersal of fish larvae and recruits from the Edward River into distributaries (Colligen Creek, Yallakool Creek, Wakool River) and how this affects native fish populations. Drift net samples did not collect elevated numbers of carp gudgeon larvae or juveniles following environmental watering actions, although sampling closer to the Edward River may help identify changes in the number of individuals entering distributaries.

The observation of environmental watering actions in Colligen and Yallakool Creek during spring resulting in an apparent increase in the early life history stages of carp gudgeon, due either to increased recruitment or downstream dispersal, is a similar result to environmental watering actions in November 2011 within Colligen Creek (Watts et al. 2013). Given that abundances of carp gudgeon larvae and juveniles were lowest in Yallakool Creek compared to all rivers in 2011 (Watts et al. 2013) and that that the total number of recruits increased significantly this year (Section 7.5.1), it appears that increasing river discharge associated with environmental watering actions can increase the number of early life-history stage carp gudgeon sampled in rivers of the Edward-Wakool system.

Figure 102. Two-way ANOVA comparing the mean number of carp gudgeon recruits hatched per day before, during and after the November 2012 Colligen Creek environmental watering action. Asterisks denote significant (P< 0.05) differences.

Murray cod

Daily age-length regressions (Figure 103) were developed from readable otoliths of 58 Murray cod larvae, while an additional 13 otoliths were considered unreadable. Due to the narrow range of lengths and small sample sizes of Murray cod larvae, regression fits for the Wakool River and Little Merran Creek were not significant (Table 26). Nonetheless, daily age-length estimates from all rivers fell within range of expected variation based on validated age estimates (Humphries 2005). Aged larvae ranged from 8.6 mm to 12.2 mm SL with a mean length of 9.7 ± 0.71 SD and mean daily age of 11 ± 4 days. No Murray cod recruits were collected using drift nets, light traps or boat tows but the hatch dates of 225 larvae were back-calculated to more closely approximate the timing of spawning in relation to environmental watering actions.

Figure 103. Estimated daily age-length curves for Murray cod sampled in the Edward-Wakool river system in November 2012.

Table 26.  Linear regression (daily age = y0+a X SL) parameters fitted to Murray cod daily age-standard length (SL) data used to back-calculated hatch dates.  Parameter estimates ± Standard Error (SE).

| Month | n | r2 | yo | SE | a | SE | F | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Colligen | 4 | 0.97 | -35.80 | 6.03 | 4.90 | 0.62 | 63.13 | 0.02 |
| Yallakool | 14 | 0.77 | -24.35 | 6.29 | 3.75 | 0.65 | 33.26 | 0.0002 |
| Edward | 12 | 0.76 | -30.42 | 7.52 | 4.03 | 0.76 | 28.41 | 0.0005 |

Back-calculated larval hatch dates of Murray cod ranged from 16 October to 4 December 2012 (Figure 104) and there were no significant differences in median hatch date among rivers. No Murray cod recruits were sampled using drift nets, light traps or boat tows but the hatch-dates of recruits (n=10) sampled in annual fisheries surveys (see section 7.5.3) fell within the seasonal timing of spawning estimated above.

There was a small peak in Murray cod larvae hatching around 20 October and a larger peak around 4 November 2012 (Figure 104); both near the onset environmental watering actions in Colligen Creek and Yallakool Creek.  However, there were no significant changes in mean abundance of Murray cod hatch dates among control or impacted river reaches for either of the environmental watering actions in Yallakool Creek or Colligen Creek. These data corroborate results from fortnightly abundance of larvae (see section 7.5.1) and the peer-reviewed literature (Humphries et al. 1999; Humphries 2005) further suggesting that spawning of Murray cod is unrelated to river discharge. However, it remains uncertain whether environmental flows affected recruitment success of large bodied species in the Edward-Wakool system. It was not possible to evaluate whether environmental watering actions influenced recruitment success of Murray cod given that only 10 individuals were sampled in fish community surveys. Additional efforts targeting young-of-year recruits of large bodied species including Murray cod, golden perch and silver perch are warranted and could include back-pack electrofishing or community-based angling events. More effective sampling of young-of-year large bodied species will be needed to statistically evaluate whether future environmental watering actions influence recruitment success of these species.

Figure 104. Back-calculated daily hatch dates of Murray cod larvae (n=225) sampled in the Edward-Wakool system in 2012. Black lines represent daily discharge and grey lines represent the proportion of larvae hatched each day.  Green and blue bands represent the timing of environmental watering actions

Fish community recruitment pilot study

A total of 391 otoliths from seven species were prepared for age analysis to determine if hatch dates could be compared with the timing of environmental watering actions. Of the aged fish, 100 were estimated to be greater than 1 year old and 14 of these were considered unreliable age estimates. Daily ages of 277 juvenile fish were determined including: 88 Australian smelt, 28 goldfish, 8 carp, 10 Murray cod, 119 carp gudgeon and 24 bony herring. Hatch dates were estimated and plotted for all species (Figure 105-108) but given the small sample size available for each species/month/river combination and the lack of validated age estimates for most species, these data were not used to examine the effects of environmental flows on fish recruitment. These data do, however, illustrate the wide seasonal range of recruit hatch dates among species and how this information could be used to examine the effects of environmental flows. Based on these results, the influence of future environmental watering actions could be evaluated using back-calculated hatch dates across a wide variety of target species and non-native fishes. In order to apply this methodology to evaluate the outcomes of future environmental watering actions, age-length curves need to be established and validated for each target species and these data could then be extended to fish community length data to estimate periods of peak recruitment.

Flow (ML/day)Flow (ML/day)

Figure 105. Spawning numbers (bubble sizes) from all sites situated on Yallakool Creek, per species (left axis). Yallakool flow is on right axis. Environmental flow period is highlighted.

Flow (ML/day)

Figure 106. Recruit numbers (bubble sizes) from all sites on the Wakool River (except Wakool Reserve), per species. Wakool River flow is on the right axis. (note; no environmental water delivered to the Wakool)

Flow (ML/day)

Figure 107. Recruit numbers (bubble sizes) from all sites on Colligen Creek, per species. Colligen Creek flow is on the right axis.

DOFlow (ML/day)

Figure 108. Recruit numbers (bubble sizes) from all sites in the Werai, per species. Edward River flow on the right axis.

Long-term fish community structure (2010-2013)

Key findings

- The analysis of the fish community from 2010 to 2013 recorded nine of the 21 species thought to occur in the central Murray region of the Murray-Darling Basin prior to European settlement. Apex predators and flow specialists had higher biomass in channel habitat and flow generalists had higher biomass in wetland habitat.

- Flooding and subsequent blackwater events in 2010 and 2011 are still having a strong influence on fish community structure. Blackwater events in 2010 triggered fish kills throughout the system with the exception of the upstream zone where irrigation outfalls were used to deliver environmental water to improve water quality. Murray cod largely disappeared from all but the upstream sections of this system.

- There has been a small recovery of native fish populations in the system since the blackwater events, however there has been limited recruitment. Successive years of environmental water delivery targeted at Murray Cod recruitment in the upper zone may have contributed to recovery of the fish community through connection of critical habitat, maintenance of low flow refuges and providing conditions to promote dispersal of individuals to recolonise areas impacted by the blackwater events. However, the environmental watering in 2012-13 did not trigger widespread recruitment that is necessary for population growth.

- The lack of evidence for recent recruitment of flow dependent specialists such as golden perch and silver perch suggests that watering strategies have been insufficient to trigger recruitment in these species.

Background

A system-wide monitoring program was established in 2010 to provide baseline information on native fish population status in the Edward-Wakool system and inform environmental water delivery and habitat (e.g. refuge pool) management. The program involved establishing long-term fish monitoring sites that have been sampled consecutively for four years. The overall objective was to determine fish community outcomes from environmental water delivery, including learning from the flow regime delivered within a watering year, and to also examine long term change trajectories.

Unregulated flooding events in the summer of 2010-11 provided information on fish community transition from one largely dependent on drought refuge, to one that responded to a large natural flooding event. This has provided improved understanding of the effect of flow regimes on the fish population, and the benefits of environmental water delivery to the system both during drought, flood (including hypoxic blackwater), and post flood years.

Effective delivery of environmental water requires knowledge and understanding of fish community response to flow. Within the Edward Wakool system a lack of information is precluding effective fisheries management. For example, there is little information on spatial fish community distribution within the system, no data on habitat responses to flow management during drought or high flows, and little data on impacts and recovery from hypoxic blackwater events. The present study sought to gather information on each of these areas to help to progress management outcomes in the short term. The aims of the fish population surveys include:

- To determine whether peaks in fish abundance and biomass align with years in which environmental watering occurred

- To determine if environmental water delivery can assist in recovery of populations that were negatively impacted by blackwater events at the end of the milleni um drought in 2009. The release of environmental water is expected to increase the abundance of native fish in blackwater affected zones.

- To determining if exotic pest fish (e.g. carp) distribution and abundance respond to water delivery actions.

Methods

Water quality parameters (pH, dissolved oxygen, temperature, turbidity) were recorded at each site using a Model U10 Horiba multi-probe water quality meter. Readings were taken at the surface, 0.5 m, 1 m and then at 1 m increments until the bottom was reached. The water quality readings are only short-term indicators but can give a strong appreciation of unseasonal stratification, particularly within deep pool habitat.

Fish were collected from 43 sites within the Edward Wakool system, with sampling occurring annually between 2010 and 2013. Sites were stratified between wetland and channel habitats to determine the role of these different habitat types in supporting fish communities. These habitats were further stratified by position in the system; upper, middle and lower and a Werai zone) to capture any potential differences that may arise over a larger spatial scale (Figure 109). These sites provide data on the distribution, abundance and diversity of juvenile and adult fish in the system.

Fish were collected using a standardised electrofishing (Figure 110),protocol including 10 bait traps (Figure 110) established by the Sustainable Rivers Audit (SRA) for the Murray Darling Basin (Davies et al. 2010). All bait traps were baited with cat food to enhance the potential capture of southern pygmy perch. This was augmented by a netting strategy to capture any cryptic species that may have been present. This involved setting two 15 m long (2 m drop) monofilament multi panel gill nets with single 35, 75, 100 mm mesh panels for two hours (Figure 111). Additionally, two 3 mm dual wing fyke nets and two 25 mm single wing fyke nets were set and retrieved the following day to encompass diurnal periods (Figure 112). At the completion of each electrofishing and netting operation, all fish were identified, counted and measured (maximum of 50 individuals per species per shot). A sub sample of suspected young-of-year fish collected during the course of routine sampling, euthanized and preserved for age recruitment analyses (see section 7.5.2 on fish recruitment).

Figure 109. The Edward-Wakool River system highlighting all major rivers and creeks including all zones, and sampling sites delineated by habitat type.

Lower

Zone

Middle

Zone

Upper

Zone

Werai Zone

Calculating fish health metrics

Winemiller and Rose (1992) identified a classification system for North American fish and determining likely responses to flows. The work largely identified that fish differ in expected responses to different flow scenarios which are based on specific ecological needs. King et al. (2013) further extrapolated these concepts to the reproductive life history of Murray-Darling fish species. However, neither author extended this work to develop specific hydrographs which link complex ecological processes with flow events. Baumgartner et al. (2013) extended this earlier work by defining environmental water delivery requirements of different groups. Recognising that one flow would not be sufficient to meet the needs of all species, fish with similar trophic and reproductive strategies were grouped, functionally, and hydrographs were developed to guide water delivery planning. Four native fish flow groups and one general alien species group are used; foraging (flow) generalists, apex predators, flow dependant specialists, floodplain (off-channel) specialists and alien species. Rationale for these groupings and nomenclature are presented in Baumgartner et al. (2013).

Fish community data was summarised to compare results initially to three main SRA Indicators (these are fully explained in Robinson 2012). The SRA derived Indicators calculated were; Expectedness (provides a comparison of existing catch composition with historical fish distributions), Nativeness (combination of abundance and biomass describing the proportion of the community comprised of native fish), and Recruitment (provides a proportion of the entire native fish population that is recruiting within a zone). Recruitment was further divided; recruiting taxa (proportion of native species present recruiting), and recruiting sites (proportion of sites where recruitment occurs). These Indicators produce a score that is related to Reference conditions, and receive a condition rating (Extremely Poor (0-20), Very Poor (21-40), poor (41-60), Moderate (61-80), Good (81-100).

Due to natural flow attenuation, hydrology of the Edward-Wakool system varies in an upstream to downstream direction. Flows in the lower end of the system are consequently lower and it was hypothesised that this may have contributed to some flow-related fish community structuring. It was also hypothesised that fish communities differ at local scales between channel and wetland habitat. Both of these factors were likely to influence spatial structure in fish communities, which may vary on an annual scale depending on catchment conditions. To account for these factors, fish community structure was analysed using PERMANOVA (PRIMER, with region (upper, middle and lower), habitat type (wetland or channel) and year (2010, 2011, 2012 or 2013) as fixed factors. Tests were performed using 999 Monte Carlo randomisations to calculate approximate probabilities.

Figure 110. (2a left) Boat electrofishing used to sample juvenile and adult fish communities. (2b right) Bait traps baited with canned cat food are set to sample small fish not efficiently sampled during routine electrofishing.

Figure 111. Monofilament multi panel gill nets 15 metre long (2 metre drop) with single 35, 75, 100 mm mesh panels are set for 2 hours at survey sites.

Figure 112. Large mesh single wing and small mesh dual wing fyke nets are set and retrieved the following day to encompass diurnal periods.

Results and discussion

Hydrology and water quality

For nine years prior to August 2010 the Edward-Wakool system experienced a period of severe drought with extended low flows, and extreme low flows between 2007 and 2009. In August 2010 system-wide flooding occurred with widespread inundation and overbank flows (Figure 113). Lasting several weeks, flow exceeded bankfull capacity in September 2010 in the upper Edward, the upper Wakool River, Yallakool Creek, Merran Creek and the Upper Colligen-Niemur system. A follow up unregulated high flow occurred in November-December 2010 which resulted in hypoxic blackwater fish kill (particularly in Murray cod). Fish kills were observed throughout the system but were most significant in the middle and lower zones, including Werai zone. Unregulated high flow events have been occurring in most of the system each spring since the initial flooding in August 2010, but no further fish kills have been associated with these events.

Hypoxic Blackwater event

Extreme low flow period where most of system disconnected from Edward

Figure 113.. Hydrograph for the Edward River (right axis) and Wakool, Yallakool, Colligen-Niemur-Merran (left axis) between 2003 and 2013. The drought period extends from 2003 to August 2010, with extreme low flow period between 2007 to 2009. In August 2010 flooding was experienced throughout the system. In December 2010 follow up overbank flooding occurred and hypoxic blackwater fish kills occurred in the entire system.

Surface (top 20 cm) water quality readings were consistent among sites. However, during sampling in 2010, water quality deteriorated within the lower zone at certain sites (example given at Stoney Crossing, Figure 114). Dissolved oxygen, pH, salinity and temperature were significantly different in the lower water column because of stratification during low flows and disconnection during the drought. Flooding caused complete mixing of the water column and water quality at these affected sites in the lower zone returned to safe levels for fish in 2011, and has persisted since.

Depth (m)

Depth (m)

ms/cm

mg/L

Conductivity

DO

Figure 109. Electrical conductivity, and dissolved oxygen water quality parameters at each metre of water at Stony Crossing (lower zone site). in 2010 (green triangle), 2011 (red circle), 2012 (grey diamond), and 2013 (blue square). Figure depicts the change in water quality with depth and demonstrates stratification despite these readings being taken in July 2010, and water quality after significant mixing in subsequent sampling years (2011 and 2013). Red line denotes levels that are considered harmful to native fish.

Fish results

There were 18,909 fish collected in total from 2010 to 2013; 6,392 in 2010, 5,618 in 2011, 2,186 in 2012, and 4,713 in 2013 (Table 27, Table 28). Nine native species and five alien species were collected in total over the sampling period (Table 27). Individuals from three of the four native flow guilds were represented (Figure 115), as no floodplain specialists were sampled in any year. Flow generalists dominated the total and relative abundances in all zones and habitat types in 2010, but following flooding in 2010-11 the alien group dominated total and relative abundance for many of the zones (Table 27). The alien group (especially carp and goldfish) dominated total and relative biomass in all zones and habitat types in all years. Substantial increases in total and relative biomass occurred in this group after the 2010-11 flooding, but abundance declined in subsequent years (Table 27). Nine of the 21 species thought to occur in the central Murray region of the Murray-Darling Basin prior to European settlement were recorded during this annual survey (Table 29), and one additional species, freshwater blackfish, has been collected as larvae (section 7.5.2).

Table 27. Total number of fish caught in each year of sampling and the mean (and standard error in parenthesis) number of fish per site per year.

Table 28. Total number of fish caught 2013 and the mean (and standard error in parenthesis) number of fish per site.

| 2013 Summary | Upper |  |  |  | Middle |  |  |  | Lower |  |  |  | Werai |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Species | Channel |  | Wetland |  | Channel |  | Wetland |  | Channel |  | Wetland |  | Channel |  | Wetland |  |
| Apex Predators |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Murray cod | 50 | (7±2) | 0 | (0±0) | 10 | (3±2) | 8 | (8±0) | 8 | (8±0) | 0 | (0±0) | 2 | (1±0) | 0 | (0±0) |
| Flow dependent specialists |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Siver perch | 6 | (2±0) | 0 | (0±0) | 10 | (1±0) | 2 | (1±0) | 0 | (0±0) | 0 | (0±0) | 0 | (0±0) | 0 | (0±0) |
| Golden perch | 25 | (4±1) | 0 | (0±0) | 2 | (3±1) | 7 | (2±1) | 18 | (4±1) | 1 | (1±0) | 2 | (2±0) | 0 | (0±0) |
| Flow Generalists |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Australian Smelt | 228 | (23±9) | 103 | (34±31) | 102 | (20±14) | 38 | (13±2) | 38 | (6±2) | 1 | (1±0) | 42 | (14±8) | 10 | (10±0) |
| Bony Herring | 46 | (22±11) | 0 | (0±0) | 8 | (3±1) | 11 | (4±1) | 38 | (4±2) | 0 | (0±0) | 6 | (6±0) | 0 | (0±0) |
| Carp Gudgeon | 1801 | (200±193) | 193 | (97±1) | 2 | (1±0) | 79 | (16±9) | 8 | (4±3) | 100 | (33±26) | 2 | (1±0) | 313 | (157±150) |
| Flat-headed gudgeon | 0 | (0±0) | 0 | (0±0) | 0 | (0±0) | 1 | (1±0) | 0 | (0±0) | 9 | (9±0) | 0 | (0±0) | 6 | (6±0) |
| Murray-Darling Rainbowfish | 9 | (2±1) | 2 | (2±0) | 1 | (1±0) | 6 | (3±1) | 1 | (1±0) | 0 | (0±0) | 0 | (0±0) | 0 | (0±0) |
| Un-specked hardyhead | 10 | (10±0) | 3 | (3±0) | 0 | (0±0) | 3 | (2±1) | 0 | (0±0) | 0 | (0±0) | 3 | (3±0) | 0 | (0±0) |
| Alien Species |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Carp | 434 | (43±13) | 198 | (50±16) | 124 | (21±6) | 227 | (32±15) | 86 | (12±2) | 24 | (8±4) | 89 | (29±10) | 5 | (3±1) |
| Gambusia | 1 | (1±0) | 1 | (1±0) | 0 | (0±0) | 1 | (1±0) | 0 | (0±0) | 1 | (1±0) | 0 | (0±0) | 4 | (4±0) |
| Goldfish | 110 | (18±6) | 18 | (9±1) | 3 | (2±1) | 51 | (9±4) | 23 | (5±2) | 10 | (4±1) | 1 | (1±0) | 0 | (0±0) |
| Redfin Perch | 2 | (2±0) | 0 | (0±0) | 0 | (0±0) | 2 | (2±0) | 1 | (1±0) | 0 | (0±0) | 0 | (0±0) | 0 | (0±0) |

Figure 115. Some of fish species caught during fish community sampling in the Edward-Wakool. (Images of Murray cod, silver perch and common carp appear elsewhere in this document).

Table 29. PERCH list (pre-European) of expected native species for the central Murray region of the Murray-Darling Basin

| Common Name | Species Name | Found in present study |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2010 | 2011 | 2012 | 2013 |
| Flow Generalists |  |  |  |  |  |
| Freshwater catfish | Tandanus tandanus | N | N | N | N |
| Bony herring | Nematalosa erebi | Y | Y | Y | Y |
| Freshwater blackfish | Gadopsis marmoratus | N | N | N | N |
| Shortheaded lamprey | Mordacia mordax | N | N | N | N |
| Macquarie perch | Macquaria australasica | N | N | N | N |
| Murray-Darling rainbowfish | Melanotaenia fluviatilis | Y | Y | Y | Y |
| Un Specked hardyhead | Craterocephalus stercusmuscarum | Y | Y | Y | Y |
| Murray hardyhead | Craterocephalus fluviatilis | N | N | N | N |
| Australian smelt | Retropinna semoni | Y | Y | Y | Y |
| Carp gudgeon | Hypseleotris spp | Y | Y | Y | Y |
| Flatheaded gudgeon | Phylipnodon grandiceps | Y | N | N | Y |
| Dwarf Flatheadedgudgeon | Philypnodon macrostomus | N | N | N | N |
| Apex Predators |  |  |  |  |  |
| Murray cod | Maccullochella peelii | Y | Y | Y | Y |
| Trout cod | Maccullochella macquariensis | N | N | N | N |
| Flow Dependent Specialists |  |  |  |  |  |
| Golden perch | Macquaria ambigua | Y | Y | Y | Y |
| Silver perch | Bidyanus bidyanus | Y | Y | Y | Y |
| Floodplain (or Off Channel) specialists |  |  |  |  |  |
| Southern pygmy perch | Nannoperca australis | N | N | N | N |
| Southern Purple Spotted gudgeon | Mogurnda adspersa | N | N | N | N |
| Flatheaded galaxias | Galaxias rostratus | N | N | N | N |
| Olive perchlet | Ambassisagissizi | N | N | N | N |
| Mountain galaxias | Galaxias olidus | N | N | N | N |

Sustainable Rivers Audit (SRA) Indicators

Expectedness was above 70% (moderate condition rating) in all zones in 2010, but decreased in subsequent years of sampling. Each zone (except Werai zone) had fallen below 50% (poor condition rating) by 2013 (Table 30). Although many expected species were present, the moderate rating for 2010 is largely because floodplain specialists remain undetected. Expectedness in the Werai zone increased each sampling year (Table 30) indicating that successive inundation events may be providing a benefit to the wetlands.

Nativeness was above 61% (moderate condition rating) for the upper and lower zones in 2010 but below 50% (poor condition rating) for the middle zone. Nativeness subsequently decreased in every year of sampling in all zones (Table 30). The Werai zone was below 20% (extremely poor condition) in 2011, but improved in 2012 to a ‘poor’ rating before decreasing in 2013 to below 41% (very poor condition rating). As nativeness is a combination of abundance and biomass, an increase in carp is the primary influence in all zones except Werai . Targeted delivery of water which benefits native fish abundance and biomass or targets removal of carp could be utilised to improve the nativeness rating.

Recruitment (proportion of taxa) remained above 61% (moderate rating) in all zones, in all years, (except for the middle zone in 2011 and Werai 2013). However recruitment (proportion of sites) which was above 61% (moderate) in all zones, had decreased across all zones by 2013. This indicates that most native species present were recruiting across the system as a whole, particularly as the drought broke. Post flooding, most recruitment has been dominated by alien species which spawned after over bank flows in 2011. Following this flood event there has been little recruitment in other species.

Table 30. SRA indicators (Expectedness, Nativeness and two recruitment metrics) for all zones and all years

| Location | year | OE_metric | Nativeness | Recruitment (Propotion of taxa) | Recruitment (Proportion of sites) |
| --- | --- | --- | --- | --- | --- |
| Upper | 2010 | 74 | 67 | 67 | 64 |
| Upper | 2011 | 67 | 48 | 63 | 56 |
| Upper | 2012 | 58 | 36 | 63 | 52 |
| Upper | 2013 | 48 | 38 | 78 | 50 |
| Middle | 2010 | 76 | 48 | 78 | 71 |
| Middle | 2011 | 53 | 24 | 63 | 53 |
| Middle | 2012 | 57 | 38 | 75 | 63 |
| Middle | 2013 | 47 | 45 | 78 | 44 |
| Lower | 2010 | 80 | 62 | 78 | 73 |
| Lower | 2011 | 63 | 28 | 57 | 57 |
| Lower | 2012 | 56 | 52 | 75 | 66 |
| Lower | 2013 | 48 | 40 | 75 | 58 |
| Werai | 2011 | 23 | 18 | 67 | 65 |
| Werai | 2012 | 37 | 41 | 67 | 49 |
| Werai | 2013 | 35 | 30 | 57 | 48 |

Fish community structure - Abundance

Based on fish abundance data, fish communities significantly differed among zones (PERMANOVA: Pseudo F = 4.47, P < 0.001), habitat type (Pseudo F = 13.24; P < 0.001) and years (PERMANOVA: Pseudo F = 8.38; P < 0.001). Zone based differences arose largely from differing relative higher abundances of Australian smelt and carp gudgeons from middle sections than upper and lower. Fish community observations in 2011 and 2012 were largely influenced by increases in the abundance of carp gudgeon and Australian smelt from upper wetlands and from higher carp abundances from middle and lower channel habitat. Australian smelt and bony herring were generally more commonly captured in channel habitat. Carp gudgeon, Un-specked hardyhead and Murray-Darling rainbowfish were more commonly collected from wetland habitat.

Ordination using Principal Components Analysis (Figure 116) identified clear groupings based on sampling year, which were largely centred around natural flooding in 2011. For flow generalists carp gudgeon, unspecked hardyhead and Murray-Darling rainbowfish were far more abundant prior to flooding than in subsequent years. Carp gudgeon abundance declined substantially after flooding in 2011 but increased in 2012 and again in 2013.

Carp were the most abundant species in all years except 2010 but peaked following flooding in 2011 (Figure 117). Goldfish abundance also peaked following flooding in 2011 but have declined in each subsequent year. Other alien species contributed little to observed annual differences.

Apex predators were abundant throughout the system in 2010 but declined following hypoxic blackwater events in subsequent years. The group has partly recovered post-flooding but are still in low abundance (Figure 118).

Figure 116. Principal components ordination of fish collection sites in the Edward/Wakool system. Groupings are based on similarity following conversion of standardised abundance to Bray-Curtis similarities measures after pooling electrofishing and netting data and applying a fourth root transformation. Sites are stratified into lower (triangle), middle (square) and upper (circle) geographic regions. Channel sites are solid and wetland sites are hollow. Colour depicts the year of sampling in either 2010 (green), 2011 (dark blue), 2012 (light blue) and 2013 (pink). The overlayed vector plot identifies species contributing to group separation. A longer line suggested a stronger contribution to separation in ordinal space. Species codes refer to carp (cypcar), goldfish (caraur), carp gudgeon (hypspp), yabby (chedes), gambusia (gamhol), Murray Rainbowfish (melflu), un-specked hardyhead (craste), Murray cod (macpee), Australian smelt (retsem), golden perch (macamb), flatheaded gudgeon (phigra) and redfin perch (Perflu).

Fish community structure - Biomass

Fish communities, based on fish biomass data, significantly differed among zones (PERMANOVA: Pseudo F = 6.89, P < 0.001), habitat type (Pseudo F = 12.43; P < 0.001) and years (PERMANOVA: Pseudo F = 6.33; P < 0.001) (Figure 117).

By abundance, there were more small-bodied flow generalist species (Figure 118) than any other across all zones. By biomass, however, carp and goldfish dominated catches in each zone but more were collected from upper zones (Figure 119). Apex predators (Figure 120) and flow specialists (Figure 121) contributed more to overall catches from the lower zone than upper or middle.

Differences among habitat types were largely driven by golden perch and Murray cod. These species were far more abundant in channel habitat than wetland suggesting that in channel flow delivery would be much more efficient at maintaining habitat quality and for stimulating movement. Carp biomass was generally higher within channel but goldfish biomass was greater from wetland habitat.

Significant differences in biomass among years were largely due to an increase in carp and goldfish following flooding in 2011 (Figure 118). In addition, Murray cod, Murray-Darling rainbowfish, Australian smelt and Golden perch were in substantially greater abundance prior to the widespread hypoxic blackwater event. Murray cod were not collected in 2011 and few in 2012 (Figure 119). Many larger fish were collected in 2013 demonstrating that the species is recovering.

Figure 117. Principal components ordination of biomass of fish collection sites in the Edward/Wakool system. Groupings are based on similarity following conversion of standardised biomass to Bray-Curtis similarities measures after pooling electrofishing and netting data and applying a fourth root transformation. Sites are stratified into lower (triangle), middle (square) and upper (circle) geographic regions. Channel sites are solid and wetland sites are hollow. Colour depicts the year of sampling in either 2010 (green), 2011 (dark blue), 2012 (light blue) and 2013 (pink). The overlayed vector plot identifies species contributing to group separation. A longer line suggested a stronger contribution to separation in ordinal space. Species codes refer to carp (cypcar), goldfish (caraur), carp gudgeon (hypspp), yabby (chedes), gambusia (gamhol), Murray Rainbowfish (melflu), un-specked hardyhead (craste), Murray cod (macpee), Australian smelt (retsem), golden perch (macamb), flatheaded gudgeon (phigra) and redfin perch (Perflu).

Figure 118. Total biomass of flow generalist species (biomass pooled among species) caught at each sampling site in the Edward-Wakool river system (2010-2013).

Figure 119. Total biomass of alien species (biomass pooled among species) caught at each sampling site in the Edward-Wakool river system (2010-2013).

Figure 120. Total biomass of apex predators (Murray cod) caught at each sampling site in the Edward-Wakool river system (2010-2013).

Figure 121. Total biomass of flow specialist species (biomass pooled among species) caught at each sampling site in the Edward-Wakool river system (2010-2013).

Discussion

Environmental flow delivery and water quality management during drought

There was stronger native fish recruitment and lower alien species recruitment during drought conditions. Native fish recruitment during drought was dominated by small bodied generalist species which are likely to be able to persist under such conditions. During drought, a main issue impacting on fish persistence is water quality (Closs and Lake 1996). Reduced water quality was experienced at sites in the lower zone, likely impacting native fish populations, which were least abundant in this zone at that time. Environmental water delivery to target poor water quality should be a priority during low flow periods. Watkins et al. (2010) suggested maintenance of continuous flows in warmer months to avoid stratification in waterholes <3 m in depth. If insufficient water is available (e.g. severe drought) to allow this to occur, then environmental water should be targeted at avoiding mixing with stratified layers, but allowing the upper layers of the water column to maintain fish populations.

There was high biomass of apex predators and flow specialists in channel habitat and high abundance of flow generalists in wetland habitat. During drought, flow should be delivered to maintain source populations, including those which utilise both channel and wetland sites. Specific refuge holes in channel (e.g. Wakool Reserve) and wetlands or complexes (e.g. Werai zone) should be targeted during drought periods to maintain refuges and allow source populations to persist until larger-scale targeted flows can be delivered or natural high unregulated flows return to allow for dispersal. In-channel flows should be provided to promote a productivity boost to the system, and allow movement and spawning opportunities for in-channel spawners, such as apex predators and flow dependant specialists. Wetland watering should be targeted to areas with good native populations (source populations maintained during drought), preferably with low numbers of aliens, with consideration given to water quality.

Influence of flooding on fish community structure

Flooding, and subsequent blackwater events had the greatest influence on fish community structure. Firstly, following the blackwater events the apex predators, such as Murray cod, largely disappeared from all but the upper zone, where in 2010 and 2011 irrigation offtakes were used improve water quality. Murray cod were largely absent from lower zones following flooding, but the species is slowly beginning to recolonise the hypoxic blackwater-affected areas. A targeted re-stocking program has sought to re-introduce Murray cod and golden perch to blackwater affected areas. Each fish was marked with Calcein, to determine the overall contribution of stocking to population recovery. However, only a single stocked fish has been recovered from the release sites, demonstrating that recolonisation is from natural recruitment. Delivery of environmental water during the Murray cod spawning season (October – December) could be a mechanism to facilitate natural recovery by providing opportunities for downstream larval drift.

Secondly, flooding triggered the proliferation and widespread recruitment of carp and goldfish throughout the system. Both species are known to respond to high flows and demonstrated strong recruitment responses. Alien species initially spiked with strong flood recruitment, however this increase in abundance and biomass did not persist and has consistently reduced each subsequent year. It is likely that the flood created conditions favourable for larval survival in these alien species (Brown et al. 2005). In addition, carp and goldfish have known tolerances to hypoxic conditions and an ability to perform anaerobic metabolism during periods of stress (Balashov and Recoubratsky 2011) and these physiological characteristics would have helped these species persist through the hypoxic blackwater conditions.

Environmental watering outcomes for specific flow guilds

Flow generalists

Flow generalists were in highest abundance following the low flow period of the drought in all habitat types and zones. This may indicate a possible preference for low flow recruitment (Balcombe and Closs 2000), or an ability to respond quickly to small increases in flow that occurred during the drought (Gilligan et al. 2009). This highlights the importance of having low flow periods within channel. Flow generalists also increased in the Werai wetlands each year indicating that the inundation events in this wetland system may increase the ability of flow generalists to proliferate in floodplain wetlands. Therefore, establishment of some semi-permanent wetlands in the Werai zone may be warranted, and may also help to encourage other species (other than aliens) to utilise these habitats as habitat conditions improve.

Apex Predators (Murray cod)

Murray Cod are known to spawn irrespective of flow conditions (Humphries 2005) and peaked in abundance and biomass during drought conditions. The susceptibility to hypoxic blackwater water was obvious, but it remains unknown whether the species would have benefited from the flood, and subsequent wetland inundation, if water quality did not deteriorate. Successive years of environmental water delivery targeted at Murray Cod recruitment in the upper zone may have contributed to species persistence. Ongoing spawning and recruitment of populations in the upper zone will be critical to support population expansion back into the lower and middle zones. Flow delivery for apex predators should therefore aim to support maintenance of nest sites, connection of critical habitat, support low flow refuges (such as the Wakool reserve) and also promote opportunities for larval and adult dispersal.

Flow dependant specialists

Flow dependant specialists were widespread throughout the system (mainly in channel habitats) and maintained populations, even following the system-wide hypoxic blackwater event. However, little recruitment has been observed in this group.These species are thought to have flexible life history strategies, and it has been hypothesised that they spawn only in years where conditions favour post-larval recruitment (Mallen-Cooper and Stuart 2006). It is likely that conditions were not suitable during the flooding and subsequent hypoxic blackwater event. The lack of evidence for recent recruitment suggests that watering strategies have, so far, been insufficient to satisfy this life history need. To date, juveniles have been collected only from wetland habitats in 2010. Maintenance of refuges will be required to provide benefits to this group whilst specific flow requirements for this gorup are further investigated and refined.

Floodplain specialists

Due to the complete absence of floodplain specialists, environmental water delivery for this group should be focused on restoring the habitat for this group within a number of target wetlands (e.g. Werai zone). Once adequate habitat is established, the group could be re-introduced through a conservation stocking program and these wetlands maintained through environmental water delivery (Baumgartner et al. 2013). The Werai zone shows great potential for such a targeted program as it is already able to support high abundances of some small-bodied flow generalist fish after several flow event years, and historically would have contained permanent and semi-permanent wetlands holding floodplain specialists. The re-establishment of floodplain specialists should form a part of the future research needs within the system.

Aliens

The alien group (mostly carp and goldfish) dominated biomass throughout the entire system in all years, zones and habitat types. Biomass and abundance peaked after flooding, and a major challenge for environmental watering is to avoid increases in alien fish populations, but this should not be to the detriment of environmental watering for native fish species. Even though there was prolific carp recruitment in 2011, subsequent re-wetting of wetlands including Werai zone did not result in the same recruitment success in this species (section 7.5.2). In 2012-13 carp were restricted to mostly short home range movements with few individuals moving large distances (section 7.5.1), however, during flooding the carp population moved significant distances and system wide recruitment resulted. The lack of substantial movement or recruitment during periods of targeted environmental watering is encouraging from a carp management perspective. It demonstrated that outside of large flood events, there was observed declines in overall biomass of aliens. Longer term confirmation of these trends during future watering events will provide management options to minimise further expansion of carp throughout the system.

General Conclusion

Annual sampling of fish populations within the Edward-Wakool has already contributed to the management of environmental water. In addition it has provided information on community changes in response to changes from drought to flooding and hypoxic blackwater events of 2010-11. The data demonstrate a consistent and system wide reduction in carp biomass since flooding. It is also helping to understand recolonisation pathways for Murray cod following hypoxic blackwater. The annual fish sampling program will help guide long term environmental flow delivery strategies and help to improve overall ecosystem health in the Edward-Wakool system, whilst also assessing the incremental value in annual water deliveries for fish recruitment and abundance.

Synthesis

Objective 1: Support habitat requirements of native aquatic species, maintain health of existing extent of riparian, floodplain and wetland native vegetation communities

Summary of main findings

The Yallakool Creek environmental watering from 19 October 2012 to 7 December 2012 was held in the range of approximately 360 ML/day to 683 ML/day during this period with a median discharge of 526 ML/day. This objective of this event was to maintain inundation of habitat for Murray cod nests and maintain the flow until cod eggs could hatch and drift downstream. In Yallakool Creek a modelled discharge scenario of 560 ML/day resulted in a 22% increase in wetted benthic surface area relative to the base flow 170 ML/day scenario. There was a significant increase in submerged aquatic vegetation during this event, however the increase was short lived, because when the water levels were reduced at the end of the environmental watering event the submerged aquatic vegetation was fully exposed and vegetation was desiccated.

The Colligen Creek environmental watering in November and December 2012 consisted of two freshes to promote golden perch and silver perch spawning. The first fresh reached a peak of approximately 903 ML/day and the second fresh reached a peak of approximately 655 ML/day. In Colligen Creek a modelled environmental flow of 800 ML/day resulted in a 14% increase in wetted benthic surface area relative to the base flow 200 ML/day scenario. These events did not result in any noticeable change in submerged aquatic vegetation, which is not unexpected as the objective of the environmental watering was for short duration of peaks to encourage movement and spawning of golden perch and silver perch.

The Yallakool Creek environmental watering action in February 2013 commenced on 1 February 2013 and finished on 23 February 2013. The fresh increased over 3 days to a peak of 430 ML/d on 13 February 2013. This event was aimed to provide opportunities for small bodied fish to breed. A discharge of 430 ML/day in Yallakool Creek has not been modelled, but is between the modelled median discharge (271 ML/day) and environmental watering modelled discharge (560 ML/day). Based on this, the wetted benthic surface area on the peak of the environmental watering is estimated to have increased between 9.5% and 22% relative to the base flow 170 ML/day scenario. These events did not result in any reportable change in submerged aquatic vegetation.

A watering action in Yallakool and Colligen Creeks commenced on 13 March 2013 and ceased on 5 April 2013. In Yallakool Creek there was a one day duration peak of 563 ML/d and in Colligen Creek a one day duration peak of 499 ML/d. The objective of these watering actions was to test whether a spawning response could be achieved in small bodied fish from a 30 cm rise in water levels in autumn. A secondary objective was to test whether or not a small water level rise would result in the movement of medium/large bodied fish during autumn. The peak of the environmental watering in Yallakool Creek is estimated to have resulted in a short-lived increase in wetted benthic surface area of 22% relative to the base flow 170 ML/day scenario. A discharge of 499 ML/day in Yallakool Creek was not modelled but is between the modelled median discharge (314 ML/day) and estimated half bank discharge (800 ML/day). Based on this, the wetted benthic surface area on the peak of the environmental watering is estimated to have increased between 3 % and 14 % relative to the base flow 200 ML/day scenario. There was no reportable change in submerged aquatic vegetation associated with this action.

The modelled wetted benthic surface area during the maximum daily discharge of the unregulated flows in Colligen Creek (2808 ML/day on 2/8/2012) and Yallakool Creek (1913 ML/day on 2/8/2012) was considerably higher than that during the base flow or environmental watering scenarios in these rivers. On the peak of this unregulated flow event the modelled wetted benthic surface area relative to the base flow scenario increased by 47.8% in Colligen Creek and 58.9% in Yallakool Creek. During these events there would have been a considerable increase in inundated riverbank vegetation. The modelled bankfull scenarios produced even larger increase in wetted benthic surface area than the unregulated flows.

Conclusion

In 2012 the CEWO determined that mid-Murray region was considered to be in a moderate to-wet resource availability scenario and the catchment condition was considered to be wet (CEWO 2012). Under that scenario the ecological management objectives were expected to be in the moderate to wet range. The objective of the moderate range is to ‘maintain ecological health and resilience to improve the health and resilience of aquatic ecosystems’ and the objective for the wet range is to ‘Improve the health and resilience of aquatic ecosystems’ (CEWO 2012). The environmental watering action in Yallakool Creek from October to December 2012 was of sufficient duration that it achieved the objective of maintaining inundation of habitat for Murray cod nests and maintaining the flow until cod eggs could hatch and drift downstream. The duration of inundation provided the opportunity for submerged vegetation to significantly increase in area and there was a visible increase in the activity of macroinvertebrates and other organisms in this newly grown vegetation. However, the benefits of this increase in submerged aquatic vegetation were short lived, because when the water levels were reduced at the end of the environmental watering event the aquatic vegetation was fully exposed and submerged vegetation was desiccated.

The environmental watering actions in Yallakool Creek in February 2013 and in Yallakool Creek and Colligen Creek in March 2013 were targeted to provide opportunities for small bodied fish to breed. It is unlikely that the short duration of the peak of these events would have been sufficient to create suitable breeding and nursery habitat for small bodied fishes. However, these events would have wetted the river banks and this may have made some contribution to sustaining riverbank plants that would provide important habitat during any subsequent higher flows.

An important finding is that the relationship between discharge and wetted benthic surface area in these rivers is not linear. For example, an increase from 200 ML/day to 800 ML/day in Colligen Creek resulted in a modelled 14% increase in wetted benthic area, whereas an increase from 170 ML/day to 800 ML/day in Yallakool Creek resulted in a modelled 30% increase in wetted benthic area. It is likely that that the effect of environmental watering on the habitat of native aquatic species is strongly influenced by the geomorphology of the rivers in the Edward-Wakool system.

From the data presented in this report it can be concluded that the 2012-13 environmental watering in the Edward-Wakool system contributed to maintaining the habitat of native aquatic species. However, there is no evidence that the environmental watering improved the habitat requirements of native aquatic species, other than a short-term improvement in submerged aquatic habitat in Yallakool Creek from October to December 2012.

Suggestions for future monitoring and assessment

The modelling of in-channel habitat inundation area provides critical information to allow integration of results across a range of indicators. Inundation modelling for additional flow scenarios and over a longer river distance would facilitate better planning for the magnitude and duration of environmental watering events. It would also assist in the interpretation of monitoring results and enable better links among parameters indicators. Extending the monitoring of habitat parameters to additional sites across the catchment would facilitate assessment of longer term condition trend in this system.

Objective 2: Support ecosystem functions that relate to mobilisation, transport and dispersal of biotic and abiotic material

The overarching conclusion from the monitoring of water chemistry (nutrients, organic carbon) and stream metabolism is that throughout the entire period, encompassing base flow and freshes, water quality posed no threat at all to these aquatic ecosystems. At no stage did the dissolved oxygen concentration fall below 4 mg O2/L, which may threaten viability of invertebrates and fish communities. There were no ‘blackwater events’, which usually arise from water from inundated floodplains, devoid of oxygen, flowing back into the stream channels, and this is more commonly a problem when combined with high water temperatures. The hydrograph peaks throughout 2012-13 were insufficient for such inundation to occur. The following synthesis must be read with a large caveat: “But the water did not get onto the floodplain”. The possible outcomes of floodplain inundation are discussed at the end of this section.

With the exception of NOx on one occasion, bioavailable nutrient concentrations, ammonia, filterable reactive phosphorus (FRP) and NOx (nitrate plus nitrite) remained below the ANZECC Trigger concentrations of 20, 20 and 40 μg/L respectively. The organic carbon concentrations in this system can be subject to large fluctuations especially during periods of high flow, however the normal baseline concentrations observed during the study period are towards the lower end of those normally observed in lakes and rivers- generally in the range 2-10 mg/L (Morel and Hering 1993). The ANZECC (2000) water quality guidelines do not provide trigger levels for total organic carbon and dissolved organic carbon, reflecting the fact that there is great diversity in the ‘normal’ concentrations of organic carbon between ecosystems and also that there can be considerable variation in the chemical and biological reactivity of the mixture of organic compounds making up the DOC and TOC at a particular site.

The 2012-13 monitoring data show that there were no large nutrient and organic carbon additions, at least of sufficient duration to be detected by the fortnightly sampling. Some addition of organic carbon occurred during the unregulated flow in August 2012, although much of this may have occurred upstream of the study sites as the water quality in the study rivers was similar to that of the source water from the Edward River. Larger increases in Little Merran Creek and the addition of some bioavailable nutrients in this river may have been due to a combination of different water source and greater area of inundated riverbank in this system. This highlights the value of connection with the floodplain for the mobilisation of carbon and nutrients in these lowland river systems. One of the anticipated ecological benefits of freshes is the delivery of bioavailable materials (nutrients, carbon) to stimulate plant growth and hence enhance the food supply to higher level organisms (notably fish). Elevated nutrient and organic carbon concentrations generally originate from the floodplain and benches in the stream channel above the usual water level can also be repositories of organic C and nutrients. However, there were no large fow events or overbank flows and hence no large increases in nutrient and organic carbon were recorded during 2012-13.

Rates of gross primary production (GPP) and ecosystem respiration (ER) in these streams were typical of lowland streams with good water quality. These rates were constrained by some of the key driving variables. GPP rates were controlled by a combination of very low nutrient concentrations and strong light attenuation limiting the biomass of primary producers. This turbidity-induced light attenuation means that any stream reach deeper than about 1.0 m (Little Merran Creek) to 1.5 m (Colligen Creek) will not have sufficient light reaching the sediment to sustain biofilm growth.

The key point from all of the water quality measurements in 2012-13, is that the nutrient and organic carbon concentrations and rates of primary production and organic matter processing were sufficient to provide the energy base to support the the recruitment of small bodied fish and apex predators such as Murray Cod. However, the question whether fish populations would increase further with higher concentrations and rates can only be determined by further monitoring after flow events that bring higher nutrient and organic carbon concentrations from the floodplain to the river channel.

In the absence of freshes that inundated the floodplain, the effect of such flows on water quality in the study streams remains a matter of conjecture. It is highly likely that water flowing back to the stream channel from the floodplain would be much higher in nutrients and organic carbon than exist in the stream during in-channel flows. Decisions regarding the timing of any proposed large freshes should take into account the prospect for creating ‘blackwater events’. The severity of blackwater events would be highly impacted by the timing of flow through the dependence of a number of the key processes on water temperature (Howitt et al. 2007). High flows or overbank flows during periods of low water temperature are less likely to result in hypoxic blackwater events due to slower rates of organic carbon leaching and microbial metabolism, coupled with higher oxygen solubility, however, consideration also has to be given to the period of time the water will be sitting on the floodplain.

Objective 3: Support breeding and recruitment of frogs, turtles, invertebrates

This objective was assessed through monitoring frogs, shrimp and zooplankton. Turtles were not assessed as part of this monitoring and evaluation program in 2012-13.

Summary of main findings

The watering actions in Colligen and Yallakool Creeks during the 2012 – 2013 sampling period had no effect on the abundance of zooplankton, including individual size classes of zooplankton, nor did it appear to stimulate reproduction. Zooplankton abundance was instead highly seasonal, affected by factors unrelated to flow, such as temperature. It is possible that the magnitude of the watering actions were insufficient to inundate habitat and stimulate productivity thereby increasing abundance and taxonomic diversity of zooplankton. Connectivity and upstream sources may also play a role in zooplankton abundance, as suggested by the similarity in zooplankton abundance of Colligen Creek and Yallakool Creek to the Edward River.

The abundance of shrimp during 2012 – 2013 was not significantly different across the four rivers. The timing of shrimp spawning also did not seem to be influenced by environmental watering. In fact, Colligen Creek and Yallakool Creek had less shrimp overall compared to the control rivers, perhaps as a result of increased flows reducing the size and availability of slackwaters crucial to larval development and juvenile recruitment. In addition, sharp increases in flow may result in higher larvae mortality. The magnitude of flow provided by the watering actions in Colligen Creek and Yallakool Creek may have decreased the size and availability of slackwaters. Larger flows reaching near bankfull levels are likely to inundate benches and create a greater area of slackwaters, resulting in a general increase in shrimp habitat availability.

There was little response of frogs to watering actions in Colligen and Yallakool Creeks. In Colligen Creek frog calling increased when flows were sufficiently high to inundate vegetation. However, no response to environmental watering was evident in Yallakool Creek. The lack of frog response in Colligen and Yallakool Creeks to the environmental freshes (and the other focus reaches which did not receive freshes) may be due to the unprecedented hot weather and low rainfall conditions experienced in the region at this time. Furthermore, riverbank cover was low in each focus reach, which potentially reduced suitable habitat available for frogs and may also explain the lack of a response to watering actions.

Conclusions

The watering actions in Colligen and Yallakool Creeks during 2012–2013 had little influence on the abundance of zooplankton, shrimp and frogs. In addition, few changes occurred in the ‘control’ rivers at the time of the watering actions, suggesting the objective of ‘maintaining ecological health and resilience’ in the mid-Murray region under the moderate resource availability scenario was not achieved. Under the experimental design used here, we would expect to see a decline in condition (in this case abundances of zooplankton, shrimp and frogs) in rivers not receiving environmental water compared to those receiving environmental water, if ecological health and resilience had been maintained by the environmental watering. The suggestion that slackwater habitats for shrimp were possibly reduced by these watering actions needs to be considered when future watering actions are planned.

The objective to ‘Improve the health and resilience of aquatic ecosystems’ under the wet resource availability scenario may be expected to translate to higher abundances and evidence of breeding among zooplankton, shrimp and frogs in rivers receiving environmental water compared to control rivers. This was not observed during 2012-2013, therefore this objective was not achieved. The primary reason for this appears to be the lack of inundation of habitat areas (slackwaters and inundated vegetation) during watering actions. Freshes remained in-channel and did not reach sufficient heights to introduce nutrients and stimulate zooplankton production, nor inundate vegetation and large slackwaters to create habitat for frogs and shrimp respectively. More work is needed to model habitat inundation under a range of flow scenarios to assist the planning of future watering actions.

Objective 4: Support ecosystem functions that relate to longitudinal connectivity and lateral connectivity to maintain populations

Longitudinal and lateral connectivity is important for supporting ecosystem functions and is critical for the long-term survival of populations of fish and other aquatic organisms. The Edward-Wakool system recedes to a series of pools during low flow conditions, reconnecting when flows resume. During dry periods, off channel wetlands are regularly disconnected from main channel habitats but become connected during higher flow events. The flow regime in the Edward-Wakool system has been significantly altered by river regulation (Green 2001; Watkins et al. 2010), with changes to the timing and volume of flows. Environmental watering actions can help to increase longitudinal and lateral connectivity within this system. When connectivity is restored to aquatic ecosystems fish and other organisms can move between habitats to feed, reproduce, colonise new habitats, and avoid unfavourable conditions. Longitudnal and lateral connectivity is also vital for healthy ecosystem functions by facilitating the transport of nutrients and carbon and thus supporting primary production that underpins food webs.

Summary of main findings

The environmental watering actions in 2012-13 did not result in a significant increase in lateral connection or inundation of riverbank habitat and backwaters (objective 1). Consequently, during the 2012-13 environmental watering actions there were no significant input of nutrient and organic carbon, or detectable change in ecosystem metabolism (objective 2) or zooplankton abundance (objective 3). This suggests that the environmental watering actions did not produce significant ecological changes that would be expected if there was increased lateral connectivity and inundation of backwater habitats. There was evidence of some increased lateral connection during the unregulated flows in August 2012, and there was an increase in nutrients and carbon observed at this time. This highlights the value of lateral connection with the floodplain for the mobilisation of carbon and nutrients in these lowland river systems, The rates of primary production appeared to be sufficient to provide the energy base to maintain the fish population in this system in the absence of significant increased lateral connections. However, the question whether fish populations would increase further with higher primary production rates remains, and can be answered only by further monitoring of flow events that result in significant lateral connection.

The environmental watering actions in the Edward-Wakool system in 2012-13 had an effect on the longitudinal movements of four large-bodied fish species that were monitored in the Wakool River, Yallakool Creek and the Edward River using acoustic telemetry. However, these longitudinal movements did not result in detectible differences in spawning or recruitment based on our sampling of larvae and juveniles. Movement of fish in Colligen Creek could not be assessed due to the absence of an acoustic array and tagged fish in this system.

All four tagged fish species responded quickly to changes in flow, displaying increased activity with increasing temperature and flow during spring and early summer. This period of increased activity corresponded with spawning periods for these species. Fish displayed high site fidelity, and movement was linked to the hydrograph. Although all tagged species displayed increased activity in response to environmental delivery, this increased activity did not result in displacement movements, but may represent survival related behaviour, such as an increased feeding response.

Of the fish that moved in response to delivery of environmental water, all species, except Murray cod displayed a preference to move into the Yallakool Creek habitat and not into the Wakool River control reach. This could be a direct response to the environmental watering actions and change in longitudinal connectivity in this system or could be influenced by other factors or constraints affecting fish movement. In contrast, Murray cod made displacing movements during spring and early summer predominantly into the Wakool River. This pattern of movement was consistent with spawning related behaviour, with cod moving upstream prior to a sedentary period, suggestive of nesting, before returning down river. Back calculated spawning dates for young of year Murray cod recruits collected later in the year aligns with this period.

The results confirm that the Wakool Reserve is an important refuge pool in the region and is used by all fish species during periods of low flow. The flow regimes delivered in 2012-13 were sufficient for fish to leave the pool and access new habitat, and also to subsequently return when flows ceased. However, movement patterns in 2012-13 differed from previous years. A strong movement response similar to that observed during flooding in 2010 was not observed in 2012-13. Both the displacement distance, and the number of fish undertaking these movements was reduced in 2012-13. Knowledge of fish migration patterns, home range, spawning related activity and responses to environmental watering can provide information for improving the management of environmental watering actions and management of native and alien fish and aquatic habitat.

Objective 5: Support breeding and recruitment of native fish

We use a multiple lines of evidence approach (Downes et al. 2002), to assess whether the environmental watering actions delivered from October 2012 to April 2013 to rivers in the Edward-Wakool system achieved the CEWO objectives of supporting the breeding and recruitment of native fish. Specific aims of the watering actions included eliciting a spawning response from golden and silver perch, enhancing the nursery and dispersal conditions of Murray Cod larvae, and promoting spawning and recruitment of small-bodied fish. We investigated if the watering actions led to a) spawning related movement of flow specialists such as golden and silver perch, b) increases in the abundance of eggs and larvae of the fish species during or immediately after water actions, and c) an increase in the number of 0+ recruits of fish species in rivers that received the watering actions.

Edward-Wakool fish assemblage

Ten species of native fish have been identified in the Edward-Wakool river system since 2010. Using the presence of larvae and juveniles, we detected that eight of these species spawned, and spawning was independent of most environmental watering actions (Table 33). Overall, the hydrological conditions were very similar to 2011-2012, where the majority of fish species also spawned (Watts et al. 2013). While it is encouraging to note that most of the native fish species populations in the Edward-Wakool system appear self-sustaining, it is important to keep in mind that the diversity of composition of the community is no doubt greatly reduced as a result of river regulation, and therefore, it may well only be the species that have life histories amenable to persisting in highly regulated systems, that are currently present in the area.

Worthy targets of restoration should aim to consider the promotion of environmental (including hydraulic) conditions previously present within the system, as well as to increase locally extant populations. While historical records for the Edwards-Wakool river system are elusive, it is likely to have supported a similar fish community to that found upstream in the Barmah-Millewa area of the Murray River, and downstream in the Koondrook-Pericoota region. For example, King et al. (2009) cites that the Barmah-Millewa region is home to 18 native fish species, nearly twice the number currently found in the Edward-Wakool system to date. Species presumably once present Edward Wakool region may no longer be present because changes in hydrological conditions as a result of many years of river regulation leaving them unable to persist. However, they should not be forgotten when setting restoration targets and goals for what environmental watering actions might be able to achieve. As asserted by King et al. (2009a), the regulation and reduced variability in flows as a result of regulation will inevitably lead to a reduced fish fauna that reflecting a more homogenous subset of life history strategies and flow requirements. It is vital therefore that planning for environmental watering actions encompassing the diversity of environmental conditions required by different fish species, as well as other key taxonomic groups.

Table 33. Multiple of lines of evidence results to assess the effect of 2012-2013 environmental watering actions on the breeding and recruitment of native fish present in the Edward Wakool system. Yellow – no response, green – significant response, white – response not assessable. A= adults, L/J = larvae/juveniles.

| Flow guild | Species | Collected in 2012-13 |  | Response to environmental watering actions |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | A | L/J | Fish spawning movement | Fish larvae & eggs | Fish recruitment |
| Native species Mode 1 | Murray cod |  |  | no | no | no |
|  | River blackfish |  |  | not targeted | low abundance not assessable | low abundance not assessable |
| Native species Mode 2 | golden perch |  |  | no | no | low abundance not assessable |
|  | silver perch |  |  | no | no | low abundance not assessable |
| Native species Mode 3 | Australian smelt |  |  | not targeted | no | no |
|  | carp gudgeon |  |  | not targeted | yes | yes |
|  | flathead gudgeon |  |  | not targeted | no | low abundance not assessable |
|  | Murray river rainbowfish |  |  | not targeted | low abundance not assessable | low abundance not assessable |
|  | unspecked hardyhead |  |  | not targeted | low abundance not assessable | low abundance not assessable |
|  | bony herring |  |  | not targeted | not collected | low abundance not assessable |
| Alien species | carp |  |  | no | no | no |
|  | goldfish |  |  | not targeted | not collected | no |
|  | gambusia |  |  | not targeted | low abundance not assessable | low abundance not assessable |
|  | redfin |  |  | not targeted | not collected | low abundance not assessable |
|  | oriental weatherloach |  |  | not targeted | low abundance not assessable | not collected |

Golden and silver perch

Using multiple lines of evidence, we conclude that environmental watering actions did not result in a spawning outcome for golden and silver perch. No large-scale increase in upstream displacement movements (pre-spawning migration) by golden perch were detected, (though there was an increase in the proportion of tagged fish moving from the refuge pool into Yallakool Creek during all three environmental watering events). Movement in Colligen Creek could not be assessed due to the absence of an acoustic array. It is not known if any of these movements were spawning related behaviour because most adult golden perch migrated downstream outside the larval monitoring zone. No eggs or larvae of silver or golden perch were found during or immediately after the watering actions, and no young-of-year were collected from any of the focus rivers or community sampling sites. These findings repeat those reported in 2011-2012, and possible reasons for the environmental watering actions failing to result in a golden or silver perch spawning event are discussed below.

Golden and silver perch are considered to be flow recruitment specialists because they are found to spawn on flow pulses that occur either within-channel or overbank (Mallen-Cooper and Stuart 2003; Roberts et al. 2008; King et al. 2009; Zampatti and Leigh 2013). The seasonal timing, duration and magnitude of these rises are three important factors thought to determine the degree of spawning and consequent recruitment for these species (Mallen-Cooper and Stuart 2003; King et al. 2009; Zampatti and Leigh 2011). Even though temperatures were adequate for golden and silver perch to spawn (>18°C), the 2012-2013 environmental watering in the Edward-Wakool system did not result in spawning relating movements. Mallen-Cooper and Stuart (2003) found that strong year-classes of golden perch were found in the mid-Murray when variability of within channel flows in late spring and summer was greatest, with water levels increasing one to two metres. Further, golden perch were found to spawn and recruit on flows where discharge exceeded 70% of bankfull conditions. King et al. (2009b) monitored the spawning and recruitment of golden and silver perch in the Murray River in the Barmah-Millewa from 2003-2006 and found that low levels of spawning could occurr for both species during within channel high flows, but that a large increase in spawning intensity occurred as a result of an overbank flood in 2005. The spawning responses to the within channel flows occurred when discharge was at near bank full levels (King et al. 2009b). In the lower Murray, South Australia, Zampatti and Leigh (2013) found strong recruitment of golden perch followed spring-summer spawning when water temperatures would have exceeded 20°C and magnitude of peak flow in which fish larvae were found occurred on a pulse with discharge levels 5 times greater than at the start of the pulse, and took three months to reach is peak. In contrast, environmental water delivered to the Colligen in November 2012 peaked at 903 ML/day, equating to only 22% of the discharge needed to reach bankfull, and took 6 days to reach its peak. While additional work is required to identify the most appropriate flows needed by golden and silver perch spawn and recruit in significant numbers in this region, it is likely that the magnitude and rate at which the pulse of environmental water is ramped up will need to be much greater than what has been delivered to date. Also the timing of flows needs to be closely considered.

Murray cod

There was no evidence that the nursery and larval dispersal conditions of Murray cod were enhanced as a result of the November 2012 Yallakool watering action. The watering action was provided to maintain the water level heights moving through Yallakool Creek during the Murray cod breeding season. Murray cod did not utilise Yallakool Creek habitat in response to environmental water delivery, however increased activity within the Yallakool - Wakool confluence refuge pool was detected. Increased activity may be indicative of a feeding response to flow delivery. Murray cod larvae and young-of-year recruits were found in Yallakool Creek, but not significantly more than the rivers that did not receive environmental watering actions.

The findings from this study support the large body of literature that now exists which consistently conclude that Murray Cod spawn at peak times in November-December, and regardless of flow conditions (Rowland 1983; Humphries et al. 2002; Gilligan and Schiller 2003; King et al. 2003; Humphries 2005; Koehn and Harrington 2005; King et al. 2009b). While flow conditions may not influence Murray cod spawning, speculation exists that recruitment of Murray cod can be greater in times of high flows, however evidence for this remains inconclusive (Rowland 1998; Ye et al. 2000, King et al. 2009b). For increased recruitment to occur as a result of environmental flows, a significant increase in habitat and food resources must be provided with the delivered flows, at a level substantial enough to lead to increased survivorship and growth of larvae and juveniles (King et al. 2009b). We hypothesised that watering actions delivered in the Yallakool in November 2012 would maximise recruitment by inundating more nursery habitats. This hypothesis could not be supported, as the number of 0+ recruits sampled in the Yallakool at the end of the study period was not significantly different than in other rivers that did not receive any environmental water. The lack of enhanced recruitment in Murray Cod in the Yallakool could be due to two possible reasons – firstly, inundation modelling showed that the Yallakool flows lead to only a small (22%) increase in wetted benthic surface area, suggesting that nursery and habitat areas were not increased significantly, and secondly, there was no significant increase food resources (zooplankton) for larval and juvenile cod as a result of the watering actions. We recommend that future watering actions targeted at enhancing native fish populations must take into consideration the importance of promoting in-stream productivity in order to provide a food rich environment essential for larval and juvenile growth, survival and ultimate recruitment.

Small bodied fish

There was little response from small-bodied fish to the environmental water actions. However, the spawning and recruitment of carp gudgeon, one of the five small bodied fish species found in the Edward-Wakool system, benefited from the early season environmental watering actions that took place in the 2012-2013 watering year. There were greater numbers of larvae found after the first November environmental water action in Colligen Creek compared to the other rivers. The number of recruits was also significantly higher in Yallakool Creek and Colligen Creek immediately after the November environmental watering actions compared to rivers that did not receive environmental water. However, the number of recruits in the Edward River, the source of the environmental water, was equally as high as those found in the Yallakool and Colligen rivers during this time. Therefore, it is not possible to conclude if the increase in numbers of recruits in the Yallakool and Colligen is due to increased recruitment occurring within these rivers themselves, or is due to the more recruits moving into these systems from the Edward River. Carp gudgeon are commonly found to be one of the most numerically abundant species across the Murray Darling Basin, and its prevalence throughout the system suggest this species flourishes under regulated conditions (Humphries et al. 1999).

Larvae of small-bodied, seasonal spanners like flathead gudgeon, unspecked hardy head and Murray River rainbow fish were present in the Edward-Wakool region, but in very low numbers. While this could be due to sampling bias, studies by Meredith et al. (2002) have found these species can be collected in much higher numbers with quatrefoil light traps, and therefore, it may be that the low numbers of larvae collected for these species, is an indication that these species currently have relatively low population sizes in the Edward-Wakool system. Fish species that occur in low populations may do so because they persist in non-ideal current hydrological and habitat conditions, and understanding their ecology is vital if their populations are to be encouraged to flourish with environmental flows. For example, macrophytes and submerged littoral vegetation are important habitat spawning substrate for numerous native species, including unspecked hardyhead and Murray river rainbowfish (Llewellyn 1970). The use of environmental flows to promote the establishment and persistence of macrophytes in the Edward-Wakool river system, is therefore a worthy restoration goal, and likely to have significant and positive impact on the size of numerous resident native fish populations.

Role of flow for enhancing fish recruitment

Understanding the causal mechanisms which underlie recruitment strength is critical for effective conservation of fish (Fuiman and Werner 2002). One of the dominant fish recruitment hypotheses relevant to freshwater fish is the “Match/Mismatch” hypothesis; which proposes that fish recruitment will be high if the peak of the prey availability temporally matches the peak in larval production, while a mismatch will lead to poor recruitment (Cushing 1990). Zooplankton is the major food item for larval fish and alignment between zooplankton abundance and larval fish abundance may be an important determinant of recruitment strength (Cushing 1990; King 2003; Humphries et al. 2013). The use of environmental watering actions to promote strong zooplankton production, during periods when larval production is high, may be key to enhancing strong recruitment levels of fish populations in the Edward Wakool. Comparisons of recruitment levels across years, and across rivers, under different magnitudes and flow durations, will provide a greater understanding of how zooplankton densities shape the survival and subsequent recruitment of native fish in the area, and the role of flow in shaping these recruitment events.

Conclusion

In summary, in 2012-13 there were a small number of significant responses to the October to December 2012 in-channel environmental watering, however some of the expected benefits of environmental watering were not observed. This is consistent with the findings from the 2011-12 environmental watering in this system (Watts et al. 2013). In contrast, there were almost no significant responses to the February, March and April environmental watering events in Colligen Creek and Yallakool Creek. The environmental watering in the Edward-Wakool system in 2012-13 achieved the objective of maintaining the ecological health and resilience of the ecosystem but the timing, magnitude and duration of watering events were not sufficient to make a significant improvement in the the health and resilience of the aquatic ecosystem in the Edward-Wakool system.

Environmental watering recommendations

The possibility of a wet to moderate resource availability scenario meant that ecological objectives for environmental water use in the Edward-Wakool system in 2012-13 were expressed in terms of improving ecological outcomes (CEWO 2012). However, above average temperatures and below average rainfall in 2012-13 meant that environmental water largely contributed to maintaining ecological outcomes, consistent with a moderate resource availability scenario.

The monitoring and evaluation of environmental watering in the Edward-Wakool system in the 2012-13 showed that the objective of improving ecological outcomes was generally not achieved, however, in some cases there is evidence that the environmental water contributed to maintaining ecological outcomes.

Recommendations relating to the timing of environmental watering

- To achieve the objective of ‘improving the health and resilience of aquatic ecosystems’, environmental watering actions under water use option 1 should be targeted during spring and early summer. This is the time of year when the greatest benefit for spawning and recruitment of most aquatic species can be realised. An additional benefit of undertaking environmental watering in spring and early summer is that it is less likely to trigger water quality issues than environmental watering undertaken in late summer or early autumn when water temperatures are higher and the concentration of dissolved oxygen is lower. Furthermore, delivery of environmental water in spring or early summer may be more straightforward to implement given existing operational constraints, as it can be difficult for river operators to meet all license holders water needs in the Edward-Wakool system during late summer and early autumn during periods of high consumptive demand.

- Environmental watering actions under water use option 2 (Edward-Wakool system refuge habitat) can be implemented at any time of the year to avoid damage to key assets and provide refuge during hypoxic blackwater events. If there is a high likelihood of a blackwater event, the ecosystem can benefit from environmental watering to provide refuges that have a higher concentration of dissolved oxygen. This option was implemented in 2011-12.

- Decisions involving the timing of environmental watering should consider the water temperature at the proposed time of of the environmental watering actions because it will strongly influence the success of fish spawning, the risk of hypoxic blackwater events and rate of ecosystem productivity.

Recommendations relating to the magnitude of environmental watering

- To achieve the objective of ‘improving the health and resilience of aquatic ecosystems’ under water use option 1, the magnitude of environmental watering freshes should be larger than environmental flow actions delivered in 2012-13. In contrast to the small or absent responses to environmental watering in Yallakool Creek and Colligen Creek in 2012-13, the larger magnitude unregulated flow events in August and September 2012 inundated a significantly larger area of riverbank and triggered an increase in river productivity. Better ecological outcomes could be achieved through delivery of environmental freshes of sufficient magnitude to inundate low lying benches and backwaters and create shallow water habitat and slackwaters and inundate riverbank vegetation. Additional modelling of inchannel inundation should be undertaken during the planning of environmental watering actions to assist with optimisation of flow magnitude to help achieve watering objectives and maximise the creation of shallow inundated areas. It is possible that for a given volume of environmental water, a better ecological outcome could have been achieved in 2012-13 by delivery of fewer larger freshes rather than several smaller freshes, however this hypothesis needs to be tested in an adaptive management context.

- Smaller magnitude freshes (such as those delivered in 2012-13) can be delivered to achieve the ecological objective of ‘avoid damage’, or to ‘provide capacity for recovery or maintain health’. When smaller magnitude watering actions are being planned and implemented it is important that realistic watering objectives are set for each watering action.

- A comprehensive community engagement program will be required to facilitate the delivery of larger environmental freshes to the Edward-Wakool system. The in-channel inundation modelling, mentioned in recommendation 4, will enable scenarios to be presented to stakeholders prior to implementation, which will help identify and minimise risks and serve to inform and engage stakeholders in the planning process.

Recommendations relating to the duration of environmental watering

- The duration and shape of the hydrograph of environmental watering events should be carefully planned to avoid rapid rates of recession to minimise stranding of aquatic biota and desiccation of newly established submerged plants. For example, future watering actions could include a longer, gradual recession to ensure a portion of newly established submerged habitat remains inundated and has the opportunity to increase in area following the flow recession. This will also ensure that organisms utilising the inundated shallow areas have sufficient time to return to in-channel habitats and avoid stranding.

- There is a need to outline multiple objectives when setting the duration of environmental watering events. For example, the objective of the environmental watering event in Yallakool Creek from October to December 2012 was targeted for Murray cod but was also of sufficient duration that it resulted in a significant increase in aquatic vegetation that was not observed during the shorter duration environmental watering events in Colligen Creek or Yallakool Creek later in the year.

General recommendations for environmental watering

- The quality of the source water should be carefully considered prior to each environmental watering action as it will influence the outcome of environmental watering. The quality of source water for environmental watering actions in the Edward-Wakool system (e.g. the Edward River or Mulwala canal) can vary considerably. For example, on occasions when there is considerable overbank flooding in the Murray catchment, the water in the Edward River or the Mulwala canal may carry high dissolved carbon loads (Watts et al 2013).

- Decisions around the timing of environmental watering should consider the antecedent hydrological conditions because they can strongly influence the success of subsequent environmental watering actions.

Acknowledgements

We extend our thanks to the Wakool River Association, Edward Wakool Angler Association and landholders in the Edward-Wakool river system for allowing access to their properties and for their keen interest in this project. Field surveys and community liaison were supported by staff at the Murray CMA, with particular thanks to Josh Campbell. Vincent Kelly at the State Water Corporation provided information n on river and escapes discharge and Paula D’Santos for OEH provided information on watering actions. We thank James Abell, Randy Adjonu, Clare Baxter, Josh Campbell, Jonathon Doyle, Chris Ensor, Paris Grant-Preece, Jamie Hutchinson, Nick Ingram, Luke Jones, Tim Kaminskas, Kate Martin, Rohan Rehwinkel, Jack Simpson, Chris Smith, John Stuart and Emma Wilson for undertaking the field and/ or laboratory work. Maps presented in this report were prepared by Simon McDonald and Deanna Duffy (Charles Sturt University Spatial Analysis Unit), Rod Martin (NSW DPI) and Ian Wooden (NSW DPI). Inundation modelling was undertaken by Hugh Peterson. Tina Hines at the Monash University Water Studies Centre processed carbon and nutrient samples. Thanks to Dr Paul Humphries and Dr Nick Bond for reviewing the report and providing many helpful comments. Fish sampling was carried out under NSW Fisheries license (P11/0003-1.0) and approved by the CSU Animal Care and Ethics Committee (10/101). Frog and tadpole surveys were approved by The Animal Ethics Committee of the Chief Executive of the Office of Environment and Heritage (AEC approval number: 110912/03) and were approved under NPWS scientific license (SL100218). This project was funded by Commonwealth Environmental Water with in-kind contributions from Charles Sturt University, Murray Catchment Management Authority, Industry and Investment NSW, Monash University and NSW Office of Environment and Heritage. This research also received in-kind support (for R. Kopf) from the CSIRO Cluster Collaboration Fund and the Ecological Responses to Altered Flow Regimes Research Cluster which represents a collaboration between the CSIRO Water for a Healthy Country Flagship, Griffith University, the University of New South Wales, Monash University, Charles Sturt University, La Trobe University and the Arthur Rylah Institute of the Victorian Department of Sustainability and Environment.

References

Agostinho, A.A., Pelicice, F.M., Petry, A.C., Gomes, L.C., and Júlio Jr, H.F. (2007) Fish diversity in the upper Paraná River basin: habitats, fisheries, management and conservation. Aquatic Ecosystem Health & Management 10, 174-186.

Anstis, M. (2002) 'Tadpoles of South-eastern Australia: a guide with keys.' (Reed New Holland: Sydney, NSW)

ANZECC water quality guidelines (2000) Australia and New Zealand guidelines for fresh and marine water quality . In ‘Australian and New Zealand environment and conservation council and Agriculture and resource management council of Australia and New Zealand’. Available at http://www.mincos.gov.au/publications/australian_and_new_zealand_guidelines_for_fresh_and_marine_water_quality

Arthington, A.H., Bunn, S.E., Poff, N.L., and Naiman, R.J. (2006) The challenge of providing environmental flow rules to sustain river ecosystems. Ecological Applications 16, 1311-1318.

Arthington, A.H., Naiman, R.J., McClain, M.E., and Nilsson, C. (2010) Preserving the biodiversity and ecological services of rivers: new challenges and research opportunities. Freshwater Biology 55, 1-16.

Atkinson, B.L., Grace, M.R., Hart, B.T. and Vanderkruk, K.EN. (2008) Sediment instability affects the rate and location of primary production and respiration in a sand-bed stream. Journal of the North American Benthological Society 27, 581-592.

Baker, A. and Spencer, R.G. (2004) Characterization of dissolved organic matter from source to sea using fluorescence and absorbance spectroscopy. Science of the Total Environment 333(1), 217-232.

Balashov, D.A. and Recoubratsky, A.V. (2011) Hypoxia tolerance of hybrids of carp Cyprinus carpio and golden carp Carassius auratus. Journal of Ichthyology 51, 641-645.

Balcombe, S.R. and Closs, G.P. (2000) Variation in carp gudgeon (Hypseleotris spp.) catch rate in dense macrophytes. Journal of Freshwater Ecology 15, 389-395.

Baldwin, D.S. (1999) Dissolved organic matter and phosphorus leached from fresh and 'terrestrially' aged river red gum leaves: implications for assessing river-floodplain interactions. Freshwater Biology 41, 675-685.

Baldwin, D.S. and Mitchell, A.M. (2000) The effects of drying and re-flooding on the sediment and soil nutrient dynamics of lowland river-floodplain systems: a synthesis. Regulated Rivers: Research & Management 16, 457-467.

Baranyi, C., Hein, T., Holarek, C., Keckeis, S. and Schimer, F. (2002) Zooplankton biomass and community structure in a Danube River floodplain system: effects of hydrology. Freshwater Biology 47, 473-482.

Barnett, A., Abrantes, K.G., Stevens, J.D., Bruce, B.D., and Semmens, J.M. (2010) Fine-scale Movements of the Broadnose Sevengill Shark and Its Main Prey, the Gummy Shark. PLoS ONE 5(12), e15464. doi:10.1371/journal.pone.0015464

Basu, B.K. and Pick, F.R. (1997) Phytoplankton and zooplankton development in a lowland temperate river. Journal of Plankton Research 19, 237-253.

Basu, B.K. and Pick, F.R. (1996) Factors regulating phytoplankton and zooplankton biomass in temperate rivers. Limnology and Oceanography 41(7), 1572-1577.

Bateman, H.L., Harner, M.J. and Chung-MacCoubrey, A. (2008) Abundance and reproduction of toads (Bufo) along a regulated river in the southwestern United States: Importance of flooding in riparian ecosystems. Journal of Arid Environments 72(9), 1613-1619.

Baumgartner, L.J., Conallin, J., Wooden, I., Campbell, B., Gee R., Robinson, W.A. and Mallen-Cooper M. (2013) Using flow guilds of freshwater fish in an adaptive management framework to simplify environmental flow delivery of semi-arid riverine systems. Fish and Fisheries Early access DOI: 10.1111/faf.12023

Bertilsson, S. and Bergh, S. (1999) Photochemical reactivity of XAD-4 and XAD-8 adsorbable dissolved organic compounds from humic waters. Chemosphere 39(13), 2289-2300.

Biggs, B.J.F., Smith, R.A. and Duncan, M.J. (1999) Velocity and sediment disturbance of periphyton in headwater streams: biomass and metabolism. Journal of the North American Benthological Society 18, 222-241

Blom, C.W.P.M., Voesenek, L.A.C.J., Banga, M., Engelaar, W.M.H.G., Rijnders, J.H.G.M., Vandesteeg, H.M. and Visser, E.J.W. (1994) Physiological ecology of riverside species - adaptive responses of plants to submergence. Annals of Botany 74 (3), 253-263.

Blom, C.W.P.M. and Voesenek, L.A.C.J. (1996) Flooding - the survival strategies of plants. Trends in Ecology & Evolution 11 (7), 290-295.

Bogan, M.T. and Lytle, D.A. (2007) Seasonal flow variation allows 'time-sharing' by disparate aquatic insect communities in montane desert streams. Freshwater Biology 52(2), 290-304.

Bogenhuber, D., Linklater, D., Pay, T., Stoffels, R. and Healy, S. (2013) The Darling Anabranch Adaptive Management Monitoring Program Draft Report 2010–2013 Baseline to a decade. Draft Report prepared for the NSW Office of Environment and Heritage by The Murray-Darling Freshwater Research Centre, MDFRC Publication 11/2013, June, 171pp

Boon, P.I., Frankenberg, J., Hillman, T.J., Oliver, R.L. and Shiel, R.J. (1990) Billabongs. In: ‘The Murray’ (Eds N. Mackay and D. Eastburn), pp. 183-198. Murray Darling Basin Commission, Canberra.

Bormans, M., Ford, P.W. and Fabbro, L. (2005) Spatial and temporal variability in cyanobacterial populations controlled by physical processes. Journal of Plankton Research 27,61–70.

Bowen, Z.H., Bovee, K.D. and Waddle, T.J. (2003) Effects of flow regulation on shallow-water habitat dynamics and floodplain connectivity. Transactions of the American Fisheries Society 132, 809-823.

Brock M, Capon S, Porter J (2006) Disturbance of plant communities dependent on desert rivers. In 'Ecology of Desert Rivers’. (Ed. R Kingsford) pp. 100-132. (Cambridge University Press: Cambridge, UK).

Brown, P., Sivakumaran, K.P., Stoessel, D. and Giles, A. (2005) Population biology of carp (Cyprinus carpio L.) in the mid-Murray River and Barmah Forest Wetlands, Australia. Marine and Freshwater Research 56, 1151-1164.

Burns, A. and Ryder, D.S. (2001) The potential for biofilms as ecological indicators in Australian riverine systems. Ecological Restoration and Management 2, 53-63.

Butler, G.L., Mackay, B., Rowland, S.J. and Pease, B.C. (2009) Retention of intra-peritoneal transmitters and post-operative recovery of four Australian native fish species. Marine and Freshwater Research 60, 361-370.

Cadwallader, P.L. (1978) Some causes of the decline in range and abundance of native fishes in the Murray-Darling River system. Proceedings of the Royal Society of Victoria 90, 211-224.

Cairns, J.J., McCormick, P.V. and Niederlehner, B.R. (1993) A proposed framework for developing indicators of ecosystem health. Hydrobiologia 263, 1-44.

Carpenter, S.R. and Biggs, R. (2009) Freshwaters: managing across scales in space and time. In ‘Principles of ecosystem stewardship – resilience-based natural resource management in a changing world’. (Eds  F.S. Chapin, C. Folke, and G.P. Kofinas) pp. 197-220. (Springer Science + Business Media, New York)

Carter, G. (2011) Integrated Monitoring of Environmental Flows: Hunter River low flow phytoplankton study, NSW Office of Water, Sydney.

Casanova, M., Brock, M. (2000) How do depth, duration and frequency of flooding influence the establishment of wetland plant communities? Plant Ecology 147, 237-250.

Chapin, F.S., Folke, C. and Kofinas, G.P. (2009) A framework for understanding change. In ‘Principles of ecosystem stewardship – resilience-based natural resource management in a changing world’. (Eds F.S. Chapin, C. Folke, and G.P. Kofinas) pp. 3-28. (Springer Science + Business Media, New York)

Clements, S., Jepsen, D., Karnowski, M. and Schreck, C.B. (2005) Optimization of an acoustic telemetry array for detecting transmitter-implanted fish. North American Journal of Fisheries Management 25, 429-436.

Closs, G.E. and Lake, P.S. (1996) Drought, differential mortality and the coexistence of a native and an introduced fish species in a south east Australian intermittent stream. Environmental Biology of Fishes 47, 17-26.

Coble, P.G. (1996) Characterization of marine and terrestrial DOM in seawater using excitation-emission matrix spectroscopy. Marine Chemistry  51, 325-346.

Commonwealth Environmental Water office (2012) Annual Water Use Options 2012-13: Mid-Murray Region’. Commonwealth Environmental Water Holder, Canberra.

Commonwealth Environmental Water Office (2013) Environmental watering in the Edward-Wakool River System: Environmental Watering 2012-13  Accessed February 25, 2013 from: http://www.environment.gov.au/ewater/southern/murray/edward-wakool.html

Covich, A.P., Palmer, M.A. and Crowl, T.A. (1999) The role of benthic invertebrate species in fresh-water ecosystems. BioScience 49, 119–28.

Crook, D. (2004a) Is the home range concept compatible with the movements of two species of lowland river fish. Journal of Animal Ecology 73, 353-366.

Crook, D. (2004b) Movements associated with home range establishment by two species of lowland river fish. Canadian Journal of Fisheries Aquatic Science 61, 2183-2193.

Cross, W.F., Baxter, C.V., Donner, K.C., Rosi-Marshall, E.J., Kennedy, T.A., Hall, R.O., Wellard Kelly, H.A., and Rogers, R.S. (2011) Ecosystem ecology meets adaptive management: food web response to a controlled flood on the Colorado River, Glen Canyon. Ecological Applications 21, 2016-2033.

Crowl, T.A., McDowell, W.H., Covich, A.P. and  Johnson,  S.L. (2001) Freshwater shrimp effects on detrital processing and nutrients in a tropical headwater stream. Ecology 82, 775–73.

Cushing, D.H. (1990). Plankton production and year-class strength in fish populations: an update of the match/mismatch hypothesis. Advances in Marine Biology 26, 250-293.

Dahlen, J., Bertilsson, S., and Pettersson, C. (1996) Effects of UV-A irradiation on dissolved organic matter in humic surface waters. Environment International 22(5), 501-506.

Davies, P.E., Harris, J.H., Hillman, T.J. and Walker, K.F. (2010) The Sustainable Rivers Audit: assessing river ecosystem health in the Murray-Darling Basin, Australia. Marine and Freshwater Research, 61, (7) pp. 764-777.

Davis, J.A. and Barmuta, L.J. (1989) An ecologically useful classification of mean and near-bed flows in streams and rivers. Freshwater Biology 21, 271-282.

Downes, B.J., Barmuta, L.A., Fairweather, P.G., Faith, D.P., Keough, M.J., Lake, P.S., Mapstone, B.D and Quinn, G.P. (2002) 'Monitoring Ecological Impacts: Concepts and practice in flowing waters'. (Cambridge University Press, Melbourne)

Dickerson, K.D., Medley, K.A. and Havel, J.E. (2010) Spatial variation in zooplankton community structure is related to hydrologic flow units in the Missouri River, USA. River Research and Applications 26, 605-618.

Driver, P.D., Harris, J.H., Closs, G.P. and Koen, T.B. (2005) Effects of flow regulation on carp (Cyprinus carpio L.) recruitment in the Murray–Darling Basin, Australia, River Research and Applications 21, 327-335.

Department of Sustainability and Environment (2004) Effective community engagement: workbook and tools (Version 1 and 2). Victorian Government Department of Sustainability and Environment, Melbourne.

Edgar, B. and Davis, R. (2007) Managing algal blooms in Australia, in B Edgar (Ed.) Salt, Sediment, Nutrients and Interactions: Findings from the National Rivers Contaminants Program, Land and Water Australia, Murray-Darling Basin Commission, Canberra, Australia, pp 18–22.

Elliott S., Lead, J.R. and Baker, A. (2006) Characterisation of the fluorescence from freshwater, planktonic bacteria. Water Research 40, 2075-2083.

Fairweather, P.G. (1999) State of the environment indicators of 'river health': exploring the metaphor. Freshwater Biology 41, 211-220.

Ferrari, I., Farabegoli, A., and Mazzoni, R. (1989) Abundance and diversity of planktonic rotifers in the Po River. Hydrobiologia 186/187, 201-208.

Frazier, P.S., Page, K.J., Louis, J., Briggs, S. and Robertson, A. (2003). Relating Wetland Inundation to River Flow Using Landsat TM Data. International Journal of Remote Sensing 24 (19), 3755-3770.

Fuiman, L.A. and Werner, R.G. (2002). 'Fishery science: the unique contributions of early life stages'. (Blackwell, Oxford)

Gehrke, P.C. and Harris, J.H. (2001) Regional-scale effects of flow regulation on lowland riverine fish communities in New South Wales, Australia. Regulated Rivers-Research and Management 17(4-5), 369-391.

Gehrke, P.C., Revell, M.B. and Philbey A.W. (1993) Effects of river red gum, Eucalyptus camaldulensis, litter on golden perch, Macquaria ambigua. Journal of Fish Biology 43, 265-279.

Giling, D., Reich, P. and Thompson, R.M. (2009) Losss of riparian vegetation alters the ecosystem role of a freshwater crayfish (Cheraxdestructor) in an Australian intermittent lowland stream. Journal ofthe North American Benthological Society 28, 626–637.

Gillespie, G. and Hines, H. (1999) Status of temperate riverine frogs in South-Eastern Australia. In 'Declines and Disappearances of Australian Frogs'. (Ed. A. Campbell) pp. 109-130. (Environment Australia: Canberra)

Gilligan, D. and Schiller, C.B. (2003) Downstream transport of larval and juvenile fish in the Murray River. NSW Fisheries Report Series No. 50. NSW Fisheries, Sydney, Australia.

Gilligan, D., Rolls, R., Merrick, J., Lintermans, M., Duncan, P. and Kohen, J. (2007) Scoping the knowledge requirements for Murray crayfish (Euastacus armatus), NSW Department of Primary Industries – Fisheries Final Report Series No. 89.

Gilligan, D., Vey, A. and Asmus, M. (2009) Identifying drought refuges in the Wakool system and assessing status of fish populations and water quality before, during and after the provision of environmental, stock and domestic flows, NSW Department of Primary Industries-Fisheries Final Report Series

Grace, M.R. and Imberger, S.J. (2006) Stream Metabolism: Performing & Interpreting Measurements. Water Studies Centre Monash University, Murray Darling Basin Commission  and New South Wales Department of Environment and Climate Change.  204 pp. Accessed at http://monash.edu/science/wsc/

Green, D. (2001) The Edward-Wakool System: River Regulation and Environmental Flows. Department of Land and Water Conservation. Unpublished Report.

Grubbs, S.A. and Taylor, J.M. (2004) The influence of flow impoundment and river regulation on the distribution of riverine macroinvertebrates at Mammoth Cave National Park, Kentucky, USA. Hydrobiologia 520(1-3), 19-28.

Hadwen, W.L., Fellows, C.S., Westhorpe, D.P., Rees, G.N., Mitrovic, S.M., Taylor, B., Baldwin, D.S., Silvester, E., and Croome, R. (2010) Longitudinal trends in river functioning: patterns of nutrient and carbon processing in three Australian rivers. River Research and Applications 26(9), 1129-1152.

Hancock, M.A. and Bunn, S.E. (1997) Population dynamics and life history of Paratya australiensis Kemp, 1917 (Decapoda:Atyidae) in upland rainforest streams, south-eastern Queensland, Australia. Marine and Freshwater Research 48, 361–369.

Harris, J.H. and Gehrke, P.C. (1994) Development of predictive models linking fish population recruitment with streamflow. In: 'Proceedings of the Australian Society for Fish Biology Workshop: Population Dynamics for Fisheries Managagement'. (Ed. D.A. Hancock) pp. 195-99. (Bureau of Rural Resources Proceedings, Canberra)

Havel, J.E. and Pattinson, K.R. (2004) Spatial distribution and seasonal dynamics of plankton in a terminal multiple-series reservoir. Lake and Rervoir management, 20(1), 14-26.

Hazell, D., Osborne, W., and Lindenmayer, D. (2003) Impact of post-European stream change on frog habitat: southeastern Australia. Biodiversity and Conservation 12(2), 301-320.

Healey, M., Thompson, D. and Robertson, A. (1997) Amphibian communities associated with billabong habitats on the Murrumbidgee floodplain, Australia. Australian Journal of Ecology 22, 270-278.

Heard, G.W., Robertson, P., and Scroggie, M.P. (2006) Assessing detection probabilities for the endangered growling grass frog (Litoria raniformis) in southern Victoria. Wildlife Research 33(7), 557–564

Hladyz, S., Watkins, S.C., Whitworth, K.L., and Baldwin, D.S. (2011) Flows and hypoxic blackwater events in managed ephemeral river channels. Journal of Hydrology 401(1-2), 117-125.

Hobbs, H. H. J. (1981). The crayfishes of Georgia. Smithsonian Contributions to Zoology 318, 1-549.

Howitt, J.A., Baldwin, D.S, and Hawking, J.(2004) A Review of the Existing Studies on Lake Mulwala and its Catchment, Prepared by the Murray Darling Freshwater Research Centre for Goulburn Murray Water. Pp96.

Howitt, J.A., Baldwin, D.S., Rees, G.N., and Williams, J.L. (2007) Modelling blackwater: Predicting water quality during flooding of lowland river forests. Ecological Modelling 203(3-4), 229-242.

Howitt, J.A., Baldwin, D.S., Rees, G.N., and Hart, B.T. (2008) Photodegradation, interaction with iron oxides and bioavailability of dissolved organic matter from forested floodplain sources. Marine and Freshwater Research 59(9), 780-791.

Humphries, P. (2005). Spawning time and early life history of Murray cod, Maccullochella peelii peelii (Mitchell) in an Australian river. Environmental Biology of Fishes 72, 393-407.

Humphries, P. and Lake, P.S. (2000). Fish larvae and the management of regulated rivers. Regulated Rivers: Research & Management 16, 421-432.

Humphries, P., Cook, R.A., Richardson, A.J. and Serafini, L.G. (2006) Creating a disturbance: manipulating slackwaters in a lowland river. River Research Applications 22, 525–542.

Humphries, P., King, A.J. and Koehn, J.D. (1999) Fish, flows and flood plains: Links between freshwater fishes and their environment in the Murray-Darling River system, Australia. Environmental Biology of Fishes 56, 129-151.

Humphries, P., Serafini, L.G. and King, A.J. (2002). River regulation and fish larvae: variation through space and time. Freshwater Biology 47, 1307-1331.

Humphries, P., Richardson, A.J., Wilson, G.A. and Ellison, T. (2013) River regulation and recruitment in a protracted-spawning riverine fish. Ecological Applications 23, 208-225.

Jenkins, K.M. and Boulton, A.J. (2003) Connectivity in a dryland river: Short-term microinvertebrate recruitment following floodplain inundation. Ecology 84(10), 2708–2723

Johansson, M.E. and Nilsson, C. (2002) Responses of Riparian Plants to Flooding in Free-Flowing and Regulated Boreal Rivers: an Experimental Study. Journal of Applied Ecology 39, 971-986.

Johnston, K. and Robson, B.J. (2009) Habitat use by five sympatric Australian freshwater crayfish species (Parastacidae). Freshwater biology 54, 1629-1641.

Junk, W.J., Bayley, P.B.,  Sparks, R.E.  (1989) The flood pulse concept in river-floodplain systems. Canadian Journal of Fisheries and Aquatic Sciences 106, 110-127.

Kehr, J.M, Merritt, D.M. and Stromberg, J.C .(2013) Linkages between primary seed dispersal, hydrochory and flood timing in a semi-arid region river. Journal of Vegetation Science, http://dx.doi.org/10.1111/jvs.12061

King, A.J. (2005) Ontogenetic dietary shifts of fish in an Australian floodplain river. Marine and Freshwater Research 56, 215-225.

King, A.J., Humphries, P. and Lake, P.S. (2003). Fish recruitment on floodplains: The roles of patterns of flooding and life history characteristics. Canadian Journal of Fisheries and Aquatic Sciences 60, 773-786.

King, A.J., Ward, K.A., O'Connor, P., Green, D., Tonkin, Z. and Mahoney, J. (2009a) Adaptive management of an environmental watering event to enhance native fish spawning and recruitment. Freshwater Biology 55, 17-31.

King, A.J., Tonkin, Z. and Mahoney, J. (2009b) Environmental flow enhances native fish spawning and recruitment in the Murray River, Australia. River Research and Applications 25, 1205-1218.

King, A.J, Humphries, P. and McCasker, N. (2013) Reproduction and early life history. In: 'Ecology of Australian Freshwater Fish'. (Eds. P Humphries  and K Walker ). pp.159-192. (CSIRO Pub.: Collingwood, Vic.)

Kingsford, R.T., Brandis, K.J., Jenkins, K.M., Nairn, L.C. and Rayner, T.S. (2010) Measuring ecosystem responses to flow across temporal and spatial scales. In 'Ecosystem response modelling in the Murray-Darling Basin'. (Eds N Saintilan and I Overton) pp. 15-36. (CSIRO Pub.: Collingwood, Vic.)

Kobayashi, T. (1997) Associations between environmental variables and zooplankton body masses in a regulated Australian River. Marine and Freshwater Research 48, 523-529.

Koehn J.D. and Harrington D.J. (2005) Collection and distribution of early life stages the Murray cod (Maccullochella peelii peelii) in components of a regulated river system. Australian Journal of Zoology 53, 137-144.

Koehn J.D. and Harrington D.J. (2006) Conditions and timing of the spawning of Murray cod (Maccullochella peelii peellii) and the endangered trout cod (M. macquariensis) in regulated and unregulated rivers. Rivers Research and Applications 22, 327-343.

Koster, W., Crook, D., Dawson, D. and Moloney, P. (2012) Status of fish populations in the lower Goulburn River (2003-2012). Department of Sustainability and Environment. (Unpublished).

Lampert, W. (1997) Zooplankton research: the contribution of limnology to general ecological paradigms. Aquatic Ecology  31(1), 19-27.

Landsberg, J., James, C.D., Maconochie. J., Nicholls, A.O., Stol, J. and Tynan, R. (2002) Scale-related effects of grazing on native plant communities in an arid rangeland region of South Australia. Journal of Applied Ecology 39(3), 427-444.

Lintermans, M. 2007 Fishes of the Murray-Darling Basin: an introductory guide. MDBC publication 10/07

Lowe, B.J. (2002) Impacts of altered flow regime on riverbank vegetation in a lowland floodplain river. Unpublished PhD thesis, Charles Sturt University.

Llewellyn, L.C. (1979) Some observations on the spawning and development of the Mitchellian freshwater hardyhead Craterocephalus fluviatilis McCulloch from inland waters in New South Wales. Australian Zoologist 20, 269-288.

Lucas, M.C., Baras, E., Thom, T.J., Duncan, A., and Slavík, O. (2001) Migration of freshwater fishes. pp. 420. (Blackwell Science: Oxford)

Lunt, I.D., Jansen, A., Binns, D.L. and Kenny, S.A. (2007) Long‐term effects of exclusion of grazing stock on degraded herbaceous plant communities in a riparian Eucalyptus camaldulensis forest in south‐eastern Australia. Austral Ecology 32(8), 937-949.

Maier, H.G, Kingston, G.B, Clark, T., Frazer, T. and Sanderson, A. (2004) Risk-based approach for assessing the effectiveness of flow management in controlling cyanobacterial blooms in rivers. River Research and Applications 20, 459–471.

Mallen-Cooper, M. and Stuart, I.G. (2003) Age, growth and non-flood recruitment of two potamodromous fishes in a large semi-arid/temperature river system. River Research and Applications 19, 697-719.

Mallen-Cooper, M. and Stuart, I.G. (2006) Fish, floods and fallacy. Australasian Science 27, 19.

March, J.G., Benstead, J.P., Pringle, C.M., and Ruebel, M.W. (2001) Linking shrimp assemblages with rates of detrital processing along an elevational gradient in a tropical stream. Canadian Journal of Fisheries and Aquatic Sciences, 58(3) 470-478.

Masson, S., Pinel-Alloul, B. and Dutilleul, P. (2004) Spatial heterogeneity of zooplankton biomass and size structure in southern Québec lakes: variation among lakes and within lake among epi-, meta - and hypolimnion strata. Journal of Plankton Research 26(12), 1441-1458.

May, R.C. (1974) Larval mortality in fishes. In ‘The Early Life History of Fish’. (Ed. J. H. S. Blaxter.) pp. 3-19. (Springer-Verlag: Berlin)

McKinnon, L. (1995). Emersion of Murray crayfish, Euastacus armatus (Decapoda: Parastacidae), from the Murray River due to post-flood water quality. Proceedings-Royal Society of Victoria 107, 31-38.

Meredith, S., Gawne, B., Sharpe, C., Whiterod, N., Conallin, A. and Zukowski, S. (2002) Dryland floodplain Ecosystems: Influence of Flow pattern in fish production. Technical Report 1/2002, Murray Darling Freshwater Research Centre, Mildura 44 pp.

Milton, D.A., Arthington, A.H. (1984) Reproductive strategy and growth of the crimson-spotted rainbowfish, Melanotaenia splendida fluviatilis (Castelnau) (Pisces: Melanotaeniidae) in south-eastern Queensland. Australian Journal of Marine & Freshwater Research 35, 75-83.

Mitrovic, S.M., Oliver, R.L., Rees, G., Bowling, L.C. and Buckney, R.T. (2003) Critical flow velocities for the growth and dominance of Anabaena circinalis in some turbid freshwater rivers. Freshwater Biology 48, 164–174.

Mobed, J.J., Hemmingsen, S.L., Autry, J.L. and McGown, L.B. (1996) Fluorescence characterization of IHSS humic substances: total luminescence spectra with absorbance correction. Environmental Science & Technology 30(10), 3061-3065.

Moran, M.A. and Hodson, R.E. (1990) Bacterial production on humic and nonhumic components of dissolved organic carbon. Limnology and Oceanography 35(8), 1744-1756.

Morel, F.M.M. and Hering, J.G. (1993) 'Principles and Applications of Aquatic Chemistry.' pp. 588 (John Wiley and Sons, Inc: New York)

Morris, S., van Aardt, W.J., and Ahern, M.D. (2005)The effect of lead on the metabolic and energetic status of the Yabby, Cherax destructor, during environmental hypoxia. Aquatic Toxicology 75 (1), 16-31.

Murray Catchment Management Authority (2013) Catchment Action Plan 2013-2023. Murray Catchment Management Authority.

Murray-Darling Basin Commission (2004) Native fish strategy for the Murray-Darling Basin 2003–2013. Murray-Darling Basin Commission Publication No. 25/04.MDBC, Canberra, Australia.

Murray-Darling Basin Authority (2012) Basin Plan. Adopted by minister for sustainability, Environment, Water, Populations and Communities under subparagraph 44(3)(b)(i) of the Water Act 2007, Canberra.

Nicol J. (2004) Vegetation Dynamics of the Menindee Lakes with Reference to the Seed Bank. PhD Thesis. The University of Adelaide, Adelaide, SA.

Nilsson, C., Reidy, C.A., Dynesius, M., and Revenga, C. (2005) Fragmentation and Flow Regulation of the World's Large River Systems. Science 308(5720), 405-408.

Ning, N.S.P., Nielsen, D.L., Hillman, T.J. and Suter, P.J.  (2010) Microinvertebrate dynamics in riverine slackwater and mid-channel habitats in relation to physico-chemical parameters and food availability. River Research and Applications 26, 279-296.

Ning, N.S.P., Gawne, B., Cook, R.A., and Nielsen, D.L. (2012) Zooplankton dynamics in response to the transition from drought to flooding in four Murray–Darling Basin rivers affected by differing levels of flow regulation. Hydrobiologia 702, 45-62.

O'Connell, M., Baldwin, D.S., Robertson, A.I. and Rees, G. (2000) Release and bioavailability of dissolved organic matter from floodplain litter: influence of origin and oxygen levels. Freshwater Biology 45, 333-342.

O’Connor, J.P., O’Mahony, D.J., and O’Mahony, J.M. (2004) Movements of Macquaria ambigua, in the Murray River, south-eastern Australia. Journal of Fish Biology 66, 392-403

Odum, H.T. (1956) Primary production in flowing waters, Limnology & Oceanography 2, 102–117.

Oliver, R.L., Hart, B.T., Olley, J., Grace, M.R., Rees, C.M. and Caitcheon, C.G. (1999) The Darling River: Algal Growth and the Cycling and Sources of Nutrients, Murray Darling Basin Commission Report M386, CRC for Freshwater Ecology & CSIRO Land and Water, pp 200.

Overton, I.C. (2005) Modelling Floodplain Inundation on a Regulated River: Integrating GIS, Remote Sensing and Hydrological Models. River Research and Applications. 21 (9), 991-1001.

Overton, I.C, McEwan, K. and Sherrah, J.R. (2006) The River Murray Floodplain Inundation Model – Hume Dam to Lower Lakes. CSIRO Water for a Healthy Country Technical Report 2006. CSIRO: Canberra.

Peterson, C.G. (1996) Response of benthic algal communities to natural physical disturbances. In ‘Algal ecology: freshwater benthic ecosystems’. (Eds. R.J. Stevenson., M.L. Bothwell, and R.L. Lowe) pp 375-403 (Academic Press, San Diego).

Petts, G.E. (1984) Impounded rivers: perspectives for ecological management.(John Wiley, Chichester, UK)

Poff, N.L. (2009) Managing for Variability to Sustain Freshwater Ecosystems. Journal of Water Resources Planning and Management 135(1), 1-4.

Poff, N.L, Allan, J.D., Bain, M.B., Karr, J.R., Prestegaard, K.L., Richter, B.D., Sparks, R.E. and Stromberg, J.C. (1997) The natural flow regime. BioScience 47, 769-784.

Poff NL, Olden JD, Merritt DM, Pepin DM (2007) Homogenization of regional river dynamics by dams and global biodiversity implications. Proceedings of the National Academy of Sciences 104: 5732-5737.

Poff, N.L. and Zimmerman, J.K.H. (2010) Ecological responses to altered flow regimes: a literature review to inform the science and management of environmental flows. Freshwater Biology 55, 194-205.

Price, A.E. and Humphries, P. (2010) The role of dispersal and retention in the early life stages of shrimp in a lowland river. Canadian Journal of Fisheries and Aquatic Sciences 67(4), 720-729.

Price, A.E., Humphries, P., Gawne, B., and Thoms, M.C. (2013) Effects of discharge regulation on slackwater characteristics at multiple scales in a lowland river. Canadian Journal of Fisheries and Aquatic Sciences 70, 253-262.

Pusey, B.J., Kennard, M.J. and Arthington A.H. (2004). 'Freshwater fishes of North-eastern Australia.'  (CSIRO Publishing, Melbourne)

Quinn, J.M. (1991) Guidelines for the control of undesirable biological growths in water. Consultancy report 6213/2. Water Quality Centre, Hamilton, New Zealand. Cited by ANZECC and AMCANZ (2000) Australian and New Zealand Guidelines for Fresh and Marine Water Quality, Volume 1, The Guidelines. http://www.deh.gov.au/water/quality/nwqms/volume1.html

R Development Core Team (2013) R: A Language and Environment for Statistical Computing. R Foundation for Statistical Computing, Vienna, Austria.

Ralph, T., Spencer, J. and Rayner, S. (2011) Fish. In 'Floodplain Wetland Biota in the Murray-Darling Basin.’ (Eds K Rogers and TJ Ralph) pp. 205-251. (CSIRO Publishing: Collingwood, VIC).

Reckendorfer, W., Keckeis, H., Winkler, G. and Schiemer, F. (1999) Zooplankton abundance in the River Danube, Austria: the significance of inshore retention. Freshwater Biology 41, 583-591.

Reid, D.D., Harris, J.H. and Chapman, D.J. (1997) NSW Inland Commercial Fishery Data Analysis. Cronulla, N.S.W. Fisheries. ISBN 0 73109410 7

Reid, M., Ogden, R. and Thoms, M. (2011) The influence of flood frequency, geomorphic setting and grazing on plant communities and plant biomass on a large dryland floodplain. Journal of Arid Environments 75(9), 815-826.

Reynolds, L.F. (1983) Migration patterns of five fish species in the Murray-Darling River system. Marine and Freshwater Research 34, 857-871.

Reynolds, C.S. and Descy, J.P. (1996) The production, biomass and structure of phytoplankton in large rivers. Archiv fuer Hydrobiologie Supplement 113, 161–187.

Ritchie, R.J. (2006) Consistent sets of spectrophotometric chlorophyll equations for acetone, methanol and ethanol solvents. Photosynthesis Research 89,27-41.

Richardson, A.J., Growns, J.E. and Cook, R.A. (2004) Distribution and life history of caridean shrimps in regulated lowland rivers in southern Australia. Marine and Freshwater Research 55, 295-308

Roberts, D.T., Duivenvoorden. L.J. and Stuart, I.G. (2008) Factors influencing recruitment patterns of Golden Perch (Macquaria ambigua oriens) within a hydrologically variable and regulated Australian tropical river system. Ecology of Freshwater Fish 17, 577-589

Roberts, J. and Marston, F. (2011) Water regime for wetland and floodplain plants: A source book for the Murray-Darling Basin. National Water Commission, Canberra.

Roberts, B.J., Mulholland, P.J. and Hill, W.R. (2007) Multiple Scales of Temporal Variability in Ecosystem Metabolism Rates: Results from 2 Years of Continuous Monitoring in a Forested Headwater Stream, Ecosystems 10, 588–606.

Robertson, A.I. (1997) Land-water linkages in floodplain river systems: the influence of domestic stock. In 'Frontiers in Ecology: Building the Links. (Eds N Klomp and I Lunt) pp. 207-218. (Elsevier Science Ltd: Oxford, UK).

Robertson, A.I., Bacon, P. and Heagney, G. (2001) The responses of floodplain primary production to flood frequency and timing. Journal of Applied Ecology 38, 126-136.

Robertson, A.I., Bunn, S.E., Boon, P.I., and Walker, K.F. (1999) Sources, sinks and transformations of organic carbon in Australian floodplain rivers. Marine and Freshwater Research 50, 813-829

Robertson, A.I. and Rowling, R. (2000) Effects of livestock on riparian zone vegetation in an Australian dryland river. Regulated Rivers: Research and Management 16, 527-541.

Robinson, W.A. (2012). Calculating statistics, metrics, sub-indicators and the SRA Fish theme index: A Sustainable Rivers Audit Technical report,Consultancy report to the Murray-Darling Basin Authority.

Robinson, C.T., Uehlinger, U. and Monaghan, M.T. (2003) Effects of a multi-year experimental flood regime on macroinvertebrates downstream of a reservoir Aquatic Sciences 65, 210–222.

Robinson, C.T., Uehlinger, U. and Monaghan, M.T (2004) Stream ecosystem response to multiple experimental floods from a reservoir. River Research and Applications 20, 359–377.

Rowland, S.J. (1983) Spawning of the Australian freshwater fish Murray cod, Maccullochella peeli (Mitchell), in earthen ponds. Journal of Fish Biology 23, 525-534.

Rowland, S.J. (1998) Aspects of the reproductive biology of Murray cod, Maccullochella peelii peelii. Proceedings of the Linnean Society of New South Wales 1998, 147-162.

Ryder, D.S. (2004) Response of Epixylic Biofilm Metabolism to Water Level Variability in a Regulated Floodplain River. Journal of the North American Benthological Society 23(2), 214-223.

Rzoska, J. (1978) On the Nature of Rivers. Junk, The Hague.

Saunders, J.F. and Lewis, W.M. (1988) Zooplankton abundance and transport in a tropical white-water river. Hydrobiologia 162, 147–155.

Saunders, J.F. and Lewis, W.M. (1989) Zooplankton abundance in the lower Orinoco River, Venezuela. Limnology and Oceanography 34, 397–409.

Serafini LG, Humphries P. 2004. A preliminary guide to the identification of the larvae of species of freshwater fish from the Murray-Darling Basin. CRC for Freshwater Ecology, February 2004.

Shaikh, M., Green, D. and Cross, H. (2001) A Remote Sensing Approach to Determine the Environmental Flows for Wetlands of the Lower Darling River, New South Wales, Australia. International Journal of Remote Sensing 22, 1737-1751.

Shannon, J.P., Blinn, D.W., McKinney, T., Benenati, E.P., Wilson, K.P. and O'Brien, C. (2001) Aquatic food base response to the 1996 test flood below Glen Canyon Dam, Colorado River, Arizona. Ecological Applications 11(3), 672–685.

Sheldon, F., Boulton, A.J. and Puckridge, J.T. (2002) Conservation value of variable connectivity: aquatic invertebrate assemblages of channel and floodplain habitats of a central Australian arid-zone river, Cooper Creek. Biological Conservation 103(1), 13-31.

Sherman, B.S., Webster, I.T., Jones, G.J. and Oliver, R.L. (1998) Transitions between Aulacoseira and Anabaena dominance in a turbid river weir pool. Limnology and Oceanography 43,1902–1915.

Shiel, R. (1990) Zooplankton. In 'The Murray' (Eds N. Mackay & D. Eastburn) pp. 274-284. (Murray Darling Basin Commission, Canberra, ACT)

Shiel, R.J. (1995) A guide to the identification of rotifers, cladocerans and copepods fromAustralian inland waters. Co-operative Research Centre for Freshwater Ecology, TheMurray-Darling Freshwater Research Centre, Albury, NSW.

Shiel, R.J., Walker, K.F. and Williams, W.D. (1982) Plankton of the Lower River Murray, South Australia. Marine and Freshwater Research 33, 301-327.

Sims N.C. and Thoms, M.C. (2002) 2 FLOOD PLAINS: The role of temporary storage in the exchange of sediment and nutrients-What happens when flood plains wet themselves: Vegetation response to inundation on the lower Balonne flood. IAHS Publications-Series of Proceedings and Reports-Intern Assoc Hydrological Sciences 276, 195-202.

Speas, D.W. (2000) Zooplankton density and community composition following an experimental flood in the Colorado River, Grand Canyon, Arizona. Regulated Rivers: Research & Management 16, 73–81.

Spencer, J.A. and Wassens, S. (2009) 'Responses of waterbirds, fish and frogs to environmental flows in the Lowbidgee wetlands in 2008-09. Final report for the NSW Rivers Environmental Restoration Program.' (Rivers and Wetland Unit, NSW Department of Environment and Climate Change: Sydney.)

Stanley, E.H., Powers, S.M., and Lottig, N.R. (2010) The evolving legacy of disturbance in stream ecology: concepts, contributions, and coming challenges. Journal of the North American Benthological Society 29(1), 67-83.

Stevenson R.J. (1996) An introduction to algae in aquatic ecosystems. In ‘Algal ecology: freshwater benthic ecosystems’. (Eds. R.J. Stevenson., M.L. Bothwell, and R.L. Lowe) pp 3-30 (Academic Press, San Diego).

Stuart, I.G. and Jones, M.J. (2006) Movement of common carp, Cyprinus carpio, in a regulated lowland Australian river: implications for management. Fisheries Management and Ecology 13, 213-219.

Stuart, I.G., Williams, A., McKenzie, J. and Holt, T. (2006) Managing a migratory pest species: a selective trap for common carp. North American Journal of Fisheries Management 26, 888-893.

Tedesco, P.A., Hugueny, B., Oberdorff, T., Dürr, H.H., Mérigoux, S. and De Mérona, B. (2008) River hydrological seasonality influences life history strategies of tropical riverine fishes. Oecologia 156, 691-702.

Tett, P., Kelly, M.G. and Hornberger, G.M. (1975) Estimation of chlorophyll a and pheophytin a in methanol. Limnology and Oceanography 20, 887-896.

Thoms, M., Reid, M., Christianson, K., & Munro, F. (2006). Variety is the Spice of River Life: Recognising Hydraulic Diversity as a Tool for Managing Flows in Regulated Rivers. In J. S. Rowan, Duck, R.W. and Werritty, A. (Eds), Sediment dynamics and the hydromorphology of fluvial systems, Wallingford, UK, IAHS Press .  IAHS Publications 306, 169-178.

Toner, M. and Keddy, P. (1997) River hydrology and riparian wetlands - a predictive model for ecological assembly.  Ecological Applications 7 (1), 236-246.

Tonkin, Z., King, A.J. and Mahoney, J. (2008a) Effects of flooding on recruitment and dispersal of the Southern Pygmy Perch (Nannoperca australis) at a Murray River floodplain wetland. Ecological Management and Restoration 9, 196-201.

Tonkin Z., King A.J. and Robertson, A. (2008b) Validation of daily increment formation and the effects of different temperatures and feeding regimes on short-term otolith growth in Australian smelt Retropinna semoni. Ecology of Freshwater Fish 17, 312-17.

Tonkin, Z., King, A.J., Robertson, A.I. and Ramsey, D. (2011). Early fish growth varies in response to components of the flow regime in a temperate floodplain river. Freshwater Biology 56, 1769-1782.

Townsend, P.A. and Walsh, S.J. (1998) Modelling Floodplain Inundation using an Integrated GIS with Radar and Optical Remote Sensing. Geomorphology 21, 295-312.

Uehlinger, U. (2000) Resistance and resilience of ecosystem metabolism in a flood-prone river system. Freshwater Biology 45, 319–332.

Uehlinger, U., Bűhrer, H. and Reichert, P. (1996) Periphyton dynamics in a floodprone prealpine river: evaluation of significant processes by modelling. Freshwater Biology 36, 249-263.

Underwood, A.J. (1991) Beyond BACI: Experimental designs for detecting human environmental impacts on temporal variations in natural populations. Australian Journal of Marine and Freshwater Research 42, 569-587.

Unmack, P.J. (2001) Biogeography of Australian freshwater fishes. Journal of Biogeography 28, 1053-1089.

Vadadi-Fülöp, C. (2013) Microcrustacean assemblages in a large river: on the importance of the flow regime. Hydrobiologia 702 (1), 129-140.

Val, J., Chatfield, A., McCarthy, B., Summerell, G. and Jurskis, V. (2007) Red Gum Rescue Project: Ecological responses to watering stressed river red gum (Eucalyptus camaldulensis) dominated wetlands in the Lower Murray Darling. Department of Natural resources, Deniliquin NSW.

Valdez, R.A., Hoffnagle, T.L, McIvor, C.C., McKinney, T. and Leibfried W.C. (2001) Effects of a test flood on fishes of the Colorado River in Grand Canyon, Arizona. Ecological Applications 11(3), 686–700.

Vallania, A. and Corigliano, M.D. (2007) The effect of regulation caused by a dam on the distribution of the functional feeding groups of the benthos in the sub basin of the Grande River (San Luis, Argentina). Environmental Monitoring and Assessment 124, 201-209.

Van Zanten, B. and Van Dijk, G.M. (1994) Seasonal Development of Zooplankton in the Lower River Rhine During the Period 1987-1991. Water Science and Technology 29(3), 49-51.

Vietz, G.J., Sammonds, M.J. and Stewardson, M.J. (2013) Impacts of flow regulation on slackwaters in river channels. Water Resources Research 49, 1797–1811.

Vink, S., Bormans, M., Ford, P.W. and Grigg, N.J. (2005) Quantifying ecosystem metabolism in the middle reaches of the Murrumbidgee River during irrigation flow releases. Marine and Freshwater Research 56, 227-241.

Vranovský, M. (1995) The effects of current velocity upon the biomass of zooplankton in the River Danube side arms. Biologia 50, 461-464.

Wahl, D.H., Goodrich,  J., Nannini,  M.A., Dettmers, J.M. and Soluk,  D.A. (2008) Exploring riverine zooplankton in three habitats of the Illinois River ecosystem: where do they come from? Limnology and Oceanography 53, 2583–2593.

Ward, J.V., Tockner, K., and Schiemer, F. (1999) Biodiversity of floodplain river ecosystems: Ecotones and connectivity. Regulated Rivers-Research & Management 15(1-3), 125-139.

Ward, J.V., Tockner, K., Uehlinger, U. and Malard F. (2001) Understanding natural patterns and processes in river corridors as the basis for effective river restoration. Regulated Rivers: Research & Management 17, 311-323.

Warfe, D. and Barmuta, L. (2006) Habitat structural complexity mediates food web dynamics in a freshwater macrophyte community. Oecologia 150(1), 141-154.

Wassens, S., Hall, A., Osborne, W. and Watts, R.J. (2010) Habitat characteristics predict occupancy patterns of the endangered amphibian Litoria raniformis in flow-regulated flood plain wetlands. Austral Ecology 35(8), 944-955.

Wassens, S. and Maher, M. (2011) River regulation influences the composition and distribution of inland frog communities. River Research and Applications 27(2), 238-246.

Watkins, S., Hladyz, S., Whitworth, K. and Baldwin, D. (2010) Understanding the relationship between low dissolved oxygen blackwater events and managed flows in the Edward-Wakool River system. Report prepared for the Murray Catchment Management Authority by The Murray-Darling Freshwater Research Centre, MDFRC Publication 12/2010, June, 66 pp.

Watts, R.J., Ryder, D.S., Burns, A., Wilson, A.L., Nye, E.R., Zander, A. and Dehaan, R. (2006) Responses of biofilms to cyclic releases during a low flow period in the Mitta Mitta River, Victoria, Australia. Report to the Murray Darling Basin Commission. Institute for Land Water & Society Report #24, Charles Sturt University, Wagga Wagga, NSW. 30pp. [available at http://www.csu.edu.au/research/ilws/publications/report1Aug.pdf]

Watts, R.J., Ryder, D.S., Burns, A., Zander, A., Wilson, A. L., and Dehaan, R. (2008) Monitoring of a pulsed release in the Mitta Mitta River, Victoria, during the bulk water transfer from Dartmouth Dam to Hume Dam 2007-08. Institute for Land Water and Society Report No. 45. [available at http://www.csu.edu.au/research/ilws/research/publications/docs/2008report45.pdf].

Watts, R.J., Allan, C., Bowmer, K.H., Page, K.J., Ryder, D.S. and Wilson, A.L. (2009a) Pulsed flows: a review of environmental costs and benefits and Best practice. Waterlines report, National Water Commission, Canberra. [available at http://www.nwc.gov.au/resources/documents/Waterlines_report_No_16_-_Pulsed_flows_full_version.pdf]

Watts, R.J., Ryder, D.S., and Allan, C. (2009b) Environmental monitoring of variable flow trials conducted at Dartmouth Dam, 2001/02-07/08 - Synthesis of key findings and operational recommendations. ILWS Report No. 50. CSU, Albury, NSW. [available at www.csu.edu.au/research/ilws/research/docs/WATTS_MITTA_2008.pdf]

Watts, R.J., Kopf, R.K., Hladyz, S., Grace, M., Thompson, R., McCasker, N., Wassens, S., Howitt, J.A., A. Burns, and Conallin, J. (2013) Monitoring of ecosystem responses to the delivery of environmental water in the Edward-Wakool system, 2011-2012. Institute for Land, Water and Society, Charles Sturt University, Report 2. Prepared for Commonwealth Environmental Water.

Webster, I.T., Sherman, B.S., Bormans, M. and Jones, G. (2000) Management strategies for cyanobacterial blooms in an impounded lowland river. Regulated Rivers: Research and Management 16, 513–525.

Wehr, J.D., Peterson, J. and Findlay, S. (1999) Influence of three contrasting detrital carbon sources on planktonic bacterial metabolism in a mesotrophic lake. Microbial Ecology 37, 23-35

Welcomme, R.L. 1979 Fisheries ecology of floodplain rivers. (Longman: London).

Winemiller, K.O. and Rose, K.A. (1992) Patterns of Life-History Diversification in North American Fishes: Implications for Population Regulation. Canadian Journal of Fisheries Aquatic Sciences 49, 2196-2218.

Ye, Q., Jones, K. and Pierce, B.E. (2000) Murray cod (Maccullochella peelii peelii), fishery assessment report to PIRSA for Inland Waters Fishery Management Committee, South Australian Fisheries Assessment Series 2000/17. SARDI, Adelaide, Australia.

Zampatti, B. and Leigh, S. (2013) Effects of flooding on recruitment and abundance of Golden Perch (Macquaria ambigua ambigua) in the lower River Murray. Ecological Management and Restoration 14, 135-143.

Appendices

Appendix 1.  Statistical results for 2 way Analysis of Variance (ANOVA) to examine differences across rivers.

|  |  |  |  | Factor |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | River |  |  | Time |  |  | River x Time |  |  |
| Parameter | Response variable | Size class | Transform- ation | df | F-test | P value | df | F-test | P value | df | F-test | P value |
| Total zooplankton | Density (animals/L) | Total | Log 10 | 4 | 1.208 | Non sig | 17 | 13.445 | < 0.001 | 66 | 1.076 | Non sig |
| Total zooplankton | Density (animals/L) | 53–106 µm | Log 10 | 4 | 14.232 | < 0.001 | 17 | 29.942 | < 0.001 | 66 | 3.840 | < 0.001 |
| Total zooplankton | Density (animals/L) | 106–200 µm | Log 10 | 4 | 0.817 | Non sig | 17 | 43.838 | < 0.001 | 66 | 4.396 | < 0.001 |
| Total zooplankton | Density (animals/L) | 200–500 µm | Log 10 | 4 | 31.566 | < 0.001 | 17 | 55.805 | < 0.001 | 66 | 3.874 | < 0.001 |
| Rotifers | Density (animals/L) | Total | Log 10 | 4 | 3.714 | < 0.01 | 17 | 14.477 | < 0.001 | 66 | 1.759 | < 0.001 |
| Cladocera | Density (animals/L) | Total | Log 10* | 4 | 54.743 | < 0.001 | 17 | 4.284 | < 0.001 | 66 | 3.051 | < 0.001 |
| Copepods | Density (animals/L) | Total | Log 10 | 4 | 3.518 | < 0.01 | 17 | 18.918 | < 0.001 | 66 | 2.781 | < 0.001 |
| Total ovigerous zooplankton | % | Total | Arcsine | 4 | 9.079 | < 0.001 | 16 | 50.63 | < 0.001 | 63 | 2.732 | < 0.001 |
| Ovigerous rotifers | % | Total | Arcsine | 4 | 3.918 | < 0.01 | 16 | 36.475 | < 0.001 | 63 | 2.033 | < 0.001 |
| Ovigerous cladocera | % | Total | Arcsine* | 4 | 0.269 | Non sig | 16 | 4.201 | < 0.001 | 63 | 0.659 | Non sig |
| Ovigerous copepods | % | Total | Arcsine* | 4 | 5.186 | < 0.001 | 16 | 1.674 | < 0.05 | 63 | 0.959 | Non sig |

Appendix 2. Statistical results for 2 way mixed-effects Analysis of Variance (ANOVA). A significant interaction between the two fixed factors: Period (before, during, after) and CI (control rivers, impact rivers) indicates that the mean abundance of zooplankton within Impact Rivers was significantly different to changes that occurred over the same period of time within the Control Rivers. Significant interactions are in bold.

| Environmental flow | Parameter | Main effect | d.f | F-test | p-value |
| --- | --- | --- | --- | --- | --- |
| Nov 2012 – Colligen Creek fresh #1 |  |  |  |  |  |
|  | Total zooplankton | Period (B-D-A) | 2,54 | 0.380 | 0.686 |
|  |  | CI (C-I) | 1,54 | 0.0405 | 0.841 |
|  |  | Period*CI | 2,54 | 1.7151 | 0.189 |
|  | 53 – 106 µm | Period (B-D-A) | 2,54 | 0.453 | 0.638 |
|  |  | CI (C-I) | 1,54 | 0.0236 | 0.879 |
|  |  | Period*CI | 2,54 | 1.1573 | 0.322 |
|  | 106 - 200 µm | Period (B-D-A) | 2,54 | 0.1326 | 0.876 |
|  |  | CI (C-I) | 1,54 | 0.0044 | 0.947 |
|  |  | Period*CI | 2,54 | 2.1156 | 0.130 |
|  | 200 - 500 µm | Period (B-D-A) | 2,54 | 0.2757 | 0.760 |
|  |  | CI (C-I) | 1,54 | 0.2581 | 0.614 |
|  |  | Period*CI | 2,54 | 1.189 | 0.312 |
|  | Total ovigerous zoops | Period (B-D-A) | 2,54 | 5.369 | 0.007 |
|  |  | CI (C-I) | 1,54 | 0.3549 | 0.554 |
|  |  | Period*CI | 2,54 | 0.655 | 0.524 |
|  | Ovigerous rotifers | Period (B-D-A) | 2,54 | 7.329 | 0.002 |
|  |  | CI (C-I) | 1,54 | 2.205 | 0.143 |
|  |  | Period*CI | 2,54 | 0.275 | 0.761 |
|  | Ovigerous cladocera | Period (B-D-A) | 2,54 | 0.0848 | 0.919 |
|  |  | CI (C-I) | 1,54 | 0.958 | 0.332 |
|  |  | Period*CI | 2,54 | 0.9172 | 0.406 |
|  | Ovigerous copepods | Period (B-D-A) | 2,54 | 0.0677 | 0.935 |
|  |  | CI (C-I) | 1,54 | 0.461 | 0.500 |
|  |  | Period*CI | 2,54 | 0.4174 | 0.661 |
| Nov 2012 – Colligen Creek fresh #2 |  |  |  |  |  |
|  | Total zooplankton | Period (B-D-A) | 2,38 | 21.263 | <0.0001 |
|  |  | CI (C-I) | 1,1, | 1.141 | 0.479 |
|  |  | Period*CI | 2,38 | 1.947 | 0.157 |
|  | 53 – 106 µm | Period (B-D-A) | 2,38 | 9.753 | 0.004 |
|  |  | CI (C-I) | 1,1, | 0.263 | 0.696 |
|  |  | Period*CI | 2,38 | 1.947 | 0.157 |
|  | 106 - 200 µm | Period (B-D-A) | 2,38 | 20.321 | <.0001 |
|  |  | CI (C-I) | 1,1, | 9.457 | 0.2002 |
|  |  | Period*CI | 2,38 | 6.715 | 0.0032 |
|  | 200 - 500 µm | Period (B-D-A) | 2,38 | 60.4503 | <.0001 |
|  |  | CI (C-I) | 1,1, | 19.719 | 0.141 |
|  |  | Period*CI | 2,38 | 10.844 | 0.0002 |
|  | Total ovigerous zoops | Period (B-D-A) | 2,38 | 34.383 | <0.0001 |
|  |  | CI (C-I) | 1,1, | 1.328 | 0.455 |
|  |  | Period*CI | 2,38 | 2.221 | 0.1224 |
|  | Ovigerous rotifers | Period (B-D-A) | 2,38 | 23.054 | <0.0001 |
|  |  | CI (C-I) | 1,1, | 1.520 | 0.4338 |
|  |  | Period*CI | 2,38 | 4.334 | 0.0202 |
|  | Ovigerous cladocera | Period (B-D-A) | 2,38 | 13.473 | <0.0001 |
|  |  | CI (C-I) | 1,1, | 1.683 | 0.418 |
|  |  | Period*CI | 2,38 | 0.0668 | 0.9355 |
|  | Ovigerous copepods | Period (B-D-A) | 2,38 | 1.2815 | 0.2893 |
|  |  | CI (C-I) | 1,1, | 2.626 | 0.352 |
|  |  | Period*CI | 2,38 | 0.165 | 0.848 |